SECTION 08 56 19

PASS-THRU WINDOWS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2008 - 2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Ready Access Drive-Thru Windows; pass thru window products.
.
This section is based on the products of Ready Access Drive-Thru Windows, which is located at:
1815 Arthur Dr.
West Chicago, IL 60185
Toll Free Tel: 800-621-5045
Tel: 630-876-7766
Fax: 630-876-7767
Email: request info (ready@ready-access.com)
Web: http://www.ready-access.com  
 [ Click Here ] for additional information.
A trusted market leader for nearly 40 years, specified by the largest chains worldwide. Patented design, 5-10 day lead time on standard windows, and energy-efficient initiatives including the only NFRC certified pass-thru. Designs for cold climate, warm climate, west coast health code, hurricane and security.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Flush mount pass-thru windows.

    - Bump out pass-thru windows.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 07 62 00 - Sheet Metal Flashing and Trim.

    - Section 07 91 23 - Backer Rods.

    - Section 26 05 00 - Common Work Results for Electrical.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ASTM A 240 - Heat-Resisting Chromium and Chromium-Nickel Stainless Steel Plate, Sheet, and Strip for Pressure Vessels.

    - ASTM A 653 - Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

    - ASTM B 209 - Aluminum and Aluminum-Alloy Sheet and Plate.

    - ASTM B 221 - Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

    - ASTM B 580 - Standard Specification for Anodic Oxide Coatings on Aluminum.

    - ASTM B 680 - Standard Test Method for Seal Quality of Anodic Coatings on Aluminum by Acid Dissolution.

    - ASTM C 1048 - Heat-Treated Flat Glass--Kind HS, Kind FT Coated and Uncoated Glass.

    - ASTM C 1172 - Standard Specification for Laminated Architectural Flat Glass.

    - ASTM E 774 - Standard Specification for Sealed Insulating Glass Units.

    - Aluminum Association AA DAF-45 - Designation System for Aluminum Finishes.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00 - Administrative Requirements.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: Include plans, elevations, sections, and details, indicating dimensions, tolerances, materials, fabrication, glazing, fasteners, hardware, finish, electrical wiring diagrams, options, and accessories.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

    - Selection Samples: For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and patterns.

    - Verification Samples: For each finish product specified, two samples, minimum size 6 inches (150 mm) square, representing actual product, color, and patterns.

    - Manufacturer's Certificates: Certify products meet or exceed specified requirements.

    - Operation and Maintenance Manual: Submit manufacturer's operation and maintenance manual, including operation, maintenance, adjustment, and cleaning instructions, trouble shooting guide, parts list, and electrical wiring diagrams.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Minimum of 25 years successful experience continuously manufacturing pass-thru windows.

    - Installer Qualifications: Installer shall have five years experience manufacturing and fabricating windows of similar type and scope as those specified in this section.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

      - Finish areas designated by Architect.

      - Do not proceed with remaining work until workmanship, color, and sheen are approved by Architect.

      - Refinish mock-up area as required to produce acceptable work.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in manufacturer's unopened packaging with labels clearly identifying product name and manufacturer until ready for installation.

    - Storage: Store materials in clean, dry area indoors until ready for installation.

    - Handling: Protect materials and finish from damage during handling and installation.

  - SEQUENCING

    - Ensure that locating templates and other information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Ready Access Drive-Thru Windows, which is located at:
1815 Arthur Dr.
West Chicago, IL 60185
Toll Free Tel: 800-621-5045
Tel: 630-876-7766
Fax: 630-876-7767
Email: request info (ready@ready-access.com);Web: http://www.ready-access.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

** NOTE TO SPECIFIER ** Select the flush mount or bump-out pass thru window(s) as required from the following paragraphs and delete those not required.

  - FLUSH MOUNTED PASS-THRU WINDOWS

    - Single Panel Pass-Thru Windows: Ready Access 275 Series Single Panel Pass-Thru Window.

** NOTE TO SPECIFIER ** Select one of the following size paragraphs and delete those not required.

      - Opening:

** NOTE TO SPECIFIER ** Select one of the following size paragraphs and delete those not required.

        - Overall Opening 47-1/2 inches wide by 43-1/2 inches high and a Service Opening 19 inches wide by 36 inches high.

        - Overall Opening 47-1/2 inches wide by 35-3/4 inches high and a Service Opening 19 inches wide by 28 inches high.

** NOTE TO SPECIFIER ** The following three service opening windows meet the California health department restricted opening requirements of 432 square inches.

        - Overall Opening 47-1/2 inches wide by 35-3/4 inches high and a Service Opening 15 inches wide by 28 inches high.

        - Overall Opening 35-3/4 inches wide by 35-3/4 inches high and a Service Opening 14 inches wide by 28 inches high.

        - Custom: ________ inches wide by ________ inches high.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required. Note that 14 inch by 24 inch service opening is available Manual Open/Self Close only.

        - Manual Open/Self Close.

        - Manual Open/Manual Close.

        - Manual Open/Electronic Release.

        - Automatic Electric Open/Close.

      - Door Type: Sliding, 1 door panel.

      - Opening Direction:

** NOTE TO SPECIFIER ** Select the opening direction required and delete the one not required.

        - Right to left from outside (customers view).

        - Left to right from outside (customers view).

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Bottom Sill: Angled downward, track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Black Delrin handle with pressed-in stainless steel spring pins. Stainless steel handle mounting bracket. Stainless steel spring-loaded mounting base.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum Color dark bronze.

        - Anodized Aluminum Color clear aluminum.

        - Powder Drylac RAL#. Coat Finish Color as selected by the Architect.

** NOTE TO SPECIFIER ** Include the following paragraph for Manual Open/Electronic Release operation only.

      - Electrical: A separate 104/120 Volt, 3 Amp, 60 Hz circuit.

** NOTE TO SPECIFIER ** Include the following paragraph for Automatic Electric Open/Close operation only. Select the power required and delete the one not required.

      - Electrical:

        - A separate 115 Volt, 60 Hz, with 15 Amp branch circuit.

        - A separate 220 Volt, 50/60 Hz, with 8 Amp branch circuit.

    - Single Panel Low Profile Pass-Thru Windows: Ready Access 275 Low Profile Single Panel Series Pass-Thru Window.

      - Opening:

** NOTE TO SPECIFIER ** Select one of the following size paragraphs and delete those not required.

        - Overall Opening 47-1/2 inches wide by 43-1/2 inches high and a Service Opening 19 inches wide by 36 inches high.

        - Overall Opening 47-1/2 inches wide by 35-3/4 inches high and a Service Opening 19 inches wide by 28 inches high.

        - Custom: ________ inches wide by ________ inches high.

** NOTE TO SPECIFIER ** The following two service opening windows meet the California health department restricted opening requirements of 432 square inches.

        - Overall Opening 47-1/2 inches wide by 35-3/4 inches high and a Service Opening 15 inches wide by 28 inches high.

        - Overall Opening 35-3/4 inches wide by 35-3/4 inches high and a Service Opening 14 inches wide by 28 inches high.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required. Note that 14 inch by 24 inch service opening is available Manual Open/Self Close only.

        - Manual Open/Self Close.

        - Manual Open/Manual Release.

        - Manual Open/Electronic Release.

      - Door Type: Sliding, 1 door panel.

      - Opening Direction:

** NOTE TO SPECIFIER ** Select the opening direction required and delete the one not required.

        - Right to left from outside (customers view).

        - Left to right from outside (customers view).

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Bottom Sill: Angled downward, track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Black Delrin handle with pressed-in stainless steel spring pins. Stainless steel handle mounting bracket. Stainless steel spring-loaded mounting base.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum Color dark bronze.

        - Anodized Aluminum Color clear aluminum.

        - Powder Drylac RAL#. Coat Finish Color as selected by the Architect.

** NOTE TO SPECIFIER ** Include the following paragraph for Manual Open/Electronic Release operation only.

      - Electrical: A separate 104/120 Volt, 3 Amp, 60 Hz circuit.

** NOTE TO SPECIFIER ** The following pass-thru window meets the California health department restricted opening requirements of 432 square inches and where health codes require a fly fan.

    - Single Panel Pass-Thru Window with Transom: Ready Access 275 West Coast Single Panel Electric Slider Window with integrated double split transom and fly fan.

      - Opening:

** NOTE TO SPECIFIER ** Select one of the following opening paragraphs and delete those not required.

        - Overall Opening with Transom: 47-1/2 inches wide by 59-1/2 inches high.

        - Service Opening: 15 inches wide by 28 inches high.

        - Service Opening: 19 inches wide by 22 inches high.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required.

        - Manual Open/Self Close.

        - Manual Open/Electronic Release.

        - Automatic Electric Open/Close.

      - Door Type: Sliding, 1 door panel.

      - Opening Direction:

** NOTE TO SPECIFIER ** Select the opening direction required and delete the one not required.

        - Right to left from outside (customers view).

        - Left to right from outside (customers view).

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Bottom Sill: Angled downward, track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Black Delrin handle with pressed-in stainless steel spring pins. Stainless steel handle mounting bracket. Stainless steel spring-loaded mounting base.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum Color bronze.

        - Anodized Aluminum Color aluminum.

        - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Include the following paragraph for Automatic Electric Open/Close operation only. Select the power required and delete the one not required.

      - Electrical:

        - A separate 115 Volt, 60 Hz. 15 Amp branch circuit.

        - A separate 220 Volt, 60 Hz. 8 Amp branch circuit.

** NOTE TO SPECIFIER ** The following air curtains are optional accessories. Select the air curtain required or delete if not required. Satisfies requirements for fly fan applications.

      - Air Curtains: Access Air AA100 Fly Fan.

        - Mode of Operation: Outside fly fan.

        - Dimensions: 22 inches wide by 11-3/4 inches high by 13 inches deep.

        - Air Velocity: 600 feet per minute.

        - Electrical: A separate 104/120 Volt, 3 Amp, 60 Hz circuit.

        - UL Listed: E193191, 98AL.

        - Exterior Housing: Aluminum sheet, ASTM B 209, Alloy 5005-AQ-H34.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

        - Finish:

          - Anodized Aluminum Color dark bronze.

          - Anodized Aluminum Color clear aluminum.

          - Powder Drylac RAL#. Coat Finish Color as selected by the Architect.

      - Air Curtains: Access Air AA275 Pass-Thru Air Curtain System.

        - Modes of Operation:

          - Outside warm air flow only.

          - Inside warm air flow only.

          - Both outside and inside warm air flow.

          - Outside fly fan only.

        - Dimensions: 47-1/2 inches wide by 16 inches high by 21-3/4 inches deep.

        - Air Velocity: 600 feet per minute.

        - Heater: 7,100 watts.

        - Electrical: A separate 208/240 Volt, 32 A, 60 Hz, 1-phase circuit.

        - UL Listed: E186519, 35HL.

        - Exterior Housing: Aluminum sheet, ASTM B 209, Alloy 5005-AQ-H34.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

        - Finish:

          - Anodized Aluminum Color bronze.

          - Anodized Aluminum Color aluminum.

          - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

    - Single Panel Pass Thru Windows: 600 Series Flush-Mount Window with 3/4 inch glazing.

      - Approvals:

        - Miami-Dade County Product Approval N.O.A. No. 16-0307.06, Large and Small Missile: 11/16-inch Secur-Temp 4 Glazing, ASTM C 1036

        - Miami-Dade County Product Approval N.O.A. No. 16-0307-07, Non-Impact: 3/4-inch insulated glass, ASTM E 773 and E 774.

      - Window Dimensions:

** NOTE TO SPECIFIER ** Select the size required. Delete those not required.

        - 47-1/2 inches wide by 43-1/2 inches high by 4-1/2 inches deep.

        - 47-1/2 inches wide by 35-3/4 inches high by 4-1/2 inches deep.

        - ______ inches wide by ______ inches high by ______ inches deep.

      - Service Panel Operation:

** NOTE TO SPECIFIER ** Select the operator required. Delete those not required.

        - Manual open/self closing.

        - Manual Open/Electronic Release.

        - Fully automatic electric.

      - Service Panel Type: Sliding, 1 panel.

      - Opening Direction: Customer View-Outside

** NOTE TO SPECIFIER ** Select the opening direction required. Delete those not required.

        - Right to left.

        - Left to right.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Security:

** NOTE TO SPECIFIER ** Delect self latch or deadbolt as required. Delete the optional security bar set if not required.

        - Self-latches each time panel closes. Self-latching Adams Rite MS1847 Series with Adams Rite 1000 Series turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn and keyed mortise cylinder.

        - Security bar set.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass required. Delete the paragraphs not required.

        - 3/4-inch insulated glass, ASTM E 773 and E 774.

        - 11/16 inch Secur-Tem 4 ASTM C 1036-91 Smash and Grab

        - 3/4-inch GE Lexgard RC-750 Laminate. 3-ply clear, extruded Lexan polycarbonate laminate with Margard surface. Forced-entry protection security glazing, ASTM F 1233.

        - SP 175 3/4 inch Secur-Tem and Poly ASTM C 1036-91 Level 1

        - 3/4 inch GE Lexgard MP-750 Laminate. 3-ply clear, extruded Lexan polycarbonate and acrylic laminate with Margard surface. Bullet-resistant security glazing, ASTM F 1233. UL 752, Level 1.

      - Silicone Glazing Sealant: Dow Corning 999A, aluminum.

      - Electrical: UL Listed: E186519, 35HL.

** NOTE TO SPECIFIER ** Select the service required. Delete the one not required.

        - 115 V, 60 Hz, with 15 A branch circuit.

        - 220 V, 50/60 Hz, with 8 A branch circuit.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

        - Anodized Aluminum Color bronze.

        - Anodized Aluminum Color aluminum.

        - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

    - Single Panel Pass Thru Windows: 601 Series Flush-Mount Window.

      - Approvals:

        - Miami-Dade County Product Approval N.O.A. No. 16-0307.06, Large and Small Missile: 11/16-inch Secur-Temp 4 Glazing, ASTM C 1036

        - Miami-Dade County Product Approval N.O.A. No. 16-0307-07, Non-Impact: 3/4-inch insulated glass, ASTM E 773 and E 774.

      - Window Dimensions:

** NOTE TO SPECIFIER ** Select the size required. Delete those not required.

        - 47-1/2 inches wide by 60 inches high by 4-1/2 inches deep.

        - 47-1/2 inches wide by 52-3/4 inches high by 4-1/2 inches deep.

        - ______ inches wide by ______ inches high by ______ inches deep.

      - Service Panel Operation:

** NOTE TO SPECIFIER ** Select the operator required. Delete those not required.

        - Manual open/self closing.

        - Manual Open/Electronic Release.

        - Fully automatic electric.

      - Service Panel Type: Sliding, 1 panel.

      - Opening Direction: Customer View-Outside

** NOTE TO SPECIFIER ** Select the opening direction required. Delete those not required.

        - Right to left.

        - Left to right.

      - Security Transaction Drawer: Stainless steel, 18-5/16 inches wide by 9-1/2 inches high by 14-9/32 inches deep transfer area, money clip, change cup. Automatically locks when closed. Manual latch lock. UL-rated Level 1 with UL-rated Level 3 bullet-resistant insert in faceplate.

      - Security Cash Transaction Drawer: Stainless steel, 12 inches wide by 16-3/4 inches long by 3-3/4 inches high, money clip. Manual latch lock. UL-rated Level 3 bullet-resistant insert in faceplate.

      - Speaker System: AIPHONE Model LEM-1, prewired.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Security:

** NOTE TO SPECIFIER ** Delect self latch or deadbolt as required. Delete the optional security bar set if not required.

        - Self-latches each time panel closes. Self-latching Adams Rite MS1847 Series with Adams Rite 1000 Series turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn and keyed mortise cylinder.

        - Security bar set.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass required. Delete the paragraphs not required.

        - 3/4-inch insulated glass, ASTM E 773 and E 774.

        - 11/16 inch Secur-Tem 4 ASTM C 1036-91 Smash and Grab

        - 3/4-inch GE Lexgard RC-750 Laminate. 3-ply clear, extruded Lexan polycarbonate laminate with Margard surface. Forced-entry protection security glazing, ASTM F 1233.

        - SP 175 3/4 inch Secur-Tem and Poly ASTM C 1036-91 Level 1

        - 3/4 inch GE Lexgard MP-750 Laminate. 3-ply clear, extruded Lexan polycarbonate and acrylic laminate with Margard surface. Bullet-resistant security glazing, ASTM F 1233. UL 752, Level 1.

      - Silicone Glazing Sealant: Dow Corning 999A, aluminum.

      - Electrical: UL Listed: E186519, 35HL.

** NOTE TO SPECIFIER ** Select the service required. Delete the one not required.

        - 115 V, 60 Hz, with 15 A branch circuit.

        - 220 V, 60 Hz, with 8 A branch circuit.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

        - Anodized Aluminum Color bronze.

        - Anodized Aluminum Color aluminum.

        - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

    - Single Panel Pass Thru Windows: 603 Series Flush-Mount Window.

      - Approvals:

        - Miami-Dade County Product Approval N.O.A. No. 16-0307.06, Large and Small Missile: 11/16-inch Secur-Temp 4 Glazing, ASTM C 1036

        - Miami-Dade County Product Approval N.O.A. No. 16-0307-07, Non-Impact: 3/4-inch insulated glass, ASTM E 773 and E 774.

      - Window Dimensions:

** NOTE TO SPECIFIER ** Select the size required. Delete those not required.

        - 47-1/2 inches wide by 58-1/2 inches high by 4-1/2 inches deep.

        - 47-1/2 inches wide by 50-3/4 inches high by 4-1/2 inches deep.

        - ______ inches wide by ______ inches high by ______ inches deep.

      - Service Panel: Fixed

      - Security Transaction Drawer: Stainless steel, 18-5/16 inches wide by 9-1/2 inches high by 14-9/32 inches deep transfer area, money clip, change cup. Automatically locks when closed. Manual latch lock. UL-rated Level 1 with UL-rated Level 3 bullet-resistant insert in faceplate.

      - Security Cash Transaction Drawer: Stainless steel, 12 inches wide by 16-3/4 inches long by 3-3/4 inches high, money clip. Manual latch lock. UL-rated Level 3 bullet-resistant insert in faceplate.

      - Speaker System: AIPHONE Model LEM-1, prewired.

      - Interior Counter: Stainless steel, ASTM A 240, Type 304, No.4 finish.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Security: Security bar set.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass required. Delete the paragraphs not required.

        - 3/4-inch GE Lexgard RC-750 Laminate. 3-ply clear, extruded Lexan polycarbonate laminate with Margard surface. Forced-entry protection security glazing, ASTM F 1233.

        - SP 175 3/4 inch Secur-Tem and Poly ASTM C 1036-91 Level 1

        - 3/4 inch GE Lexgard MP-750 Laminate. 3-ply clear, extruded Lexan polycarbonate and acrylic laminate with Margard surface. Bullet-resistant security glazing, ASTM F 1233. UL 752, Level 1.

        - 1-1/4-inch GE Lexgard SP-1250 Laminate. 4-ply clear, extruded Lexan polycarbonate laminate with Margard surface. Bullet-resistant security glazing, ASTM F 1233. UL 752, Level 3.

      - Silicone Glazing Sealant: Dow Corning 999A, aluminum.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

        - Anodized Aluminum Color bronze.

        - Anodized Aluminum Color aluminum.

        - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

      - Air Curtains: Access Air AA300 Heated Air Curtain System.

        - Modes of Operation:

          - Outside warm air flow only.

          - Inside warm air flow only.

          - Both outside and inside warm air flow.

          - Outside fly fan only.

        - Dimensions: 47-1/2 inches wide by 16 inches high by 21-3/4 inches deep.

        - Air Velocity: 600 feet per minute.

        - Heater: 7,100 watts.

        - Electrical: A separate 208/240 Volt, 32 Amp, 60 Hz, 1-phase circuit.

        - UL Listed: E186519, 35HL.

        - Exterior Housing: Aluminum sheet, ASTM B 209, Alloy 5005-AQ-H34.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

        - Finish:

          - Anodized Aluminum: Color dark bronze.

          - Anodized Aluminum: Color clear aluminum.

          - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

    - Bi-Parting Pass-Thru Windows: Ready Access 131 Bi-Parting Slider Window.

      - Opening:

** NOTE TO SPECIFIER ** Select one of the following size paragraphs and delete those not required.

        - Overall Opening 53-1/2 inches wide by 37-3/4 inches high and a Service Opening of 18 inches wide by 31 inches high. Adjustable to 18 inches or 12 inches wide.

        - Overall Opening 53-1/2 inches wide by 43-3/4 inches high and a Service Opening of 18 inches wide by 37 inches high. Adjustable to 18 inches or 12 inches wide.

        - Custom: ________ inches wide by ________ inches high.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required.

        - Manual Open/Self Close.

        - Fully automatic with electronic presence sensor.

      - Door Type: Bi-parting, sliding, two door panels with two fixed side panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Bottom Sill: Track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

** NOTE TO SPECIFIER ** Delete the handle for fully automatic door operation.

      - Handle: Black Delrin handle with pressed-in stainless steel spring pins. Stainless steel handle mounting bracket. Stainless steel spring-loaded pins.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Include the following paragraph for Automatic Electric Open/Close operation only. Select the power required and delete the one not required.

      - Electrical:

        - A separate 115 Volt, 60 Hz. 15 Amp branch circuit.

        - A separate 220 Volt, 60 Hz. 8 Amp branch circuit.

** NOTE TO SPECIFIER ** The following pass-thru window meets health department requirements where health codes require a fly fan.

    - Bi-Parting Pass-Thru Windows With Transom: Ready Access 131 Bi-Parting Slider Window with integrated triple split transom and fly fan..

      - Opening:

** NOTE TO SPECIFIER ** Select one of the following size paragraphs and delete those not required.

        - Overall Opening with Transom 53-1/2 inches wide by 51-1/2 inches high and a Service Opening of 18 inches wide by 31 inches high. Adjustable to 18 inches or 12 inches wide.

        - Overall Opening with Transom 53-1/2 inches wide by 57-1/2 inches high and a Service Opening of 18 inches wide by 37 inches high. Adjustable to 18 inches or 12 inches wide.

        - Custom: ________ inches wide by ________ inches high.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required.

        - Manual Open/Self Close.

        - Fully automatic with electronic presence sensor.

      - Door Type: Bi-parting, sliding, two door panels with two fixed side panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Bottom Sill: Track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

** NOTE TO SPECIFIER ** Delete the handle for fully automatic door operation.

      - Handle: Black Delrin handle with pressed-in stainless steel spring pins. Stainless steel handle mounting bracket. Stainless steel spring-loaded pins.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Include the following paragraph for Automatic Electric Open/Close operation only. Select the power required and delete the one not required.

      - Electrical:

        - A separate 115 Volt, 60 Hz. 15 Amp branch circuit.

        - A separate 220 Volt, 60 Hz. 8 Amp branch circuit.

** NOTE TO SPECIFIER ** The following air curtains are optional accessories. Select the air curtain required or delete if not required. Satisfies requirements for fly fan applications.

      - Air Curtains: Access Air AA100 Fly Fan.

        - Mode of Operation: Outside fly fan.

        - Dimensions: 22 inches wide by 11-3/4 inches high by 13 inches deep.

        - Air Velocity: 600 feet per minute.

        - Electrical: A separate 104/120 Volt, 3 Amp, 60 Hz circuit.

        - UL Listed: E193191, 98AL.

        - Exterior Housing: Aluminum sheet, ASTM B 209, Alloy 5005-AQ-H34.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

        - Finish:

          - Anodized Aluminum: Color dark bronze.

          - Anodized Aluminum: Color clear aluminum.

          - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

      - Air Curtains: Access Air AA275 Pass-Thru Air Curtain System.

        - Modes of Operation:

          - Outside warm air flow only.

          - Inside warm air flow only.

          - Both outside and inside warm air flow.

          - Outside fly fan only.

        - Dimensions: 47-1/2 inches wide by 16 inches high by 21-3/4 inches deep.

        - Air Velocity: 600 feet per minute.

        - Heater: 7,100 watts.

        - Electrical: A separate 208/240 Volt, 32 A, 60 Hz, 1-phase circuit.

        - UL Listed: E186519, 35HL.

        - Exterior Housing: Aluminum sheet, ASTM B 209, Alloy 5005-AQ-H34.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

        - Finish:

          - Anodized Aluminum Color bronze.

          - Anodized Aluminum Color aluminum.

          - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

      - Air Curtains: Access Air AA300 Pass-Thru Air Curtain System.

        - Modes of Operation:

          - Outside warm air flow only.

          - Inside warm air flow only.

          - Both outside and inside warm air flow.

          - Outside fly fan only.

        - Dimensions: 47-1/2 inches wide by 16 inches high by 21-3/4 inches deep.

        - Air Velocity: 600 feet per minute.

        - Heater: 7,100 watts.

        - Electrical: A separate 208/240 Volt, 32 Amp, 60 Hz, 1-phase circuit.

        - UL Listed: E186519, 35HL.

        - Exterior Housing: Aluminum sheet, ASTM B 209, Alloy 5005-AQ-H34.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

        - Finish:

          - Anodized Aluminum: Color dark bronze.

          - Anodized Aluminum: Color clear aluminum.

          - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

      - Air Curtains: Access Air Inside Mount Low-Profile Window Air Curtain System.

** NOTE TO SPECIFIER ** Select the Model required. Delete those not required.

        - Model:

          - RAC-E-22 heated.

          - RAC-22 Unheated.

          - Both outside and inside warm air flow.

          - Outside fly fan only.

        - Dimensions: 26.8 inches wide by 7.1 inches high by 9.1 inches deep with 22.6 inch nozzle width.

        - Air Velocity/Nozzle Hight: 1,050 feet per minute at 36 inches, 926 per minute at 44 inches, 710 per minute at 60 inches,

        - Electrical Requirements:

** NOTE TO SPECIFIER ** Select the Model required. Delete those not required

          - RAC-E-11 Heated 1550 watts. A separate 120 Volt, 20 Amp, 60 Hz, 1-phase circuit

          - RAC-22 Unheated: A separate 120 Volt, 20 Amp, 60 Hz, 1-phase circuit

        - UL Listed: E186519, 35HL.

        - Exterior Housing: Aluminum sheet, ASTM B 209, Alloy 5005-AQ-H34.

** NOTE TO SPECIFIER ** Select Accessories if required. Delete if not required.

          - Accessories; MountingSwitch Package to Integrate Window & Fan Operation. For mounting on window with attached transom or wall.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

        - Finish:

          - Anodized Aluminum: Color black bronze.

          - Anodized Aluminum: Color clear aluminum.

    - Fold Out Pass-Thru Windows: Ready Access Flushmount 3 Window with integrated transom.

      - Opening:

        - Overall Opening with Transom: 27-1/4 inches wide by 41 inches high.

        - Service Opening: 18 inches wide by 18 inches high.

      - Door Operation:

        - Manual.

      - Door Type: Bi-parting, folding outward, 2 door panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Bottom Sill: Track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

          - Anodized Aluminum: Color dark bronze.

          - Anodized Aluminum: Color clear aluminum.

          - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Delete the optional shelf if not required.

      - Shelf: Exterior/interior, stainless steel, 28 inches wide by 10 inches deep.

    - Fold Out Pass-Thru Windows: Ready Access Flushmount 3X Window.

      - Opening:

        - Overall Opening: 27-1/4 inches wide by 26 inches high.

        - Service Opening: 18 inches wide by 18 inches high.

      - Door Operation:

        - Manual.

      - Door Type: Bi-parting, folding outward, 2 door panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Delete the optional shelf if not required.

      - Shelf: Exterior/interior, stainless steel, 28 inches wide by 10 inches deep.

    - Fold Out Pass-Thru Windows: Ready Access Flushmount 3T Window.

      - Opening:

        - Overall Opening: 27-1/4 inches wide by 41 inches high.

        - Service Opening: 18 inches wide by 33 inches high.

      - Door Operation:

        - Manual.

      - Door Type: Bi-parting, folding outward, 2 door panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Delete the optional shelf if not required.

      - Shelf: Exterior/interior, stainless steel, 28 inches wide by 10 inches deep.

  - BUMP OUT PASS-THRU WINDOWS

    - Bump-Out Pass-Thru Windows: Ready Access Bump-Out 2 Window.

      - Opening:

        - Overall Size: 27-3/4 inches wide by 48-3/4 inches high by 15-3/4 inches deep.

        - Service Opening: 19 inches wide by 32 inches high.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required.

        - Semi-Automatic with padded plunger.

        - Fully automatic with electronic presence sensor.

      - Door Type: Bi-parting, sliding, 2 door panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Counter Top: Stainless steel, ASTM A 240, Type 304, No.4 finish. Track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

          - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Specify one of the following electrical requirements for fully automatic door operation.

      - Electrical:

        - A separate 115 Volt, 60 Hz. 15 Amp branch circuit.

        - A separate 220 Volt, 60 Hz. 8 Amp branch circuit.

    - Bump-Out Pass-Thru Windows: Ready access Bump-Out 4 Window.

      - Opening:

        - Overall Size: 27-3/4 inches wide by 48-3/4 inches high by 15-3/4 inches deep.

        - Service Opening: 19 inches wide by 32 inches high.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required.

        - Semi-Automatic with padded plunger.

        - Fully automatic with electronic presence sensor.

      - Door Type: Bi-parting, sliding, 2 door panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Counter Top: Stainless steel, ASTM A 240, Type 304, No.4 finish. Track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Specify one of the following electrical requirements for fully automatic door operation.

      - Electrical:

        - A separate 115 Volt, 60 Hz. 15 Amp branch circuit.

        - A separate 220 Volt, 60 Hz. 8 Amp branch circuit.

    - Bump-Out Pass-Thru Windows: Ready Access Bump-Out 10 Window.

      - Opening:

        - Overall Size: 53-1/2 inches wide by 48-3/4 inches high by 14-1/4 inches deep.

        - Service Opening: 18 inches wide by 23 inches high. Adjustable to 18 inches or 12 inches wide.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required.

        - Manual Open/Self Close.

        - Fully automatic with electronic presence sensor.

      - Door Type: Bi-parting, sliding, 2 door panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Counter Top: Stainless steel, ASTM A 240, Type 304, No.4 finish. Track-free.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Black Delrin handle. Stainless steel handle mounting bracket.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Specify one of the following electrical requirements for fully automatic door operation.

      - Electrical:

        - A separate 115 Volt, 60 Hz. 15 Amp branch circuit.

        - A separate 220 Volt, 60 Hz. 8 Amp branch circuit.

    - Bump-Out Pass-Thru Windows: Ready Access Bump-Out 10 RX Window.

      - Opening:

        - Overall Size: 53-1/2 inches wide by 48-3/4 inches high by 14-1/4 inches deep.

        - Service Opening: 18 inches wide by 23 inches high. Adjustable to 18 inches or 12 inches wide.

      - Door Operation:

** NOTE TO SPECIFIER ** Select one of the following operation paragraphs and delete those not required.

        - Manual Open/Self Close.

        - Fully automatic with electronic presence sensor.

      - Door Type: Bi-parting, sliding, 2 door panels.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6 and 6063-T52.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Counter Top: Stainless steel, ASTM A 240, Type 304, No.4 finish. Track-free.

      - Drop Box: Exterior hinged flap, angled tunnel, 10-inch wide by 6-inch high by 10-inch deep reservoir bin.

      - Automatic Security Lock: Aluminum bar extrusion with sliding spring-loaded locking clip that automatically locks each time the window closes.

      - Manual Security Lock: Thumb turn hook lock.

** NOTE TO SPECIFIER ** Select the optional security bar set if required. Delete if not required.

      - Security Bar Set: Additional security bar for additional after hours security.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Black Delrin handle. Stainless steel handle mounting bracket.

      - Lock: Self-latching Adams Rite MS1847 Series with Adams Rite 1000 Series turn.

      - Glazing:

** NOTE TO SPECIFIER ** Select glass as required. Delete the paragraphs not required.

        - 1/4-inch (6 mm) clear tempered glass, ASTM C 1048.

        - 1/4-inch (6 mm) tinted tempered glass, ASTM C 1048.

      - Silicone Glazing Sealant: Dow Corning 999A:

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required. Additional anodized aluminum finishes requiring additional fabrication time are available.

      - Finish:

        - Anodized Aluminum: Color dark bronze.

        - Anodized Aluminum: Color clear aluminum.

        - Powder Coat Finish: Tiger Drylac RAL#.Color as selected by the Architect from the manufacturer's stock colors.

** NOTE TO SPECIFIER ** Specify one of the following electrical requirements for fully automatic door operation.

      - Electrical:

        - A separate 115 Volt, 60 Hz. 15 Amp branch circuit.

        - A separate 220 Volt, 60 Hz. 8 Amp branch circuit.

    - Bump-Out Pass-Thru Windows: Ready Access Bump-Out 602 Window.

      - Approvals:

        - Miami-Dade County Product Approval N.O.A. No. 16-0307.06, Large and Small Missile: 11/16-inch Secur-Temp 4 Glazing, ASTM C 1036

        - Miami-Dade County Product Approval N.O.A. No. 16-0307-07, Non-Impact: 3/4-inch insulated glass, ASTM E 773 and E 774.

      - Window Dimensions:

** NOTE TO SPECIFIER ** Select the size required. Delete those not required.

        - 47-1/2 inches wide by 60 inches high by 4-1/2 inches deep.

        - 47-1/2 inches wide by 52-3/4 inches high by 4-1/2 inches deep.

        - ______ inches wide by ______ inches high by ______ inches deep.

      - Service Panel Operation:

** NOTE TO SPECIFIER ** Select the operator required. Delete those not required.

        - Manual open/self closing.

        - Manual Open/Electronic Release.

        - Fully automatic electric.

      - Opening Direction: Customer View-Outside

** NOTE TO SPECIFIER ** Select the opening direction required. Delete those not required.

        - Right to left.

        - Left to right.

      - Security Transaction Drawer: Stainless steel, 18-5/16 inches wide by 9-1/2 inches high by 14-9/32 inches deep transfer area, money clip, change cup. Automatically locks when closed. Manual latch lock. UL-rated Level 1 with UL-rated Level 3 bullet-resistant insert in faceplate.

      - Security Cash Transaction Drawer: Stainless steel, 12 inches wide by 16-3/4 inches long by 3-3/4 inches high, money clip. Manual latch lock. UL-rated Level 3 bullet-resistant insert in faceplate.

      - Speaker System: AIPHONE Model LEM-1, prewired.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Security:

** NOTE TO SPECIFIER ** Delect self latch or deadbolt as required. Delete the optional security bar set if not required.

        - Self-latches each time panel closes. Self-latching Adams Rite MS1847 Series with Adams Rite 1000 Series turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn and keyed mortise cylinder.

        - Security bar set.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Handle: Stainless steel.

      - Glazing:

        - 3/4-inch insulated glass, ASTM E 773 and E 774.

        - 11/16 inch Secur-Tem 4 ASTM C 1036-91 Smash and Grab

        - 3/4-inch GE Lexgard RC-750 Laminate. 3-ply clear, extruded Lexan polycarbonate laminate with Margard surface. Forced-entry protection security glazing, ASTM F 1233.

        - SP 175 3/4 inch Secur-Tem and Poly ASTM C 1036-91 Level 1

        - 3/4 inch GE Lexgard MP-750 Laminate. 3-ply clear, extruded Lexan polycarbonate and acrylic laminate with Margard surface. Bullet-resistant security glazing, ASTM F 1233. UL 752, Level 1.

      - Silicone Glazing Sealant: Dow Corning 999A, aluminum.

      - Electrical: UL Listed: E186519, 35HL.

** NOTE TO SPECIFIER ** Select the service required. Delete the one not required.

        - 115 V, 60 Hz, with 15 A branch circuit.

        - 220 V, 60 Hz, with 8 A branch circuit.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

        - Anodized Aluminum Color bronze.

        - Anodized Aluminum Color aluminum.

        - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

    - Bump-Out Pass-Thru Windows: Ready Access Bump-Out 604 Window.

      - Approvals:

        - Miami-Dade County Product Approval N.O.A. No. 16-0307.06, Large and Small Missile: 11/16-inch Secur-Temp 4 Glazing, ASTM C 1036

        - Miami-Dade County Product Approval N.O.A. No. 16-0307-07, Non-Impact: 3/4-inch insulated glass, ASTM E 773 and E 774.

      - Window Dimensions:

** NOTE TO SPECIFIER ** Select the size required. Delete those not required.

        - 47-1/2 inches wide by 60-1/2 inches high by 17 inches deep.

        - 47-1/2 inches wide by 52-3/4 inches high by 17 inches deep.

        - ______ inches wide by ______ inches high by ______ inches deep.

      - Service Panel: None/Fixed

      - Security Transaction Drawer: Stainless steel, 18-5/16 inches wide by 9-1/2 inches high by 14-9/32 inches deep transfer area, money clip, change cup. Automatically locks when closed. Manual latch lock. UL-rated Level 1 with UL-rated Level 3 bullet-resistant insert in faceplate.

      - Security Cash Transaction Drawer: Stainless steel, 12 inches wide by 16-3/4 inches long by 3-3/4 inches high, money clip. Manual latch lock. UL-rated Level 3 bullet-resistant insert in faceplate.

      - Speaker System: AIPHONE Model LEM-1, prewired.

      - Interior Counter: Stainless steel, ASTM A 240, Type 304, No.4 finish.

      - Frame: Extruded aluminum, ASTM B 221, Alloy 6063-T6.

      - Aluminum Sheet: ASTM B 209, Alloy 5005-AQ-H34.

      - Galvanized Steel Sheet: ASTM A 653, G90.

      - Security:

** NOTE TO SPECIFIER ** Delect self latch or deadbolt as required. Delete the optional security bar set if not required.

        - Self-latches each time panel closes. Self-latching Adams Rite MS1847 Series with Adams Rite 1000 Series turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn.

        - Deadbolt lock. Adams Rite MS1850 Series with Adams Rite 4066 Series mortise thumb turn and keyed mortise cylinder.

        - Security bar set.

      - Fasteners: Stainless steel rivets and hex-head zinc-plated self-threading machine screws.

      - Glazing:

        - 3/4-inch insulated glass, ASTM E 773 and E 774.

        - 11/16 inch Secur-Tem 4 ASTM C 1036-91 Smash and Grab

        - 3/4-inch GE Lexgard RC-750 Laminate. 3-ply clear, extruded Lexan polycarbonate laminate with Margard surface. Forced-entry protection security glazing, ASTM F 1233.

        - SP 175 3/4 inch Secur-Tem and Poly ASTM C 1036-91 Level 1

        - 3/4 inch GE Lexgard MP-750 Laminate. 3-ply clear, extruded Lexan polycarbonate and acrylic laminate with Margard surface. Bullet-resistant security glazing, ASTM F 1233. UL 752, Level 1.

      - Silicone Glazing Sealant: Dow Corning 999A, aluminum.

      - Electrical: UL Listed: E186519, 35HL.

** NOTE TO SPECIFIER ** Select the service required. Delete the one not required.

        - 115 V, 60 Hz, with 15 A branch circuit.

        - 220 V, 60 Hz, with 8 A branch circuit.

** NOTE TO SPECIFIER ** Select the finish and color required. Delete those not required.

      - Finish:

        - Anodized Aluminum Color bronze.

        - Anodized Aluminum Color aluminum.

        - Powder Coat Finish Color as selected by the Architect from the manufacturer's stock colors.

        - 2.4FABRICATION

    - Fabricate components with minimum clearances and shim spacing around perimeter of assembly, yet enabling installation and dynamic movement of perimeter seal.

    - Accurately fit and secure joints and corners. Make joints flush, hairline and weatherproof.

    - Prepare components to receive anchor devices. Fabricate anchors.

    - Arrange fasteners and attachments to ensure concealment from view.

    - Prepare components with internal reinforcement for operating hardware as required.

    - Permit internal drainage weep holes and channels to migrate moisture to exterior. Furnish internal drainage of glazing spaces to exterior through weep holes.

    - Factory glaze window units.

  - ALUMINUM FINISH

    - Anodized:

      - Statuary Bronze, AA-M10-C12-C22-A44, ASTM B 580.

      - Clear, AA-M10-C12-C22-A31, ASTM B 680.

    - Powder Coat Painted:

      - Paint: Tiger Drylac Series 49.

      - System: Heated phosphate-cleaned, electrostatic powder-coated, infrared oven-cured.

- EXECUTION

  - EXAMINATION

    - Examine openings and areas to receive pass-thru windows for substrate conditions that would adversely affect installation or subsequent use.

    - If openings or substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

    - Ensure openings to receive pass-thru windows are plumb, level, square, accurately aligned, correctly located, and in tolerance.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Install pass-thru windows plumb, level, square, true to line, and without warp or rack. Maintain dimensional tolerances and alignment with adjacent Work.

    - Install thermal isolation where components penetrate or disrupt building insulation. Pack fibrous insulation in shim spaces at perimeter of assembly to maintain continuity of thermal barrier.

    - Install pass-thru window components weathertight.

    - Anchor pass-thru windows securely in place to supports. Use attachment methods permitting adjustment for construction tolerances, irregularities, alignment, and expansion and contraction.

    - Separate aluminum from other metal surfaces with bituminous coatings or other means approved by Architect.

    - Coordinate installation of related sheet metal flashing as specified in Section 07 62 00 - Sheet Metal Flashing and Trim.

    - Install perimeter joint sealants as specified in Section 07 91 23 - Backer Rods.

** NOTE TO SPECIFIER ** Delete the following sentence if electrical power is not required to operate the pass-thru windows.

    - H.Electrical: Install electrical power as specified in Section 26 05 00 - Common Work Results for Electrical.

    - Repair minor damages to finish in accordance with manufacturer's instructions and as approved by Architect.

    - Remove and replace damaged components that cannot be successfully repaired as determined by Architect.

  - ADJUSTING

    - Adjust doors to be weathertight in closed position.

    - Adjust doors and operating hardware to function properly and for smooth operation without binding.

  - CLEANING

    - Remove protective material from factory finished aluminum surfaces.

    - Clean pass-thru windows promptly after installation in accordance with manufacturer's instructions.

    - Remove excess joint sealant in accordance with sealant manufacturer's instructions.

    - Do not use harsh cleaning materials or methods that would damage glazing or finish.

  - PROTECTION

    - Protect installed products until completion of project.

    - Protect installed pass-thru windows to ensure that, except for normal weathering, pass-thru windows will be without damage or deterioration at time of substantial completion.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION