SECTION 08 35 00

ACCORDION FOLDING DOORS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2019 -    2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Woodfold Mfg. Inc.; interior accordion folding doors.
This section is based on the products of Woodfold Mfg. Inc., which is located at:<BR />1811    18th Ave.<BR />Forest Grove, OR 97116-0346<BR />Tel: 503-357-7181<BR />Fax: 503-357-7185<BR />Email: request info (woodfold@woodfold.com)
Web: http://www.woodfold.com    
 [ Click Here ] for additional information.
Woodfold Manufacturing has been crafting value for over 60 years. We supply custom Accordion Doors, Acoustic Accordion Doors, Hardwood Roll up Doors, (coiling style), and a range of exterior, aluminum products for facades, parking structures, hotels, hospitals, multi-family and mixed use locations, where space and opening ease is important.

- GENERAL

  - SECTION INCLUDES

    - Wood accordion folding doors.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Division 01: Administrative, procedural, and temporary work requirements.

    - Section 08 7100 - Door Hardware.

    - Section 09 9100 - Painting.

    - Section [__ ____ - _______]: Connection to power supply and control wiring.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ASTM International (ASTM):

      - ASTM E90 - Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements.

      - ASTM E336 - Standard Method of Measurement of Airborne Sound Insulation in Buildings.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00.

    - Product Data:

      - Manufacturer's data sheets for door units and hardware.

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Typical installation methods.

    - Verification Samples: For each finish product specified, two samples, minimum size 4 inches (102 mm) square representing actual product, color, and patterns.

    - Shop Drawings: Indicate opening sizes, jamb, sill and head conditions. Indicate materials, construction, and finish. Include relationship with adjacent construction.

** NOTE TO SPECIFIER ** Delete if sustainable design documentation is not required.

    - Sustainable Design Submittals:

      - Recycled Content.

      - Low-Emitting Materials.

** NOTE TO SPECIFIER ** Include the following for submission of closeout submittals for the Owner's record purposes.

    - Closeout Submittals:

      - Operation and Maintenance Data.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum of five years documented experience.

    - Installer Qualifications: Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

    - Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

    - Regulatory Requirements: Comply with applicable accessibility code for door opening force.

** NOTE TO SPECIFIER ** Delete if sustainable design documentation is not required.Verify current recycled content with Woodfold Mfg., Inc.

    - Sustainable Product Requirements:

      - Sheet products fabricated using low-emitting, urea formaldehyde-free binders.

      - Recycled Content, Plywood Core: Minimum ___ percent.

      - Recycled Content, Steel: Minimum 25 percent.

      - Recycled Content, Aluminum: Minimum 25 percent.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Construct a mock-up of one full sized door, showing door construction, hardware and operations with actual materials in sufficient time for Architect's review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

      - Size: One full-size door.

      - Show: Door construction, hardware, and operation.

      - The intent of a mock-up is to demonstrate quality of workmanship and visual appearance.

      - If the mock-up is not acceptable, rebuild the mock-up until satisfactory results are achieved.

      - Retain mock-up during construction as a standard for comparison with completed work.

      - Do not alter or remove mock-up until work is completed or removal is authorized.

      - Approved mock-up may remain as part of the Work.

  - DELIVERY, STORAGE, AND HANDLING

    - Store and handle in strict compliance with manufacturer's written instructions and recommendations.

    - Protect from damage due to weather, excessive temperature, and construction operations.

      - Do not deliver doors until proper protection can be provided and until needed for installation.

      - Store products in manufacturer's unopened packaging until ready for installation.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - WARRANTY

    - Manufacturer's standard limited warranty against manufacturing defects.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Woodfold Mfg. Inc., which is located at:<BR />1811    18th Ave.<BR />Forest Grove, OR 97116-0346<BR />Tel: 503-357-7181<BR />Fax: 503-357-7185<BR />Email: request info (woodfold@woodfold.com);Web: http://www.woodfold.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with the provisions of Section 01 60 00.

** NOTE TO SPECIFIER ** Delete article if not required.

  - ACCORDION FOLDING DOORS

** NOTE TO SPECIFIER ** Light Oak laminate and Chalk laminate door panel coverings are available with Class 1 fire retardant panel core. Delete if not required. **I don't know if this is true - cannot find on their website**

    - Performance Requirements: Class 1 fire retardant door panel core material.

    - Basis of Design: Accordion Folding Doors: Series 240 Door as manufactured by Woodfold Manufacturing, Inc.

      - Standards Compliance: ADA/BOMA SEC 4.1311.

** NOTE TO SPECIFIER ** Delete track options not required.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners for surface mounting.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners, with aluminum hat channel for recess mounting.

      - Track: 1/2 x 1-1/8 inch (13 x 29 mm) aluminum, I-beam profile, with ceiling clips; capable of being formed to 12-inch (305 mm) radius curve.

      - Roller Assembly: Nylon wheels on ball-bearing steel axles; riveted to hinge, dual trolley at lead post, single trolley at alternate panels, type for specified track.

      - Hinge Assembly:18 gauge steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension.

** NOTE TO SPECIFIER ** The following paragraph is for standard panel core material. Delete if specifying decorative inserts.

      - Panel Core: 1/4 inch (6 mm) thick medium density fiberboard with rabbeted edge for panel connector.

** NOTE TO SPECIFIER ** Delete facing options not required then delete finish options not required.

      - Panel Facing: Laminate.

        - Finish: Pattern: _____ on sight-exposed face.

        - Finish: Pattern: As selected by Architect from Manufacturer's full range.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Panel Facing: Hardwood veneer.

        - Species: _____.

        - Species: As selected by Architect.

        - Finish: Sealer and clear lacquer topcoats on sight-exposed face.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Aluminum: Manufacturers standard finish in color complimenting panel finish.

      - Panel Connectors: Continuous non-rigid vinyl, inserted and glued into panel rabbet.

      - Lead Post: Extruded aluminum, 3/4 x 2-3/4 inch (19 x 70 mm) cross-section.

** NOTE TO SPECIFIER ** The following paragraph is for pairs of doors or intersecting configurations. Delete if not required.

      - Intersecting Jamb Molding: Types required for indicated configurations.

      - Handle: Rigid molded polyvinyl chloride.

** NOTE TO SPECIFIER ** Delete latching options not required. Thumbturn both sides with deadlatch is standard.

      - Latching: Thumbturn both sides with deadlatch.

      - Latching: Magnetic catch.

      - Latching: Thumbturn one side with deadlatch.

      - Latching: Thumbturn one side, with keylock on side, with deadlatch.

      - Latching: Keylock both sides, with deadlatch.

** NOTE TO SPECIFIER ** Delete article if not required. or delete the basis of design paragraphs not required.

  - ACCORDION FOLDING SECURITY DOORS

    - Basis of Design: Accordion Folding Security Doors: Series 440 Visifold Door as manufactured by Woodfold Manufacturing, Inc.

      - Standards Compliance: ADA/BOMA SEC 4.1311.

** NOTE TO SPECIFIER ** Delete track options not required.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners for surface mounting.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners, with aluminum hat channel for recess mounting.

      - Track: 1/2 x 1-1/8 inch (13 x 29 mm) aluminum, I-beam profile, with ceiling clips; capable of being formed to 12-inch (305 mm) radius curve.

      - Roller Assembly: Nylon wheels on ball-bearing steel axles; riveted to hinge, dual trolley at lead post, single trolley at alternate panels, type for specified track.

      - Hinge Assembly: 18 gauge steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension.

      - Panel Frame: Aluminum vertical channels, inside dimension between webs to accept 1/8 inch (3 mm) thick material.

** NOTE TO SPECIFIER ** Delete panels option not required.

      - Panels: 1/8 inch (3 mm) thick transparent acrylic sheet.

      - Panels: 1/8 inch (3 mm) thick _____.

      - Panel Connectors: Continuous non-rigid vinyl, inserted into panel frame channel.

      - Lead Post: Extruded aluminum, 3/4 x 2-3/4 inch (19 x 70 mm) cross section.

** NOTE TO SPECIFIER ** The following paragraph is for specifying pairs of doors or intersecting configurations. Delete if not required.

      - Intersecting Jamb Molding: Types for indicated configurations.

      - Handle: Rigid molded polyvinyl chloride.

** NOTE TO SPECIFIER ** Delete latching options not required. Thumbturn both sides with deadlatch is standard.

      - Latching: Thumbturn both sides with deadlatch.

      - Latching: Magnetic catch.

      - Latching: Thumbturn one side with deadlatch.

      - Latching: Thumbturn one side, with keylock one side, with deadlatch.

      - Latching: Keylock both sides, with deadlatch.

** NOTE TO SPECIFIER ** Delete finish options not required.

      - Panel Finish: Clear.

      - Panel Finish: Bronze.

      - Panel Finish: _____.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

    - Basis of Design: Accordion Folding Security Doors: Series 540 Visifold Plus Door as manufactured by Woodfold Manufacturing, Inc.

      - Standards Compliance: ADA/BOMA SEC 4.1311.

** NOTE TO SPECIFIER ** Delete track options not required.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners for surface mounting.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners, with aluminum hat channel for recess mounting.

      - Track: 1/2 x 1-1/8 inch (13 x 29 mm) aluminum, I-beam profile, with ceiling clips; capable of being formed to 12-inch (305 mm) radius curve.

      - Roller Assembly: Nylon wheels on ball-bearing steel axles; riveted to hinge, dual trolley at lead post, single trolley at alternate panels, type for specified track.

      - Hinge Assembly: 18 gage steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension.

** NOTE TO SPECIFIER ** Delete panels option not required.

      - Panels: 1/8 inch (3 mm) thick transparent acrylic sheet.

      - Panels: 1/8 inch (3 mm) thick _____.

      - Panel Connectors: Continuous non-rigid vinyl, inserted into panel frame channel.

      - Lead Post: Extruded aluminum, 3/4 x 2-3/4 inch (19 x 70 mm) cross section.

** NOTE TO SPECIFIER ** The following paragraph is for specifying pairs of doors or intersecting configurations. Delete if not required.

      - Intersecting Jamb Molding: Types for indicated configurations.

      - Handle: Rigid molded polyvinyl chloride.

** NOTE TO SPECIFIER ** Select latching option required. Delete options not required.

      - Latching: Keylock both sides, with deadlatch.

      - Latching: Magnetic catch.

      - Latching: Thumbturn one side with deadlatch.

      - Latching: Thumbturn both sides with springlatch.

      - Latching: Thumbturn one side, with keylock one side, with deadlatch.

      - Latching: Thumbturn both sides with deadlatch.

** NOTE TO SPECIFIER ** Delete panel finish options not required.

      - Panel Finish: Clear.

      - Panel Finish: Bronze.

      - Panel Finish: _____.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

    - Basis of Design: Accordion Folding Security Doors: Series 640 Aumifold Door as manufactured by Woodfold Manufacturing, Inc.

** NOTE TO SPECIFIER ** Delete track options not required.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners for surface mounting.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners, with aluminum hat channel for recess mounting.

      - Track: 1/2 x 1-1/8 inch (13 x 29 mm) aluminum, I-beam profile, with ceiling clips; capable of being formed to 12-inch (305 mm) radius curve.

      - Roller Assembly: Nylon wheels on ball-bearing steel axles; riveted to hinge, dual trolley at lead post, single trolley at alternate panels, type for specified track.

      - Hinge Assembly: 18 gage steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension.

      - Panels: 0.08 inch (2 mm) thick solid aluminum, 4-1/4 inches (108 mm) wide.

      - Panel Connectors: Continuous non-rigid vinyl, inserted into panel frame channel.

      - Lead Post: Extruded aluminum, 3/4 x 2-3/4 inch (19 x 70 mm) cross section.

** NOTE TO SPECIFIER ** The following paragraph is for specifying pairs of doors or intersecting configurations. Delete if not required.

      - Intersecting Jamb Molding: Types for indicated configurations.

      - Handle: Rigid molded polyvinyl chloride.

** NOTE TO SPECIFIER ** Delete latching options not required.

      - Latching: Thumbturn both sides with deadlatch.

      - Latching: Magnetic catch.

      - Latching: Thumbturn one side with deadlatch.

      - Latching: Thumbturn one side, with keylock one side, with deadlatch.

      - Latching: Keylock both sides, with deadlatch.

** NOTE TO SPECIFIER ** Delete panel finish options not required.

      - Panel Finish: Clear anodized aluminum, solid.

      - Panel Finish: Clear anodized aluminum, decorative perforation, nominal 1/2 x 1 inch (13 x 25 mm) openings.

      - Steel Finish: Manufacturer's standard, in color complementing panel finish.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

    - Basis of Design: Accordion Folding Security Doors: Series 740 Alumifold Door as manufactured by Woodfold Manufacturing, Inc.

** NOTE TO SPECIFIER ** Delete track options not required.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners for surface mounting.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners, with aluminum hat channel for recess mounting.

      - Track: 1/2 x 1-1/8 inch (13 x 29 mm) aluminum, I-beam profile, with ceiling clips; capable of being formed to 12-inch (305 mm) radius curve.

      - Roller Assembly: Nylon wheels on ball-bearing steel axles; riveted to hinge, dual trolley at lead post, single trolley at alternate panels, type for specified track.

      - Hinge Assembly: 18 gage steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension.

      - Panels: 0.08 inch (2 mm) thick solid aluminum, 4-1/4 inches (108 mm) wide.

      - Panel Connectors: Continuous non-rigid vinyl, inserted into panel frame channel.

      - Lead Post: Extruded aluminum, 3/4 x 2-3/4 inch (19 x 70 mm) cross section.

** NOTE TO SPECIFIER ** The following paragraph is for specifying pairs of doors or intersecting configurations. Delete if not required.

      - Intersecting Jamb Molding: Types for indicated configurations.

      - Handle: Rigid molded polyvinyl chloride.

** NOTE TO SPECIFIER ** Delete latching options not required.

      - Latching: Keylock both sides, with deadlatch.

      - Latching: Thumbturn both sides with springlatch.

      - Latching: Thumbturn both sides with deadlatch.

** NOTE TO SPECIFIER ** Delete panel finish options not required.

      - Panel Finish: Clear anodized aluminum, solid.

      - Panel Finish: Clear anodized aluminum, decorative perforation, nominal 1/2 x 1 inch (13 x 25 mm) openings.

      - Steel Finish: Manufacturer's standard, in color complementing panel finish.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

** NOTE TO SPECIFIER ** Delete article if not required.

  - ACCORDION FOLDING ROOM DIVIDER DOORS

    - Basis of Design: Accordion Folding Room Divider Doors: Series 220 Door as manufactured by Woodfold Manufacturing, Inc.

      - Standards Compliance: ADA/BOMA SEC 4.1311.

      - Track: 1-1/8 x 1 inch (29 x 25 mm) aluminum, pre-punched for screw fasteners for surface mounting.

      - Roller Assembly: Nylon wheels with Lexan axles; riveted to hinge, single trolley at alternate panels.

      - Hinge Assembly: 18 gage steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension.

      - Panels: 1/4 inch (6 mm) thick medium density fiberboard with rabbeted edge for panel connector.

      - Panel Connectors: Continuous non-rigid vinyl, inserted and glued into panel rabbet.

      - Handle: Rigid molded polyvinyl chloride, full length of door, with molded finger pull front side.

      - Latching: Magnetic catch, with strike plate.

** NOTE TO SPECIFIER ** Delete facing options not required then delete finish options not required.

      - Panel Facing: Laminate.

        - Finish: Pattern: _____ on sight-exposed face.

        - Finish: Pattern: As selected by Architect from Manufacturer's full range.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Panel Facing: Hardwood veneer.

        - Species: _____.

        - Species: As selected by Architect.

        - Finish: Sealer and clear lacquer topcoats on sight-exposed face.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

** NOTE TO SPECIFIER ** Delete article if not required or delete basis of design paragraphs not required.

  - ACCORDION FOLDING ACOUSTIC RATED DOORS

    - Basis of Design: Accordion Folding Acoustic Rated Doors: Series 2100 Door as manufactured by Woodfold Manufacturing, Inc.

      - Sound Transmission Class (STC): 21, tested to ASTM E90.

      - Track: 2 x 1-1/8 inch (51 x29 mm) aluminum, pre-punched for screw fasteners for surface mounting.

      - Head Molding: Hardwood molding for sound seal to top sweep strips.

      - Roller Assembly: Nylon wheels on ball-bearing steel axles; riveted to hinge, dual trolley at lead post, single trolley at alternate panels.

      - Hinge Assembly: 18 gauge steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension.

      - Sound Seal: Extruded vinyl sweeps attached to top and bottom hinge assemblies; 1/2 inch (13 mm) wide sweep for top hinge, 1-3/8 inches (35 mm) wide sweep for bottom hinge.

      - Panels: 1/4 inch (6 mm) thick medium density fiberboard with rabbeted edge for panel connector.

      - Panel Connectors: Continuous non-rigid vinyl, inserted and glued into panel rabbet.

      - Lead Post: Extruded aluminum, 3/4 x 2-3/4 inch (19 x 70 mm) cross section.

      - Interlocking Jamb Molding: Types for indicated configurations; aluminum with sound seal gaskets.

      - Handle: Rigid molded polyvinyl chloride.

** NOTE TO SPECIFIER ** Select latching option required. Delete options not required.

      - Latching: Thumbturn both sides with deadlatch.

      - Latching: Thumbturn one sides with deadlatch.

      - Latching: Thumbturn one side, pull with keylock one side, with deadlatch.

      - Latching: Keylock both sides, with deadlatch.

      - Latching: Magnetic catch.

** NOTE TO SPECIFIER ** Delete facing options not required then delete finish options not required.

      - Panel Facing: Laminate.

        - Finish: Pattern: _____ on sight-exposed face.

        - Finish: Pattern: As selected by Architect from Manufacturer's full range.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Panel Facing: Hardwood veneer.

        - Species: _____.

        - Species: As selected by Architect.

        - Finish: Sealer and clear lacquer topcoats on sight-exposed face.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

** NOTE TO SPECIFIER ** Delete if not required.

    - Basis of Design: Accordion Folding Acoustic Rated Doors: Series 3300 Door as manufactured by Woodfold Manufacturing, Inc.

      - Field Sound Transmission Class (FSTC): 33, tested to ASTM E336.

      - Track: 2-1/2 x 1-3/4 inch (51 x 44 mm) aluminum, pre-punched for screw fasteners for surface mounting.

** NOTE TO SPECIFIER ** Delete head molding or head track whichever is not required.

      - Head Molding: Hardwood molding for sound seal to top sweep strips, painted white.

      - Head Track: Aluminum ceiling guard, two-piece, for sound seal to top sweep strips.

      - Roller Assembly: Nylon wheels on ball-bearing steel axles; rivet-attached to connecting clips, dual trolley at lead post, single trolley at alternate panels, type for specified track.

      - Hinge Assembly: 18 gage steel, continuous pin, riveted to top and bottom of door panel, with automatic stop at full extension; pantograph action for dual-wall door assembly.

      - Panels: 1/4 inch (6 mm) thick medium density fiberboard with rabbeted edge for panel connector.

      - Acoustical Liner: Applied to inside faces of both walls.

      - Panel Connectors: Continuous non-rigid vinyl, inserted and glued into panel rabbet.

      - Lead Post: Extruded aluminum, 2-7/8 x 4-1/2 inch (73 mm x 114 mm) cross section with jamb molding.

** NOTE TO SPECIFIER ** Delete jamb panel options not required.

      - Jamb Panel Type: Flush.

      - Jamb Panel Type: Post.

      - Jamb Panel Type: Sliding.

      - Handle: Rigid molded polyvinyl chloride.

** NOTE TO SPECIFIER ** Select latching option required. Delete options not required.

      - Latching: Thumbturn both sides with deadlatch.

      - Latching: Thumbturn one sides with deadlatch.

      - Latching: Thumbturn one side, with keylock one side, with deadlatch.

      - Latching: Keylock both sides, with deadlatch.

      - Latching: Magnetic catch.

** NOTE TO SPECIFIER ** Delete facing options not required then delete finish options not required.

      - Panel Facing: Laminate.

        - Finish: Pattern: _____ on sight-exposed face.

        - Finish: Pattern: As selected by Architect from Manufacturer's full range.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Panel Facing: Hardwood veneer.

        - Species: _____.

        - Species: As selected by Architect.

        - Finish: Sealer and clear lacquer topcoats on sight-exposed face.

        - Finish: Unfinished.

        - Finish: Special: ________.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

      - Head Molding Finish: Coordinated with hardware and track system.

** NOTE TO SPECIFIER ** Delete if not required. Delete door series model not required. Series 4166 has a 6 inch (152 mm) panel width, Series 4188 has an 8 inch (203 mm) panel width.

    - Accordion Folding Acoustic Rated Doors:

      - Basis of Design: Series 4166. As manufactured by Woodfold Manufacturing, Inc.

      - Basis of Design: Series 4188. As manufactured by Woodfold Manufacturing, Inc.

      - Field Sound Transmission Class (FSTC): 41, tested to ASTM E336.

      - Track: 2 x 1-1/4 inch (51 x 32 mm) aluminum, pre-punched for screw fasteners for surface mounting.

** NOTE TO SPECIFIER ** Delete two of the three following paragraphs. Whichever two are not required.

      - Head Molding: Hardwood molding for sound seal to top sweep strips.

      - Head Track: Aluminum, for sound seal to top sweep strips.

      - Ceiling Guard Track: Aluminum.

      - Roller Assembly: Nylon-tired steel ball-bearing wheels; 4 wheeled carriers on each volute and at lead post, typed specified for track.

      - Hinge Assembly: 16 gage steel, pantograph action for dual-wall door assembly.

      - Panels: 1/2 inch (12 mm) thick medium density fiberboard with routed edge for vinyl hinging.

** NOTE TO SPECIFIER ** Delete panel width option not required.

        - Panel Width Door 4166: 6 inch (152 mm).

        - Panel Width Door 4188: 8 inch (203 mm).

      - Acoustical Liner: Applied to inside faces of both walls.

      - Panel Connectors: Continuous non-rigid vinyl, inserted and attached into routed edge.

      - Lead Post: Extruded aluminum, 3-7/16 x 4-7/8 inch (87 x 124 mm) cross section with jamb molding.

** NOTE TO SPECIFIER ** Delete jamb panel option not required.

      - Jamb Panel: Post type.

      - Jamb Panel: Sliding type.

      - Handle: Steel.

** NOTE TO SPECIFIER ** Select latching option required. Delete options not required.

      - Latching: Thumbturn both sides with deadlatch.

      - Latching: Thumbturn one side with deadlatch.

      - Latching: Thumbturn one side, with keylock one side, with deadlatch.

      - Latching: Keylock both sides, with deadlatch.

** NOTE TO SPECIFIER ** Select panel finish required. Delete finishes not required. A wide variety of non-standard finishes can be applied to standard panels, including custom wood finishes, wallcoverings, plastic laminates, custom logos, and murals. Contact manufacturer for available options.

      - Panel Facing: Laminate.

        - Pattern: _____ on sight-exposed face.

        - Pattern: As selected by Architect from Manufacturer's full range.

      - Aluminum Finish: Manufacturer's standard, in color complementing panel finish.

      - Head Molding Finish: Coordinated with track system.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until the substrates have been properly constructed and prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect in writing of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction without distortion or stress.

    - Fit and align door assembly, including hardware, level and plumb, to provide smooth operation.

    - Adjust doors for smooth operation throughout full operating range.

  - FIELD QUALITY CONTROL

    - Field Inspection: Coordinate field inspection in accordance with appropriate sections in Division 01.

** NOTE TO SPECIFIER ** Include if manufacturer provides field quality control with onsite personnel for instruction or supervision of product installation, application, erection, or construction. Delete if not required.

    - Manufacturer's Services: Coordinate manufacturer's services in accordance with appropriate sections in Division 01.

  - CLEANING AND PROTECTION

    - Clean products in accordance with the manufacturers recommendations.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION