SECTION 09 05 61

SELF-MITIGATING WATERPROOF ACOUSTIC FLOORING INSTALLATION SYSTEM

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2024 -    2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Aquaflex ; Self-Mitigating Waterproof Acoustic Flooring Installation System.
This section is based on the products of Aquaflex , which is located at:<BR />1790 Boyd St.<BR />Santa Ana, CA 92705<BR />Toll Free Tel: 800-359-3201<BR />Email: request info (info@aquaflexinc.com)
Web: https://aquaflexinc.com    
 [ Click Here ] for additional information.
Aquaflex is the world's first patented waterproof acoustic flooring installation system. This revolutionary system consists of the Aquaflex adhesive, a moisture vapor barrier and adhesive in one; Aquaflex Patch & Skim, a silica-free concrete repair mortar; the patented Aquaflex CBD trowel; and the Aquaflex Remover Kit, a bio-based presoaked sponge for easy clean-up. ALL MATERIALS ARE MADE IN THE USA!

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Waterproof silica-free concrete repair mortar.

    - Waterproof self-mitigating acoustic flooring adhesive.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 03 30 00 - Cast-in-Place Concrete.

    - Section 03 39 00 - Concrete Curing.

    - Section 03 54 13 - Gypsum Cement Underlayment.

    - Section 07 16 00 - Cementitious and Reactive Waterproofing.

    - Section 07 26 00 - Vapor Retarders.

    - Section 09 60 00 - Flooring.

    - Section 09 80 00 - Acoustic Treatment.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ACI Committee 201 Report "Guide to Durable Concrete."

      - ACI RAP Bulletin 2, "Crack Repair by Gravity Feed with Resin."

    - ASTM International (ASTM):

      - ASTM C109 and C109M - Standard Test Method for the Compressive Strength of Hydraulic Cement Mortars.

      - ASTM C856 - Standard Practice for Petrographic Examination of Hardened Concrete.

      - ASTM C1583/ACI 503.1R - Standard Test Method for Tensile Strength of Concrete Surfaces and the Bond Strength or Tensile Strength of Concrete Repair and Overlay Materials by Direct Tension (Pull-off Method).

      - ASTM D1308 - Chemical Resistance of Finishes.

      - ASTM D7234 - Standard Test Method for Pull-Off Adhesion Strength of Coatings on Concrete Using Portable Pull-Off Adhesion Tester.

      - ASTM E96 - Standard Test Method for Water Vapor Transmission of Materials; 1995.

      - ASTM E648 - Standard Test Method for Critical Radiant Flux of Floor-Covering Systems using a Radiant Heat Energy Source.

      - ASTM F1869 - Standard Test Method for Measuring Moisture Vapor Emission Rate of Concrete Subfloor Using Anhydrous Calcium Chloride.

      - ASTM F2170 - Standard Test Method for Determining Relative Humidity in Concrete Floor Slabs Using in situ Probes.

      - ASTM F710 - Standard Practice for Preparing Concrete Floors to Receive Resilient Flooring.

      - ASTM F3191 - Standard Practice for Field Determination of Substrate Water Absorption (Porosity) for Substrates to Receive Resilient Flooring.

    - International Concrete Repair Institute (ICRI):

      - Guide 310.2R - Selecting and Specifying Concrete Surface Preparation for Sealers, Coatings, Polymer Overlays and Concrete Repair.

    - South Coast Air Quality Management District (SCAQMD):

      - SQAQMD Rule No 1113 - Architectural Coatings.

    - United States Green Building Council (USGBC):

      - LEED certification or other sustainability certification.

  - DEFINITIONS

    - MVE: Moisture Vapor Emission.

    - MVER: Moisture Vapor Emission Rate; measured in lbs per1000 ft2 / 24 hours.

    - RH: Relative Humidity; measured in percentage.

    - VOC: Volatile Organic Compound; measured in grams/liter.

    - CSP: Concrete Surface Profile defined by ICRI.

    - Delta IIC: Delta Impact Insulation Class; shows what the product adds.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00 - Administrative Requirements.

    - Product Data:

      - Manufacturer's data sheets on each product to be used.

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Typical installation methods.

    - Informational Submittals:

      - Certificate of Compliance: Certification that installed products meet specified performance requirements.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum of five years documented experience.

    - Installer Qualifications: Company specializing in performing Work of this section with minimum five years documented experience with projects of similar scope and complexity.

    - Provide third party verification testing of low emitting compliance to CDPH 01350 small-chamber requirements.

    - Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Construct a mock-up with actual materials in sufficient time for Architect's review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

      - The intent of a mock-up is to demonstrate quality of workmanship and visual appearance.

      - If mock-up is not acceptable, rebuild mock-up until satisfactory results are achieved.

      - Retain mock-up during construction as a standard for comparison with completed work.

      - Do not alter or remove mock-up until work is completed or removal is authorized.

    - Inspection of warranted installation by manufacturers approved representative.

  - PRE-INSTALLATION CONFERENCE

    - Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees shall include Architect, Contractor, Owner, and/or Construction Manager, Installer, Manufacturer and involved trades.

    - Agenda is to Include:

      - Schedule, responsibilities, critical path items and approvals.

      - Project conditions, manufacturer requirements, delivery, and storage, staging and sequencing, and protection of completed work.

  - DELIVERY, STORAGE, AND HANDLING

    - Store and handle in strict compliance with manufacturer's written instructions and recommendations.

      - Store products in manufacturer's unopened packaging until ready for installation.

        - Record product codes and batch numbers and shelf life.

    - Protect from damage due to weather, excessive temperature, and construction operations.

      - Store products in a dry area with temperature maintained between 50 degrees F (10 degrees C) and 90 degrees F (32 degrees C) and protect from direct sunlight.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - WARRANTY

    - Manufacturers standard "Life of the Floor" waterproof and alkali resistant adhesive warranty.

      - Warranty to include repair or replacement of flooring materials damaged during warranty period at no cost to Owner.

      - Underwriters Insurance Coverage with A rating:

        - Limit per Occurrence: 4,000,000 dollars.

    - Installer's 2-year installation workmanship warranty.

  - SEQUENCING

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Aquaflex , which is located at:<BR />1790 Boyd St.<BR />Santa Ana, CA 92705<BR />Toll Free Tel: 800-359-3201<BR />Email: request info (info@aquaflexinc.com);Web: https://aquaflexinc.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with the provisions of Section 01 60 00.

** NOTE TO SPECIFIER ** Delete article not required.

  - MATERIALS

    - Aquaflex Patch and Skim: Waterproof Hydraulic Cement-Based Concrete Surface Repair Mortar. A hydraulic Portland/CSA cement compound for fast-track resurfacing and smoothing of concrete prior to flooring installation. For interior and exterior applications on, above or below grade. Can be installed from a 1/32 to 1/2 inch (1.5 to 13 mm) thick.

      - Exterior rated, no moisture limit, trowel grade mortar to repair concrete prior to application of MVE Control Adhesive.

      - Features:

        - Foot Traffic: Less than one hour.

        - Waterproof.

        - Silica free. Sanding will not produce respirable crystalline silica (RCS).

        - Self-healing properties.

        - Formulated with low CO Green cement.

** NOTE TO SPECIFIER ** Prepares surface of concrete to receive moisture sensitive adhesives and floor coverings. MVE Control System will protect finished flooring from moisture and pH Alkalinity.

    - Aquaflex Adhesive: Self-Mitigating Waterproof Acoustic Flooring Adhesive. A moisture-cure adhesive that has the triple function of reducing MVER, contribute to acoustic attenuation, and providea waterproof adhesive bond.

      - Exterior rated, no moisture limit, trowel grade MVE Control Adhesive.

      - Features:

        - Type: One component, zero VOC content,100 percent solids, waterproof adhesive.

        - Relative Humidity per ASTM F2170: Resistant to 100 percent exposure.

        - Alkalinity Resistance per ASTM F710: Tolerant to 14 pH exposure.

        - Resistant to topical "standing" water intrusions or flood.

        - Sub-slab vapor retarders or sheet materials that minimize the transmission of moisture or water vapor from the sub-slab support system into a concrete slab per ASTM E1745: not required.

        - Moisture Vapor Emissions Rate per ASTM F1869: Moisture vapor emissions reduction up to 80 percent of measured MVER.

        - CHPS Certified (01350), Declare RED LIST Free, LEEDv4 Material.

        - Provides acoustic attenuation with a Delta IIC 14.

- EXECUTION

  - EXAMINATION

    - Verify floors maintain floor levelness of 1/4 inch (6.35 mm) in 10 feet (3.048 m), or in accordance with flooring manufacturers recommendation.

      - Do not begin installation until the substrate has been properly prepared.

    - Allow at least 3 days after placement of concrete to begin work.

    - Concrete substrates must be structurally sound, solid, and meet industry standards as defined in ACI Committee 201 Report "Guide to Durable Concrete."

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

    - Proceed with installation only after unsatisfactory conditions have been corrected. Installation of the Aquaflex; Self-Mitigating Waterproof Acoustic Flooring Installation System indicates acceptance of surfaces and conditions.

  - PREPARATION

    - Pre-installation Testing by independent Testing Agency: Not Required.

    - Mechanically profile surface to porosity meeting ASTM F3191: Not Required.

    - Clean surfaces of dust, debris and contaminates by vacuum, followed by mopping.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

      - Apply Aquaflex Patch and Skim: Waterproof Hydraulic Cement-Based Concrete Surface Repair Mortar in accordance with manufacturer's instructions, matching adjacent surfaces at a maximum depth of 1/2-inch.

        - For 2 inch (13 mm) thickness, apply in two-coats allowing the first coat to set before applying a second.

      - Apply Aquaflex Adhesive: Self-Mitigating Waterproof Acoustic Flooring Adhesive using notched trowel in accordance with manufacturer's instructions.

  - FIELD QUALITY CONTROL

    - Field Inspection: Coordinate field inspection in accordance with appropriate sections in Division 01.

** NOTE TO SPECIFIER ** Include if manufacturer provides field quality control with onsite personnel for instruction or supervision of product installation, application, erection, or construction. Delete if not required.

    - Manufacturer's Services: Coordinate manufacturer's services in accordance with appropriate sections in Division 01.

    - Perform required inspection and install materials required to maintain manufacturer's warranty.

  - CLEANING AND PROTECTION

    - Clean products in accordance with the manufacturers recommendations.

      - Use mineral spirits to clean equipment before the adhesive cures to a hardened state. Cured adhesive found on flooring material can only be removed with provided sponge remover.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION