SECTION 32 31 16

WELDED WIRE FENCES AND GATES

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2024 - 2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** DesignMaster by Deacero; Steel Fencing.
This section is based on the products of DesignMaster by Deacero, which is located at:
8411 Irvington Blvd. , Suite B
Houston, TX 77022
Toll Free Tel: 800-867-6245
Tel: 301-335-3043
Email: request info (greg.robins@deacerousa.com)
Web: https://www.designmasterfence.com  
 [ Click Here ] for additional information.
Need it strong? Strong and beautiful? Count on Designmaster!
LEED Certified , Non-Toxic Powder Coating , 25-Year Limited Warranty , 8 colors
Designmaster is changing the waycountry-regionAmericafences, with steel-strong commercial and residential fence and gate solutions for virtually every need, from maximum security to ornamental projects. Our incomparable quality, product diversity, maintenance-free finishes and competitive pricing give you every reason to choose Designmaster, and our customer-centric attention to detail and 25-year limited warranty ensure your satisfaction and peace of mind.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Maximum Security Steel Fence. (Forte)

    - High Security Steel Fence. (Contempo)

    - Medium Security, high Aesthetics Steel Fence. (Classic) (CityMilan)

    - Standard Security, Maximum Aesthetics. (CityFlorence)

    - Crowd Control Barriers.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Division 01: Administrative, procedural, and temporary work requirements.

    - Section 03 30 00 - Cast-In-Place Concrete.

    - Section 31 23 00 - Excavation and Fill.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ASTM International (ASTM):

      - ASTM A185/A185M - Standard Specification for Steel Welded Wire Reinforcement, Plain, for Concrete.

      - ASTM A513 - Standard Specification for Electric-Resistance-Welded Carbon and Alloy Steel Mechanical Tubing.

      - ASTM A641/A641M - Standard Specifications for Zinc-Coated (Galvanized) Carbon Steel Wire.

      - ASTM A787/A787M - Standard Specification for Electric-Resistance-Welded Metallic-Coated Carbon Steel Mechanical Tubing.

      - ASTM B117 - Standard Practice for Operating Salt Spray (Fog) Apparatus.

      - ASTM C94 - Standard Specification for Ready-Mixed Concrete.

      - ASTM D3359 - Standard Practice for Measuring Adhesion by Tape Test.

    - PlaceNameplaceMiami-DadeCityCounty:

      - HVHZ - High Velocity Hurricane Zone certification.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00.

    - Product Data:

      - Manufacturer's data sheets on each product to be used.

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Typical installation methods.

** NOTE TO SPECIFIER ** Delete if not applicable to product type.

    - Verification Samples: Two representative units of each type, size, pattern, and color.

      - 12 inch (30.5 cm) long post samples.

      - Cap and bracket samples.

** NOTE TO SPECIFIER ** Delete fence panel size option not required.

      - Fence Panel Samples: 12 x 12 inch (305 x 305 mm) fence panel samples.

      - Full sized fence panel samples.

    - Shop Drawings: Include details of materials, construction, and finish. Include relationship with adjacent construction.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum of five years documented experience.

    - Installer Qualifications: Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

    - Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Construct a mock-up with actual materials in sufficient time for Architect's review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

      - The intent of a mock-up is to demonstrate quality of workmanship and visual appearance.

      - Size: Minimum two fence sections x full height.

      - Show: Fence posts, panels, and accessories.

      - Location: Where directed.

      - If the mock-up is not acceptable, rebuild the mock-up until satisfactory results are achieved.

      - Retain mock-up during construction as a standard for comparison with completed work.

      - Do not alter or remove mock-up until work is completed or removal is authorized.

  - PRE-INSTALLATION CONFERENCE

    - Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees shall include Architect, Contractor and trades involved. Agenda shall include schedule, responsibilities, critical path items and approvals.

  - DELIVERY, STORAGE, AND HANDLING

    - Store and handle in strict compliance with manufacturer's written instructions and recommendations.

    - Protect from damage due to weather, excessive temperature, and construction operations.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - WARRANTY

    - Manufacturer's standard limited warranty.

      - Coverage against corrosion of galvanized steel coatings and blistering or loosening of powder coatings.

      - Warranty Period: 25 years.

- PRODUCTS

  - PERFORMANCE REQUIREMENTS

** NOTE TO SPECIFIER ** Miami Dade below applies to Classic, Florence.

    - Standards Compliance: Miami Dade HVHZ, Notice Of Acceptance file number 15-0903.07, EXP 03/30/2022.

  - MANUFACTURERS

    - Acceptable Manufacturer: DesignMaster by Deacero, which is located at:
8411 Irvington Blvd. , Suite B
Houston, TX 77022
Toll Free Tel: 800-867-6245
Tel: 301-335-3043
Email: request info (greg.robins@deacerousa.com);Web: https://www.designmasterfence.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 01 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with the provisions of Section 01 60 00.

** NOTE TO SPECIFIER ** Delete article if not required.

  - MAXIMUM SECURITY STEEL FENCE (FORTE)

    - Basis of Design: Forte, as manufactured by Designmaster.

    - Fence Panels:

      - Resistance welded steel wire mesh, ASTM A185/A185M, 8 gauge Class 1 galvanized steel wire per ASTM A641/A641M, 1/2 x 3 inch (13 x 76 mm) mesh.

** NOTE TO SPECIFIER ** Delete height options not required.

      - Height:

        - Flat Panels: 79 inches (2007 mm).

        - Flat Panels: 99 inches (2515 mm).

        - Flat Panels: 118 inches (2997 mm).

        - Flat Panels: 142 inches (3607 mm).

        - V Brace Panels: 76 inches (1930 mm).

        - V Brace Panels: 96 inches (2438 mm).

      - Wire Breaking Load: Minimum 70,000 psi (482.6 MPa).

      - Weld Shear Strength: Minimum 1,050 pounds (476.3 kg).

      - Wind Load Rating: Minimum 110 mph (177 kph).

    - Posts:

      - Galvanized steel tube, ASTM A513 or ASTM A787, G60 coating class, 3 x 3 inches (76 x 76 mm), 11 gauge.

      - Length: To suit panel height and post mounting method.

      - Post Caps: Ultraviolet-protected plastic, sized to post dimensions, friction fit.

      - Post Bases: Not applicable.

      - Extension Arms: Same material and size as posts, welded to post tops at 45 degree angle, for attachment of:

** NOTE TO SPECIFIER ** Delete attachment option not required.

        - Barbed wire using clips and machine bolts.

        - Fence panels using post brackets.

    - Post Brackets:

      - Galvanized steel and powder coated, with self-tapping stainless steel screw or other appropriate self-tapping screw as required.

    - Cross Bars:

      - Provide universal brackets and self-tapping screws recommended by manufacturer.

** NOTE TO SPECIFIER ** Delete cross bar option not required. Three cross bars are recommended for highest security, to prevent panels from bowing or flexing.

      - Three cross bars per panel.

      - Two cross bars per panel.

    - Gates: Custom-designed and built according to construction documents. Includes hardware.

      - Basis of Design: Forte Gates, as manufactured by Designmaster.

** NOTE TO SPECIFIER ** Delete article if not required.

  - HIGH SECURITY STEEL FENCE (CONTEMPO)

    - Basis of Design: Contempo, as manufactured by Designmaster.

    - Fence Panels:

      - Resistance welded steel wire mesh, ASTM A185/A185M, double 1 gauge horizontal wires with 4 gauge vertical wires between, all are Class 1 galvanized steel wire per ASTM A641/A641M, 2 x 8 inch (51 x 203 mm) mesh, stiffened.

** NOTE TO SPECIFIER ** Delete height options not required.

      - Height: 4 feet (1.2 m).

      - Height: 6 feet (1.8 m).

      - Height: 8 feet (2.4 m).

      - Wire Breaking Load: Minimum 75,000 PSI (517.1 MPa).

      - Weld Shear Strength: Minimum 1,050 pounds (476.3 kg).

    - Posts:

      - Galvanized steel tube, ASTM A513 or ASTM A787, G60 coating class, 3 x 3 inches (76 x 76 mm), 11 gauge.

      - Length: To suit panel height and post mounting method.

      - Post caps: Ultraviolet-protected plastic, sized to post dimensions, friction fit.

      - Post bases: Not applicable.

      - Extension Arms: Same material and size as posts, welded to post tops at 45 degree angle, for attachment of:

** NOTE TO SPECIFIER ** Delete attachment option not required.

        - Barbed wire using clips and machine bolts.

        - Fence panels using post brackets.

    - Post Brackets:

      - Galvanized steel and powder coated, sized to post dimensions, with a 1-1/4 inch galvanized nut and bolt.

    - Gates: Custom-designed and built according to construction documents. Includes hardware.

      - Basis of Design: Contempo Gates, as manufactured by Designmaster.

** NOTE TO SPECIFIER ** Delete article if not required.

  - MEDIUM SECURITY STEEL FENCE (CLASSIC)

    - Basis of Design: Classic, as manufactured by Designmaster.

    - Fence Panels:

      - Resistance welded steel wire mesh, ASTM A185/A185M, 6 gauge Class 1 galvanized steel wire per ASTM A641/A641M, 2 x 6 inch (51 x 152 mm) mesh, stiffened with horizontal V-shaped braces.

** NOTE TO SPECIFIER ** Delete height options not required.

      - Height: 2 feet (0.6 m).

      - Height: 4 feet (1.2 m).

      - Height: 5 feet (1.5 m).

      - Height: 6 feet (1.8 m).

      - Height: 8 feet (2.4 m).

      - Wire Breaking Load: Minimum 80,000 PSI (551.6 MPa).

      - Weld Shear Strength: Minimum 1,050 pounds (476.3 kg).

    - Posts:

      - Galvanized steel tube, ASTM A513 or ASTM A787, G60 coating class.

** NOTE TO SPECIFIER ** Delete Dimensions option not required.

      - Dimensions: 2-1/4 x 2-1/4 inches (57 x 57 mm) inches, 16 gauge (except 12 gauge for posts supporting extension arms).

      - Dimensions: 3 x 3 (76 x 76 mm) inches, 11 gauge.

      - Length: To suit panel height and post mounting method.

      - Post caps: Ultraviolet-protected plastic, sized to post dimensions, friction fit.

      - Post bases: Steel plate bolted to bottom of posts, with four plated steel anchor bolts per base.

      - Extension Arms: Same material and size as posts, welded to post tops at 45 degree angle, for attachment of:

** NOTE TO SPECIFIER ** Delete attachment option not required.

        - Barbed wire using clips and machine bolts.

        - Fence panels using post brackets.

    - Post Brackets:

      - Galvanized steel and powder coated, sized to post dimensions, with a 1-1/4 inch (32 mm) galvanized nut and bolt.

    - Gates: Custom-designed and built according to construction documents. Includes hardware.

      - Basis of Design: Classic Gates, as manufactured by Designmaster.

** NOTE TO SPECIFIER ** Delete article if not required.

  - MEDIUM SECURITY STEEL FENCE (MILAN)

    - Basis of Design: Milan, as manufactured by Designmaster.

    - Fence Panels:

      - Resistance welded steel wire mesh, ASTM A185/A185M, double 1 gauge horizontal with 4 gauge filler, Class 1 galvanized steel wire per ASTM A641/A641M, continuous ornamental wire.

** NOTE TO SPECIFIER ** Delete height option not required.

      - Height: 63 inches (1600 mm).

      - Height: 83 inches (2108 mm).

      - Wire Breaking Load: Minimum 55,000 psi (379.2 MPa).

      - Weld Shear Strength: Minimum 1,050 lbs (476.3 kg).

    - Posts:

      - Extruded Aluminum Tube, ASTM A513 or ASTM A787, custom-designed for the panel.

      - Length: To suit panel height and post mounting method.

      - Post caps: Ultraviolet-protected plastic, sized to post dimensions, friction fit.

      - Post bases: Not applicable.

      - Extension arms: Not applicable for this design.

    - Post Brackets:

      - Galvanized steel and powder coated, sized to post dimensions.

    - Gates: Custom-designed and built according to construction documents. Includes hardware.

      - Basis of Design: Milan Gates, as manufactured by Designmaster.

** NOTE TO SPECIFIER ** Delete article if not required.

  - STANDARD SECURITY STEEL FENCE (FLORENCE)

    - Basis of Design: Florence, as manufactured by Designmaster.

    - Fence Panels:

      - Resistance welded steel wire mesh, ASTM A185/A185M, double 1 gauge horizontal with 4 gauge filler, Class 1 galvanized steel wire per ASTM A641/A641M, continuous ornamental wire.

** NOTE TO SPECIFIER ** Delete height option not required.

      - Height: 31.5 inches (800 mm).

      - Height: 48 inches (1219 mm).

      - Wire Breaking Load: Minimum 55,000 psi (379.2 MPa).

      - Weld Shear Strength: Minimum 1,050 lbs (476.3 kg).

    - Posts:

      - Extruded Aluminum Tube, ASTM A513 or ASTM A787, custom-designed for the panel.

      - Length: To suit panel height and post mounting method.

      - Post caps: Ultraviolet-protected plastic, sized to post dimensions, friction fit.

      - Post bases: Aluminum plate bolted to bottom of posts, with four plated steel anchor bolts per base.

      - Extension arms: Not applicable for this design.

    - Post Brackets:

      - Galvanized steel and powder coated, sized to post dimensions.

    - Gates: Custom-designed and built according to construction documents. Includes hardware.

      - Basis of Design: Florence Gates, as manufactured by Designmaster.

** NOTE TO SPECIFIER ** Delete article if not required.

  - CROWD CONTROL BARRIERS

    - Basis of Design: Crowd Control Barriers, as manufactured by Designmaster.

      - Height: 48 inches (1219 mm).

      - Length: 79 inches (2007 mm).

      - Openings: 2 x 8 inches (51 x 203 mm).

      - Gauge: 6 cal. (4.89 mm).

      - Horizontal Lines: 6 per panel.

      - Vertical Lines: 37 per panel.

      - Pipe Diameter: 1.5 inch (3.8 cm).

  - ACCESSORIES

    - Concrete: Comply with ASTM C94, local building codes, and Division 03 Concrete section as applicable.

  - FINISHES

    - Fence Panels, Posts, and Gates:

      - Polyester powder coated to approximately 4 mils thickness, free of both Triglycidyl Isocyanurate (TGIC) and Volatile Organic Compounds.

** NOTE TO SPECIFIER ** Delete color options not required.

      - Color: Black, RAL 9004.

      - Color: White, RAL 9010.

      - Color: Green, RAL 6005.

      - Color: Brown, RAL 7006.

      - Color: Gray, RAL 7037.

      - Color: Blue, RAL 5003.

      - Color: Red, RAL 3002.

      - Color: Yellow, RAL 1028.

      - Salt Spray Resistance: No rusting or blistering tested to ASTM B117 for 1000 hours.

      - Adhesion: Tested to ASTM D3359, Method B.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until field conditions have been properly constructed.

    - Notify Architect in writing of unsatisfactory conditions before proceeding.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

** NOTE TO SPECIFIER ** Retain installation types below as needed.

    - In-Ground Installation:

      - Drill post holes into undisturbed or compacted soil in accordance with local building codes.

      - Set posts with bottom hole in accordance with local building codes.

      - Place concrete around posts in accordance with local building codes.

      - Pour top of footings in accordance with local building codes.

    - Anchored Installation:

      - Install fencing in accordance with manufacturer's instructions and approved Shop Drawings.

      - Locate and drill holes for post bases.

      - Secure each post with four anchor bolts.

    - Installation Tolerances:

** NOTE TO SPECIFIER ** Revise tolerances below if needed.

      - Maximum Variation from Plumb: 1/4 inch in 10 feet (2 mm in 1 m).

      - Maximum Offset from True Position: 1 inch (25 mm).

  - CLEANING AND PROTECTION

    - Clean products in accordance with the manufacturers recommendations.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION