SECTION 11 68 33.33

SPORTS FIELD EQUIPMENT

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2024 - 2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Douglas Industries, Inc. ; Field Equipment.
This section is based on the products of Douglas Industries, Inc. , which is located at:
3441 S. 11th Ave.
Eldridge, IA 52748
Toll Free Tel: 800-553-8907
Email: request info ()
Web: https://douglas-sports.com  
 [ Click Here ] for additional information.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Za elective court equipment including the following:

      - Ball park netting.

      - Ball park protective padding.

      - Baseball and softball windscreens.

      - Bases, home plates, pitching rubbers and accessories.

      - Batting cages and tunnels, tunnel nets, dividers, hitting stations, Indoor batting tunnel kits, and hitting backdrops:.

      - Outfield Products:

        - Foul poles

        - Fence guards and caps.

        - Distance markers

      - Protective screens.

      - Replacement nets.

      - Wheel kits.

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 32 18 23 - Tennis Court Surfacing.

    - Section 32 31 13.26 - Tennis Court Fences and Gates.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - Selective court equipment in this Section complies with the following:

      - American Sports Builders Association (ASBA).

      - United States Tennis Association (USTA).

      - National Federation of State High School Associations (NFHS).

      - National Collegiate Athletic Association (NCAA).

      - International Amateur Athletic Association (IAFF).

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00.

    - Product Data:

      - Manufacturer's data sheets on each product to be used.

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Typical installation methods.

** NOTE TO SPECIFIER ** Delete if not applicable to product type.

    - Verification Samples: Two representative units of each type, size, pattern, and color.

    - Shop Drawings: Include details of materials, construction, and finish. Include relationship with adjacent construction.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum of five years documented experience.

    - Installer Qualifications: Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

    - Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

  - PRE-INSTALLATION CONFERENCE

    - Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees shall include Architect, Contractor and trades involved. Agenda shall include schedule, responsibilities, critical path items and approvals.

  - DELIVERY, STORAGE, AND HANDLING

    - Store and handle in strict compliance with manufacturer's written instructions and recommendations.

    - Protect from damage due to weather, excessive temperature, and construction operations.

  - PROJECT CONDITIONS

    - Maintain environmental temperature, humidity, and ventilation conditions within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - WARRANTY

    - Manufacturer's standard Limited warranty unless indicated otherwise.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Douglas Industries, Inc. , which is located at:
3441 S. 11th Ave.
Eldridge, IA 52748
Toll Free Tel: 800-553-8907
Email: request info ();Web: https://douglas-sports.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with the provisions of Section 01 60 00.

  - FIELD EQUIPMENT

** NOTE TO SPECIFIER ** Delete paragraph if not required.

    - Ball Park Netting:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Douglas Standard Backstop Cable System: Item No. 66800. For use with Custom backstop wall padding.

** NOTE TO SPECIFIER ** Delete pole options not required. Custom sizes are available.

        - Poles: Steel. Outside Diameter: 10-3/4 inch (273 mm). Wall Thickness: 0.375 inch (9.53 mm). Finish: Powder coat. Color: Black.

        - Poles: Steel. Outside Diameter: ___ inch (___ mm). Wall Thickness: ___ inch (___ mm). Finish: Powder coat. Color: Black.

** NOTE TO SPECIFIER ** Delete netting option not required. Other netting may be substituted.

        - Netting: 1.4 mm, 1-3/4 inch sq. (44 mm), No. 12 Knotted Dyneema, ultra-high molecular polyethylene, UV, and weather resistant, with 1/4 inch (6 mm) solid braid sewn rope boarder. 94 percent visibility factor, 308 lbs. (139.71 kg) break strength, 1500 lbs. (680.39 kg) rope border break strength.

        - Netting: ________.

        - Hardware: 1/4 inch (6 mm) cable, galvanized, with 6100 lbs.(2766.91 kg) break strength, 12 inch (305 mm) eyebolts, 12 inch (305 mm) turnbuckles, 1/4 inch (6 mm) cable clamps, 8 mm carabiner clips.

** NOTE TO SPECIFIER ** Digital printing is optional. Delete if not required.

        - Digital printing.

      - Douglas Standard Backstop Cable Hardware Kit: Item No. 66290. For use with custom netting and poles.

        - Hardware: 1/4 inch (6 mm) cable, galvanized, with 6100 lbs. (2766.91 kg) break strength; 500 ft. (152.4 m), four 12 inch (305 mm) turnbuckles, four 6 inch (152 mm) turnbuckles, thirty 1/4 inch (6 mm) cable clamps, eight 6 inch (152 mm) eyebolts, four hundred 8 mm spring links. Hardware is zinc coated or stainless steel.

      - Douglas Suspended Backstop Cable System: Item No. 66805. For use with custom backstop wall padding.

** NOTE TO SPECIFIER ** Delete pole option not required. Custom sizes are available.

        - Poles: Stee. Outside Diameter: 10-3/4 inch (273 mm). Wall Thickness: 0.375 inch (10 mm). Finish: Powder coat. Color: Black.

        - Poles: Steel. Outside Diameter: ___ inch (___ mm). Wall Thickness: ___ inch (___ mm). Finish: Powder coat. Color: Black.

** NOTE TO SPECIFIER ** Delete netting option not required. Other netting may be substituted.

        - Netting: 1.4 mm, 1-3/4 inch (44 mm), sq., No. 12 Knotted Dyneema, ultra-high molecular polyethylene, UV, and weather resistant, with 1/4 inch (6 mm) solid braid sewn rope boarder. 94 percent visibility factor, 308 lbs. (139.71 kg) break strength, 1500 lbs. (680.39 kg) rope border break strength.

        - Netting: ________.

        - Hardware: 1/4 inch (6 mm) cable, galvanized, with 6100 lbs.(2766.91 kg) break strength, 12 inch (305 mm) eyebolts, 12 inch (305 mm) turnbuckles, 1/4 inch (6 mm) cable clamps, 8 mm carabiner clips, galvanized.

** NOTE TO SPECIFIER ** Digital printing is optional. Delete if not required.

        - Digital printing.

      - Douglas Suspended Backstop Cable Hardware Kit, Tie-Back: Item No. 66295. To be used for custom netting and poles.

        - Hardware: Cable: 1/4 inch (6 mm) galvanized steel; 600 ft. (189 m) Break Strength: 6100 lbs. (2766.91 kg). Six, 12 inch (305 mm) turnbuckles. Four, 6 inch (152 mm) turnbuckles. Forty, 1/4 inch (6 mm) cable clamps. Eight, 6 inch (152 mm) eyebolts. Four Hundred, 8 mm spring links. Hardware to be zinc coated or stainless steel.

      - Indoor Batting Tunnel Accessory Cable Hardware Kit: Item No. 66213. For use with batting tunnel nets.

        - Hardware: Cable: 1/4 inch (6 mm); 500 ft. (152.4 m). Seventy steel c-snaps. Seventy split bracket pulleys. Six turnbuckles. Twelve 1/4 inch (6 mm) wire cable clamps. Six eyebolts. Hardware to be zinc coated or stainless steel.

      - Indoor Batting Tunnel Ceiling Suspension Kit: Item No. 66219. For use with batting tunnel nets.

        - Hardware: Cable: One spool 1/4 inch (6 mm); 500 ft. (152.4 m). Two spools 1/8 inch (3 mm); 1000 ft. (309 m). Sixty 1/8 inch (3 mm) wire clamps. Twelve 1/4 inch (6 mm) wire cable clamps. Six turnbuckles. Six eyebolts. Seventy steel c-snaps. Thirty split bracket pulleys. Hardware to be zinc coated or stainless steel.

      - Indoor Batting Tunnel Tensioning Kit: Item No. 66200. For use with batting tunnel nets.

        - Hardware: Six eyebolts. Six 8 mm spring link clips. Six 3/8 inch (10 mm) spring link clips. Six cables, 15 ft. (4.6 m) long. Three 50 ft. (15 m) lengths. Hoisting Rope: 3/8 inch (10 mm) high strength braided. Three cam lock pulley systems, Three swivel clamps.

        - Maximum Tension Load per Eyebolt: 200 lbs. (90.72 kg), with 24 inch (610 mm) sag in center for net.

      - BLR Braided Lacing Rope 3/16 Inches Poly-Pro, 1,000 Ft. Roll: Item No. 31601M.

        - 3/16 inch (5 mm) braided polypropylene lacing rope, black. UV and weather resistant. 550 lbs. (249.48 kg) break strength.

      - C-Snap, Steel Black: Item No. 32670. 3 inch Carabiner.

      - Cable 1/4 Inches GAC Galvanized Steel: Item No. 34728.

        - 4200 lbs. (1905.09 kg) break strength. Cut to length, sold per foot.

      - Cable Clamp for 1/4 Inches Cable: Item No. 34725.

      - Cable 1/8 Inches Galvanized Steel: Item No. 15102. Cut to length, sold per foot.

      - Cable Clamp for 1/8 Inches Cable: Item No. 34726.

      - Douglas No. 12 Knotted Dyneema Black 1-3/4 Inches SQ: Item No. 26012.

        - 1.4 mm, 1-3/4 inch (44 mm) sq., ultra-high molecular polyethylene, UV, and weather resistant, with 1/4 inch (6 mm) solid braid sewn rope boarder. 94 percent visibility factor, 308 lbs. (139.71 kg) break strength, 1500 lbs. (680.39 kg) rope border break strength.

      - Douglas No. 42 Knotted Ultra HDPE Netting, Black: Item No. 26142.

        - 2.4 mm, 1-3/4 inch (44 mm) sq., knotted HDPE polyethylene, UV, and weather resistant, with 1/4 inch (6 mm) solid braid sewn rope boarder. 94 percent visibility factor, 424 lbs. (192.32 kg) break strength, 5/16 inch (8 mm) rope border, 1710 lbs. (775.64 kg) break strength.

      - Douglas No. 420 Knotless Pylon Netting, 1-1/2 Inches SQ:

** NOTE TO SPECIFIER ** Delete net option not required.

        - Tape Bound with Grommets, Black Net: Item No. 22009.

        - Tape Bound with Grommets, White Net: Item No. 22005.

          - Netting: Weather treated for moisture and sunlight exposure. 125 lbs. (56.7 kg) breaking strength.

          - 1-1/2 inch (38 mm) reinforced vinyl coated polyester, double sewn.

          - Brass grommets.

** NOTE TO SPECIFIER ** Delete grommet spacing option not required.

          - Grommet Spacing: 18 inches (457 mm) along top.

          - Grommet Spacing: ________.

        - Rope Bound: Item No. 22009R.

      - Douglas Complete Netting Systems: Item No. 67200.

** NOTE TO SPECIFIER ** Delete poles options not required.

        - Poles: Steel. Outside Diameter: 10-3/4 inch (273 mm). Wall Thickness: 0.375 inch (10 mm). Finish: Powder coat. Color: Black.

        - Poles: Steel. Outside Diameter: ___ inch (___ mm). Wall Thickness: ___ inch (___ mm). Finish: Powder coat. Color: Black.

        - Height: 40 ft. (12.2 m) maximum. For installation in minimum 24 inch (610 mm) wide concrete footing.

        - Netting: Item No. 26012: No. 12 Knotted Dyneema. 1.4 mm, 1-3/4 inch (44 mm) sq., ultra-high molecular polyethylene, UV, and weather resistant, with 1/4 inch (6 mm) solid braid sewn rope boarder. 94 percent visibility factor, 308 lbs. (139.71 kg) break strength, 1500 lbs. (680.39 kg) rope border break strength.

        - Netting: Item No. 26142: Douglas No. 42 Knotted Ultra HDPE Netting. 2.4 mm, 1-3/4 inch (44 mm) sq., knotted HDPE polyethylene, UV, and weather resistant, with 1/4 inch (6 mm) solid braid sewn rope boarder. 94 percent visibility factor, 424 lbs. (192.32 kg) break strength, 5/16 inch (8 mm) rope border, 1710 lbs. (775.64 kg) break strength.

        - Netting: Item No. 36099B: Douglas Twisted Knotted Nylon Netting, 1-3/4 Inches SQ: No. 36 Medium Weight, Black: High tensile strength, black. UV and weather resistant, 5/16 inch (8 mm) rope border, 1710 lbs. (775.64 kg) break strength. 345 lbs. (156.49 kg) break strength.

        - Netting: Douglas Premium 3.0 mm Knotless Polypropylene, 1-3/4 Inches SQ:

          - High tenacity pylon netting, UV, and weather resistant, 250 lbs. (113.40 kg) break strength.

** NOTE TO SPECIFIER ** Delete color options not required.

            - Color: Black, standard.

            - Color: Dark Green.

            - Color: White.

          - 1-1/2 inch (38 mm) reinforced vinyl coated polyester edging, double-sewn.

** NOTE TO SPECIFIER ** Delete grommet spacing option not required.

          - Brass Grommet Spacing: 18 inches (457 mm) along top.

          - Brass Grommet Spacing: ________.

          - Sewn rope border, black.

        - Netting: Douglas Premium 3.0 mm Knotless Polypropylene, 4 Inches SQ:

          - High tenacity pylon netting, UV, and weather resistant, 250 lbs. (113.40 kg) break strength.

** NOTE TO SPECIFIER ** Delete color options not required.

            - Color: Black, standard.

            - Color: Dark Green.

            - Color: White.

          - Netting: Item No. 25803B: 1-1/2 inch (38 mm) reinforced vinyl coated polyester edging, double-sewn,

** NOTE TO SPECIFIER ** Delete grommet spacing option not required.

          - Brass Grommet Spacing: 18 inches (457 mm) along top.

          - Brass Grommet Spacing: ________.

          - Netting: Item No. 25803BR: Sewn rope border, black.

        - Hardware: 1/4 inch (6 mm) GAC cable, galvanized, with 6100 lbs. (2766.91 kg) break strength, 12 inch (305 mm) eyebolts, TB-12 12 inch (305 mm) turnbuckles, TB-6 6 inch (152 mm) turnbuckles, 1/4 inch (6 mm) cable clamps, 8 mm carabiner clips, galvanized.

      - Douglas Premium 3.0 mm Knotless Polypropylene, 1-3/4 Inches SQ

          - High tenacity pylon netting, UV, and weather resistant, 250 lbs. (113.40 kg) break strength.

** NOTE TO SPECIFIER ** Delete color options not required.

            - Color: Black, standard.

            - Color: Dark Green.

            - Color: White.

          - 1-1/2 inch (38 mm) reinforced vinyl coated polyester edging, double-sewn.

** NOTE TO SPECIFIER ** Delete grommet spacing option not required.

          - Brass Grommet Spacing: 18 inches (457 mm) along top.

          - Brass Grommet Spacing: ________.

          - Sewn rope border, black.

      - Douglas Premium 3.0 mm Knotless Polypropylene, 4 Inches SQ:

          - High tenacity pylon netting, UV, and weather resistant, 250 lbs. (113.40 kg) break strength.

** NOTE TO SPECIFIER ** Delete color options not required.

            - Color: Black, standard.

            - Color: Dark Green.

            - Color: White.

          - Item No. 25803B: 1-1/2 inch (38 mm) reinforced vinyl coated polyester edging, double-sewn.

** NOTE TO SPECIFIER ** Delete grommet spacing option not required.

          - Brass Grommet Spacing: 18 inches (457 mm) along top.

          - Brass Grommet Spacing: ________.

          - Item No. 25803BR: Sewn rope border, black.

      - Douglas Twisted Knotted Nylon Netting, 1-3/4 Inches SQ:

        - High tensile strength, black. UV and weather resistant, 5/16 inch (8 mm) rope border, 1710 lbs. (775.64 kg). break strength.

        - Item No. 36099A: No. 21 Light Weight, Black: 237 lbs. (107.50 kg) break strength.

        - Item No. 36099B: No. 36 Medium Weight, Black: 345 lbs.(156.49 kg) break strength.

        - Item No. 36099C: No. 42 Heavy Weight, Black: 435 lbs. (197.31 kg) break strength.

        - Item No. 36099D: No. 48 Heavy Weight, Black: 472 lbs. (214.10 kg) break strength.

        - Item No. 36099E: No. 60 Heavy Weight, Black: 720 lbs. (326.59 kg) break strength.

        - Item No. 36099G: No. 72 Heavy Weight, Black: 862 lbs. (390.00 kg) break strength.

        - Item No. 36099F: No. 96 Heavy Weight, Black: 1112 lbs. (504.39 kg) break strength.

      - Eye Bolt, Steel, Galvanized. 1/2 x 6 Inches: Item No. 34730.

      - Pully with Split Bracket, 1-7/8 Inch: Item No. 34721.

      - Spring Link Carabiner: 5mm: Item No. 32679.

      - Spring Link Carabiner: 6mm: Item No. 32680.

      - Spring Link Carabiner: 8mm: Item No. 36000.

      - TB-12 Jaw and Eye Turnbuckle, 12 Inch: Item No. 34724.

        - Steel body and fittings, galvanized steel. 1/2 inch (13 mm) thread dia., 12 inch (305 mm) take-up.

      - TB-6 Jaw and Eye Turnbuckle, 6 Inch: Item No. 36986.

        - Steel body and fittings, galvanized steel. 3/8 inch (10 mm) thread dia., 6 inch (152 mm) take-up.

** NOTE TO SPECIFIER ** Delete paragraph if not required.

    - Ball Park Protective Padding:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Logo Vinyl Rail Wraps: Item No. 29219.

        - 18 oz. vinyl, UV, and weather resistant.

        - Color: ________.

        - Color: As determined by the Architect.

        - Digital Printed Graphics: Architect to provide vector graphics.

      - Douglas Custom 3 inch (76 mm) Pad. Thick Wood Back Bolt and Backup Plate. Item No. 26310.

        - Standards Compliance: NFHS, NCAA, IAFF, ASBA compliant.

        - Plywood Backing: 3/4 inch (19 mm), exterior marine grade.

        - Padding: 3 inch (76 mm) thick, high impact, mildew resistant, polyurethane foam.

        - Vinyl Covering: 18 oz. per sq. yd., UV, and weather resistant, for all locations and weather conditions.

        - Back-Up Plates: Steel. Finish: Powder coated. Color: Black.

          - For attachment to back side of chain link fence and bolting to plywood back of padding.

        - Panel Size, Maximum: Up to 4 x 8 ft. (1.2 x 2.5 m).

        - Finished Panel Size: ___ x ___ ft. (___ x ___ m).

** NOTE TO SPECIFIER ** Delete color options not required.

        - Color: Dark Green.

        - Color: Black.

        - Color: Royal Blue.

        - Color: Navy Blue.

        - Color: Red.

        - Color: ________.

        - Color: As determined by the Architect.

        - Digital Printed Graphics: Architect to provide vector graphics.

        - Warranty: Manufacturer's Limited.

      - Douglas Custom 3 inch (76 mm) Pad: Thick Wood Back, Z-Clip Attachment for Concrete, Brick or Wood Walls. Item No. 26300.

        - Plywood Backing: 3/4 inch (19 mm), exterior marine grade.

        - Padding: 3 inch (76 mm) thick, high impact, mildew resistant, polyurethane foam.

        - Vinyl Covering: 18 oz. per sq. yd., UV, and weather resistant, for all locations and weather conditions.

        - Attachment: 2-1/2 inch (64 mm) wide, 1/8 inch (3 mm) thick, extruded aluminum z-clip channel, anchored to top, and bottom length of plywood backing and exterior wall.

        - Panel Size, Maximum: Up to 4 x 8 ft. (1.2 x 2.5 m).

        - Finished Panel Size: ___ x ___ ft. (___ x ___ m).

** NOTE TO SPECIFIER ** Delete color options not required.

        - Color: Dark Green.

        - Color: Black.

        - Color: Royal Blue.

        - Color: Navy Blue.

        - Color: Red.

        - Color: ________.

        - Color: As determined by the Architect.

        - Digital Printed Graphics: Architect to provide vector graphics.

        - Warranty: Manufacturer's Limited.

      - Douglas Custom Sewn Padding with Grommets: Custom, vinyl wrapped, reinforced, double-sewn high-impact polyethylene foam.

** NOTE TO SPECIFIER ** Delete thickness options not required.

        - Foam Padding Thickness: Item No. 29218: 2 inch (51 mm).

        - Foam Padding Thickness: Item No. 29218A: 3 inch (76 mm).

        - Foam Padding Thickness: Item No. 29218B: 4 inch (102 mm).

        - Solid Vinyl Covering: 18 oz. per sq. yd., UV, and weather resistant, antibacterial, antifungal, rot, and mildew resistant.

          - Two, 1-1/2 inch (38 mm) sewn vinyl flaps.

          - No. 2 brass grommets every 6 inches (152 mm), top and bottom.

          - Black Velcro sewn along sides.

        - Top and Bottom Hemmed Flap: Stitched with UV resistant high tensile strength polyester thread.

        - Panel Size, Maximum: Up to 4 x 8 ft. (1.2 x 2.5 m).

        - Finished Panel Size: ___ x ___ ft. (___ x ___ m).

** NOTE TO SPECIFIER ** Delete color options not required.

        - Color: Forest Green.

        - Color: Black.

        - Color: Royal Blue.

        - Color: Navy Blue.

        - Color: Red.

        - Color: ________.

        - Color: As determined by the Architect.

        - Digital Printed Graphics: Architect to provide vector graphics.

        - Warranty: Manufacturer's Limited.

      - Douglas Baseball Dugout Rail Vinyl Covered Padding, Guard Rail Vinyl Covered Padding: Item No. 29217.

        - Standards Compliance: NFHS, NCAA, IAAF, and ASBA.

        - Padding: 1 inch (25 mm) thick polyurethane 100 pus ILD high density foam.

        - Vinyl Covering: 18 oz. sq. yd. UV and weather resistant, with 1-1/2 inch (38 mm) perimeter vinyl flap.

          - Double-sewn with high tensile strength thread.

          - No. 2 brass grommets every 6 inch (152 mm).

** NOTE TO SPECIFIER ** Add length by linear foot. Maximum length is 10 feet. Contact manufacturer for custom sizes. Delete color options not required.

        - Length: ___ft (___mm). Fitting up to 2 inch (51 mm) Dia. Pipe.

        - Color: Dark Green.

        - Color: Black.

        - Color: Royal Blue.

        - Color: Navy Blue.

        - Color: Red.

        - Color: ________.

        - Color: As determined by the Architect.

** NOTE TO SPECIFIER ** The following two paragraphs are optional. Delete either or both if not required.

        - UV Protected Tie Wraps.

        - Digital Printed Graphics: UV digital printing, integrated into material.

        - Architect to provide vector graphics.

** NOTE TO SPECIFIER ** Delete paragraph if not required.

    - Baseball and Softball Windscreens:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Douglas VCP Windscreen: Warranty: 3 Year Limited.

        - Fabric: 9 x 12 weave. Windbreak: 80 percent. Material: 50 percent vinyl, 50 percent polyester open mesh, 8 oz. per sq. yd. Tensile Strength: 230 x 200.

        - Hems: Three-ply, double-sewn, up to 1-1/2 inch (38 mm) wide, with No. 2 brass grommets spaced every 12 inches (305 mm) on panel side and every 18 inches (457 mm) along top and bottom.

        - Center Reinforcement: For screens over 6 ft. (1.9 m), secure with tie wraps. Screens over 6 ft. (1.9 m), high include a center tab with grommets.

** NOTE TO SPECIFIER ** Delete height and length options not required.

        - Height: 6 ft. (1.9 m).

        - Height: 9 ft. (2.7 m).

        - Height: ________.

        - Height: As detailed on the Drawings.

        - Length: 60 ft. (18.3 m) maximum.

        - Length: ________.

        - Length: As detailed on the Drawings.

** NOTE TO SPECIFIER ** Delete air vent option not required.

        - Air Vents: AVR Vents: 10 x 8 inch (254 x 203 mm) cut outs. Perimeter hemmed with polyester webbing. Placed at 10 ft. (3.05 m) intervals.

        - Air Vents: AV Vents: 5 x 8 inch (127 x 203 mm) die cut. Heat sealed. Placed at 10 ft. (3.05 m) intervals.

** NOTE TO SPECIFIER ** Delete color options not required.

        - Color: Dark Green.

        - Color: Black.

        - Color: Open Tournament Blue.

        - Color: Royal Blue.

        - Color: Navy Blue.

        - Color: Red.

** NOTE TO SPECIFIER ** The remaining colors are also available by request.

        - Color: Purple.

        - Color: Orange.

        - Color: Burgundy.

        - Color: Gray.

        - Color: Brown.

        - Douglas Digital Windscreen Printing: Item No. 39995. Warranty: 3-4 Years.

          - UV resistant ink, full color, printed directly on material. Architect to provide vector graphics.

      - Douglas Poly Pro Plus Premium Open Mesh Windscreens. Item No. 21600.

        - Fabric: 28 x 14, 30-1/2 x 12 inch (775 x 305 mm) leno weave. Windbreak: 85 percent. Material: 100 percent polypropylene. Weather and UV resistant. Open mesh. 7 oz. per sq. yd. Tensile Strength: 380 x 185.

        - Hems: Four-ply, double-sewn, up to 1-1/2 inch (38 mm) wide, with solid vinyl inserts and 18 oz. vinyl overlay. No. 2 brass grommets spaced every 12 inches (305 mm) on panel side and every 18 inches (457 mm) along top and bottom.

        - Center Reinforcement: For screens over 6 ft. (1.9 m), secure with tie wraps Screens over 6 ft. (1.9 m) high include a center tab, with double stitched 3 inch (76 mm) wide polyester webbing and grommets placed every 18 inches (457 mm).

** NOTE TO SPECIFIER ** Delete height and length options not required.

        - Height: 6 ft. (1.9 m).

        - Height: 9 ft. (2.7 m).

        - Height: ________.

        - Height: As detailed in the Drawings

        - Length: 60 ft. (18.3 m) maximum.

        - Length: ________.

        - Length: As detailed on the Drawings

        - Air Vents: AVR Vents: 10 x 8 inch (254 x 203 mm) cut outs. Perimeter hemmed with polyester webbing. Placed at 10 ft. (3.05 m) intervals.

** NOTE TO SPECIFIER ** Delete color option not required.

        - Color: Dark Green.

        - Color: Black.

      - Douglas Poly Pro Plus Premium -Closed Mesh: Warranty: 4 Year Limited.

        - Design: 30 x 16 leno weave, 100 percent polypropylene, 95 percent blockage, 8 oz. per sq. yd. UV and weather treated. Tensile Strength: 450 x 215.

        - Hems: Three-ply, double-sewn, up to 1-1/2 (38 mm) wide, seamed with solid vinyl inserts and No. 2 brass grommets spaced every 12 inches (305 mm) on panel side and every 18 inches (457 mm) along top and bottom.

        - Center Reinforcement: For screens over 6 ft. (1.9 m), secured with tie wraps. Screens over 6 ft. (1.9 m) high include a center tab double stitched with 3 inch (76 mm) wide polyester and, with grommets places every 18 inches (457 mm).

** NOTE TO SPECIFIER ** Delete height and weight options not required.

        - Height: 6 ft. (1.9 m).

        - Height: 9 ft. (2.7 m).

        - Height: ________.

        - Height: As detailed on the Drawings.

        - Length: 60 ft. (18.3 m) maximum.

        - Length: ________.

        - Length: As detailed on the Drawings.

        - Air Vents: AVR Vents: 10 x 8 inch (254 x 203 mm) cut outs. Perimeter hemmed with polyester webbing. Placed at 10 ft. (3.05 m) intervals.

      - Douglas VCP Max Windscreen. Warranty: 4 Year Limited.

        - Standards Compliance: ASBA, USTA Compliant.

        - Fabric: 16 x 17 weave. Windbreak: 85 percent windbreak. Material: 50 percent vinyl, 50 percent polyester open mesh, 10 oz. per sq. yd. UV and weather treated. Tensile Strength: 395 x 401.

        - Hems: Three-ply, double-sewn, up to 1-1/2 inch (38 mm) wide, with No. 2 brass grommets spaced every 12 inches (305 mm) on panel side and every 18 inches (457 mm) along top and bottom.

        - Center Reinforcement: For screens over 6 ft. (1.9 m), secured with tie wraps. Screens over 6 ft. (1.9 m) high include a center tab on vinyl coated polyester sewn with 18 oz. black vinyl, with grommets places every 18 inches (457 mm).

** NOTE TO SPECIFIER ** Delete height and length options not required.

        - Height: 6 ft. (1.9 m).

        - Height: 9 ft. (2.7 m).

        - Height: ________.

        - Height: As detailed on the Drawings.

        - Length: 60 ft. (18.3 m) maximum.

        - Length: ________.

        - Length: As detailed on the Drawings.

** NOTE TO SPECIFIER ** Delete air vent option not required.

        - Air Vents: AVR Vents: 10 x 8 inch (254 x 203 mm) cut outs. Perimeter hemmed with polyester webbing. Placed at 10 ft. (3.05 m) intervals.

        - Air Vents: AV Vents: 5 x 8 inch (127 x 203 mm) die cut. Heat sealed. Placed at 10 ft. (3.05 m) intervals.

** NOTE TO SPECIFIER ** Delete color options not required.

        - Color: Open Tournament Blue: Item No. 21644.

        - Color: Black: Item No. 21642.

        - Color: Dark Green: Item No. 21640.

      - Douglas Batter's Eye: Item No. 39995BE.

        - Open mesh, vinyl coated polyester screen. Windbreak: 80 percent.

** NOTE TO SPECIFIER ** Delete size option not required.

        - Size, Maximum (HxL): Up to 20 x 40 ft (6.1 x 2.2 m)

        - Size: ___ x ___ ft. (___ x ___ m).

** NOTE TO SPECIFIER ** Custom digital printing is optional. Delete if not required.

        - Custom Digital Printing: ________.

** NOTE TO SPECIFIER ** Delete bleacher enclosure, privacy screen, color, roll and panel dimensions, and installation options not required.

      - Douglas Bleacher Enclosures: Item No. 21620BZ. Custom fabricated.

        - Standards Compliance: ASBA and USTA.

        - Material: Vinyl coated polyester with three-ply hem, reinforced screen perimeters, and brass grommets spaced every 12 to 18 inches (305 x 457 mm).

        - Architect is to provide measurements, photos, and/or sketches of bleachers and facility layout.

        - Privacy Screen: 150 Ft. Rolls. Material: 85-90 percent knit raschel and 100 percent high density polyethylene, UV stabilized, mildew and mold resistant.

** NOTE TO SPECIFIER ** Delete color options not required.

        - Color: Dark Green.

        - Color: Black.

        - Color: Blue.

        - Color: Brown.

        - Color: Beige.

** NOTE TO SPECIFIER ** Delete role height options not required.

        - Role Height: 44 inch (1118 mm).

        - Role Height: 68 inch (1727 mm).

        - Role Height: 92 inch (2337 mm).

      - Privacy Screen: Finished Panels. Material: 85 to 90 percent knit raschel and 100 percent high density polyethylene fiber, UV stabilized, mildew and mold resistant with hems and grommets.

        - Panel Dimensions and Color:

** NOTE TO SPECIFIER ** Delete size options not required.

          - Size: 68 inch x 50 ft. (1727 mm x 15.25 m), Black. Item No. 26928B.

          - Size: 92 inch x 50 ft. (2337 mm x 15.25 m), Black. Item No. 26932B.

          - Size: 68 inch x 50 ft. (1727 mm x 15.25 m), Dark Green. Item No. 26928G.

          - Size: 92 inch x 50 ft. (2337 mm x 15.25 m), Dark Green. Item No. 26932G.

** NOTE TO SPECIFIER ** Delete tie wrap options not required.

        - Tie Wrap: TR-120P. 8 inch. Item No. 31657. Pack of 100.

          - Break Strength: 120 lbs. (54.43 kg).

        - Tie Wrap: TR-120XL. 14 inch. Item No. 31655. Pack of 100.

          - Break Strength: 120 lbs. (54.43 kg).

        - Tie Wrap: TR-120XL. 17 inch. Item No. 31652. Pack of 100.

          - Break Strength: 120 lbs. (54.43 kg).

        - Tie Wrap: TR-50P. 7 inch. Item No. 31656. Pack of 100.

          - Break Strength: 50 lbs. (22.68 kg).

        - Tie Wrap: TR-50XL. 14 inch. Item No. 31651. Pack of 100.

          - Break Strength: 50 lbs. (22.68 kg).

** NOTE TO SPECIFIER ** Delete paragraph if not required.

    - Bases, Home Plates, Pitching Rubbers and Accessories.

      - Base Accessories:

** NOTE TO SPECIFIER ** Delete accessories options not required.

        - Dual Stanchion Ground Sleeve:

          - Item No. 36592. Official Ground Sleeve. 24 inch (610 mm) length.

          - Item No. 36593. Youth Ground Sleeve. 18 inch (457 mm) length.

        - Base Plug with Indicator: Item No. 36514.

          - High visibility feathered top. Fits into ground anchors. Indicator whisker colors vary.

        - Dig Out Tool: Item No. 36560.

          - Tempered steel with wooden handle. For dirt and debris removal from ground sleeves.

        - Base Ground Sleeve: Item No. 36564. 1-1/2 inch n(38 mm) square steel tubing. 8 inch (203 mm) long.

        - Round Orange Cap Base Anchor Plug: Item No. 36512.

          - Plug: 1 x 1 inch, (25 x 25 mm.) Fits into the 1-1/2 inch (38 mm) Base Ground Sleeve. Safe or use with tractors, drags and rakes.

        - Square Black Base Plug: Item No. 36513.

          - Safe or use with tractors, drags and rakes.

      - Bases:

        - Soft Touch Turf Bases: T Series . Item No. 36546B. Warranty: 1 Year. Rawlings, cut resistant polyurethane, molded, and compressible. For use on synthetic fields with 1 inch (25 mm) turf depth. Not for use with carpet style turf.

          - Base Dimensions: 15 x 15 x 1-1/2 inch (381 x 381 x 38 mm).

          - Accessory Items:

            - Home plates.

            - Pitching rubbers.

            - Single orange base.

        - Orange Soft Touch Turf Base. Item No. 36547. Warranty: 1 Year. Rawlings, cut resistant polyurethane, and compressible. For use with white T15 based on synthetic fields with 1 inch (25 mm) turf depth. Not for use with carpet style turf.

          - Base Dimensions: 15 x 15 x 1-1/2 inch (381 x 381 x 38 mm).

          - Accessory Items:

            - Home plates.

            - Pitching rubbers.

            - Set of three white bases.

        - Hollywood Impact Double First Base:

** NOTE TO SPECIFIER ** Delete item No. not required.

          - Item No. 36528: Double Base.

          - Item No. 36530H: First Base with Two Bases.

          - Rawlings, double first base. Regulation sized white for fielders and orange for outside base. 6 inch (152 mm) stanchions attached to each base. Stabilized with chevron pattern on underside.

          - Base Dimensions: Official 15 x 15 x 2-1/2 inch (381 x 381 x 64 mm).

          - Accessory Items:

            - Ground plugs.

            - Anchors.

        - Hollywood Impact First Base Set: Item No. 36526. Three bases.

          - Rawlings, molded, compressible rubber. 6 inch (152 mm) stanchions attached to each base. Stabilized with chevron pattern on underside.

          - Base Dimensions: Official 15 x 15 x 2-1/2 inch (381 x 381 x 64 mm).

          - Accessory Items:

            - Ground plugs.

            - Anchors.

        - Hollywood Impact Kwik-Release Base Sets: Babe Ruth League, Cal Ripken Baseball and approved for Little League play.

          - Double First Base with Two Bases, Youth: Item No. 36531.

          - Rawlings, compressible base, disengages from platform upon hard impact.

          - Orange Platform: Includes 6 inch (152 mm) stanchion and soft tabs interconnected with tabs on underside of top base.

          - Base Dimensions: Official 15 x 15 x 2-1/2 inch (381 x 381 x 64 mm).

          - Accessory Items:

            - Home plates.

            - Pitching rubbers.

        - Varsity Double First Base with Two Bases: Item No. 36531V.

          - Rawlings compressible base. Disengages from platform upon hard impact. Orange Platform: Includes 6 inch (152 mm) stanchion and soft tabs interconnected with tabs on underside of top base.

          - Base Dimensions: Official 15 x 15 x 2-1/2 inch (381 x 381 x 64 mm).

          - Accessory Items:

            - Ground plugs.

            - Anchors.

        - Impact Throw Down Bases, Set of 3: Item No. 36555.

          - Rawlings flexible, compressible bases with waffle bottom.

          - Base Dimensions: Official 15 x 15 x 2-1/2 inch (381 x 381 x 64 mm).

        - Original Jack Corbett MLB Hollywood Base: Item No. 36518.

          - Three bases with extended tapered lip. UV and weather resistant. Stanchions: 6 inch (152 mm.

          - Size: 15 inch (381 mm) sq.

          - Size: 18 inch (457 mm) sq.

          - Accessory Items:

            - Ground plugs.

            - Anchors.

** NOTE TO SPECIFIER ** Delete home plates and accessory options not required.

      - Home Plates:

        - Soft Touch Turf Home Plate: Item No. 36548. Warranty: 1 Year.

          - Cut resistant polyurethane, molded, compressible. For use on synthetic fields with 1 inch (25 mm) turf depth. Not for use with carpet style turf.

          - Base Dimensions: 18-5/8 x 8-3/4 x 5/8 inch (473 x 222 x 16 mm), with 15/16 inch (24 mm) barbs.

          - Accessory Items:

            - Pitching rubber.

            - Single orange.

            - Set of 3 bases.

        - Bury-All Home Plate: Item No. 36540.

          - Rawlins, non-skid molded rubber with beveled edges, black, and stabilized waffle bottom.

        - Safety First Beveled Edge Five-Spike Home Base Plate: Item No. 36544.

          - Rawlings home plate with beveled edge, black, and five metal spikes.

          - Plate Size: Official.

        - Anchor Home Plate: Item No. 36542.

          - Rawlings Pro, molded rubber with stanchion mounted steel plate and ground anchor.

        - Hollywood MLB Bury-All Home Plate: Item No. 36540H.

          - Rawlings/MLB non-skid molded rubber with waffle bottom for permanent installation.

        - Hollywood MLB Pro Style Home Plate, Universal Mount: Item No. 36545.

          - Rawlings, steel backed molded rubber plate with ground anchor and 6 inch (152 mm) stanchion for permanent installation and spikes, zinc plated, for temporary installation. Include one anchor and one plug.

** NOTE TO SPECIFIER ** Delete pitching rubber options not required.

      - Pitching Rubbers:

        - Soft Touch Turf Pitching Rubber: Item No. 36549. Warranty: 1 Year.

          - Material: Cut resistant polyurethane. For use on synthetic fields with 1 inch (25 mm) turf depth. Not for use with carpet style turf.

          - Dimensions: 24-1/4 x 6-1/8 x 3/4 inch (616 x 156 x 19 mm) with 15/16 inch (24 mm) barbs.

          - Accessories:

            - Home plates.

            - Orange base.

            - Set of three bases.

        - Official Size Hollywood 4-Way MLB Pitching Rubber: Item No. 36572H.

          - Material: Molded rubber with interior aluminum tube. For burying in pitcher's mound.

          - Size: 6 x 24 inch (152 x 610 mm).

        - Economy Pitcher's Rubber with Spikes and Metal Inserts:

          - Item No. 36556: Size: Official: 6 x 24 inch (152 x 610 mm).

          - Item No. 36558: Size: Youth: 4 x 18 inch (102 x 457 mm).

          - Rawlings, heavy-duty rubber with three metal spikes.

        - Pro Step Down Pitching Rubber, Official Size: Item No. 36576H.

          - Rawlings Pro, regulation size, heavy-duty rubber with waffle bottom.

        - Step Down Pitching Rubber - Pitcher's Mound Rubber:

          - Item No. 36576: Official Size: 24 inch (610 mm).

          - Item No. 36578: Youth Size: 18 inch (457 mm).

          - Rawlings, heavy-duty rubber with solid bottom.

** NOTE TO SPECIFIER ** Delete paragraph if not required.

    - Batting Cages, Tunnels, Tunnel Nets and Dividers, Hitting Stations, Indoor Batting Tunnel Kit:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Tunnel Frames

** NOTE TO SPECIFIER ** Delete frame options not required.

        - Douglas Batting Tunnel Frames: Item No. 66216.

        - Douglas Double Batting Tunnel Frames: Item No. 66216D.

        - Douglas Triple Batting Tunnel Frames: Item No. 66216T.

        - Warranty: 5 Year Limited.

        - Standards Compliance: NFHS, NCAA, IAAF, and ASBA.

** NOTE TO SPECIFIER ** Delete length and width options not required.

        - Batting Tunnel Length: 55 ft. (16.75 m).

        - Batting Tunnel Length: 70 ft. (21.3 m).

        - Batting Tunnel Length: 75 ft. (22.9 m).

        - Batting Tunnel Width: 12 ft. (3.7 m).

        - Batting Tunnel Width: 14 ft. (4.3 m).

        - Frame: Upright Poles:

          - Heavy-Duty Steel Tubing: Six, 4 inch (102 mm) square. Wall: 3/16 inch (5 mm). Height: 14 ft. (4.3 m).

          - Reinforcement Steel Crossbars: 2-3/8 inch (60 mm) OD. Wall thickness: 7 ga. Finish: Zinc Flo-coat galvanized, for attachment of steel cables for net attachment, with cross cable.

          - Finish: Powder coated. Color: Black.

        - Ground Sleeves: 4 inch (102 mm) sq., 24 inch (610 mm) aluminum.

        - Maximum Installed Net Height: 12 per ft. (3.7 per m).

        - Hardware:

          - Cables: For attachment to net support crossbars. Includes quick links, turnbuckles, cable clamps and carabiner clips.

          - Cable Size: 1/4 inch (6 mm) diameter. Galvanized, UV, and weather resistant. Break Strength: 6100 lbs. (2766.91 kg).

        - Accessories:

          - Batting tunnel nets.

      - Douglas Max Batting Tunnel Frames: Item No. 66470. Warranty: 7 Year Limited.

        - Standards Compliance: NFHS, NCAA, IAAF, and ASBA.

        - Upright Poles: Heavy-duty steel. 8-5/8 inch (219 mm) diameter. Wall Thickness: 0.322 inch (8 mm) wall thickness. Height: 16 ft. (4.9 m). Direct Bury: 4 ft. (1.23 m).

          - Finish: Powder coated. Color: Black.

        - Maximum Installed Net Height: 11 ft. 6 inch (3.5 m).

        - Hardware: Cable: 1/4 inch (6 mm), galvanized, UV and weather resistant. Break Strength: 6100 lbs. (2766.91 kg). Quick links, turnbuckles, cable clamps and carabiner clips.

        - Accessories:

          - Batting tunnel nets.

      - Douglas Max Double Batting Tunnel Frames: Item No. 66470D. Warranty: 7 Year Limited.

        - Standards Compliance: NFHS, NCAA, IAAF, and ASBA.

        - Included hardware installs two nets.

        - Upright Poles: Heavy-duty steel. 8-5/8 inch (219 mm). Wall Thickness: 0.322 inch (8 mm). Height: 16 ft. (4.9 m).

          - Finish: Powder coated. Color: Black. 4 ft. (1.23 m) direct bury.

        - Double T Poles: 8-5/8 inch (219 mm). Wall Thickness: 0.322 inch (8 mm).Wall Extensions: 6-5/8 x .25 inch (168 x 6 mm).

          - Finish: Powder coated. Color: Black.

        - Maximum Installed Net Height: 11 ft. 6 inch (3.5 m).

        - Hardware: 1/4 inch (6 mm) cable galvanized steel. UV and weather resistant, with 6100 lbs. (2766.91 kg) break strength. Quick links, turnbuckles, cable clamps and carabiner clips.

        - Accessories:

          - Batting tunnel nets.

      - Douglas Max Triple Batting Tunnel Frames: Item No. 66470T. Warranty: 7 Year Limited.

        - Standards Compliance: NFHS, NCAA, IAAF, and ASBA.

        - Included hardware installs three nets.

        - Upright Poles: Heavy-duty steel. 8-5/8 inch (219 mm). Wall Thickness: 0.322 inch (8 mm). Height: 16 ft. (4.9 m).

          - Finish: Powder coated. Color: Black. 4 ft. (1.23 m) direct bury.

        - Double T Poles: 8-5/8 inch (219 mm). Wall Thickness: 0.322 inch (8 mm).Wall Extensions: 6-5/8 x .25 inch (168 x 6 mm).

          - Finish: Powder coated. Color: Black.

        - Maximum Installed Net Height: 11 ft. 6 inch (3.5 m).

        - Hardware: 1/4 inch (6 mm) cable galvanized steel. UV and weather resistant, with 6100 lbs. (2766.91 kg) break strength. Quick links, turnbuckles, cable clamps and carabiner clips.

        - Accessories:

          - Batting tunnel nets.

      - Douglas Hitting Station Systems: Warranty: 5 Years.

** NOTE TO SPECIFIER ** Delete item No. not required.

        - Item No. 66255: Two Poles, 55 ft. (16.75 m).

        - Item No. 66270: Three Poles, 70 ft. (21.3 m) to 75 ft. (22.9 m).

        - Poles: Heavy-duty steel, 4 inch (102 mm) sq. Height: 13 ft. (4 m).

          - Buried in Ground Sleeve: 3 ft (0.9 m)

          - Finish: Powder coated. Color: Black.

        - Netting: No. 42 Twisted Knotted Nylon Black 1-3/4 inch (44 mm) SQ Mesh with Rope Border.

        - Maximum Installed Net Height: 10 ft. (3.05 m).

        - Hardware: Cable: 1/4 inch (6 mm), galvanized steel. UV and weather resistant. Break Strength: 6100 lbs. (2766.91 kg). Turnbuckles, c-snaps, collars, and ground sleeves.

      - Indoor Batting Tunnel Accessory Cable Hardware Kit: Item No. 66213.

        - Hardware required to set up and hang batting tunnel net for wall to wall installation:

          - One Spool: 1/4 inch (6 mm) cable, 500 ft. (152.4 m).

          - Seventy Steel C-Snaps.

          - Seventy Split Bracket Pulleys.

          - Six Turnbuckles.

          - Twelve 1/4 inch (6 mm) Wire Cable Clamps.

          - Six Eyebolts.

      - Indoor Batting Tunnel Ceiling Suspension Kit: Item No. 66219.

        - Hardware required to set up and hang batting tunnel net for wall to wall installation:

          - One Spool: 1/4 inch (6 mm) cable, 500 ft. (152.4 m).

          - Two Spools: 1/8 inch (3 mm) cable, 1000 ft. (309 m).

          - Sixty 1/8 inch (3 mm) Wire Cable Clamps.

          - Twelve 1/4 inch (6 mm) Wire Cable Clamps.

          - Six Turnbuckles.

          - Six Eyebolts.

          - Seventy Steel C-Snaps.

          - Thirty Split Bracket Pulleys.

      - Indoor Batting Tunnel Tensioning Kit: Item No. 66200.

        - Hardware required to set up and hang batting tunnel net for wall to wall installation:

          - Six Eyebolts.

          - Six 8 mm Spring Link Clips.

          - Six 3/8 inch (10 mm) Spring Link Clips.

          - Six Cables, 15 ft. (4.6 m).

          - Three Pieces 3/8 inch (10 mm) High Strength Braided Hoisting Rope, 50 ft. (15.25 m).

          - Three Cam Lock Pulley Systems.

          - Three Swivel Clamps.

      - Pole Pad 4-Sided for 4 inch square Batting Tunnel Pole: Item No. 39032.

        - Form Fitted Pole Protective Padding: For 4 inch (102 mm) square poles.

          - Padding: 2 inch (51 mm) thick dense foam. UV and weather resistant.

            - Height: 72 inch (1829 mm).

          - Cover: Vinyl. Secured by sewn-in Velcro straps.

** NOTE TO SPECIFIER ** Delete color options not required.

          - Color: Dark Green.

          - Color: Black.

          - Color: Royal Blue.

          - Color: Navy Blue.

          - Color: Red.

      - Vinyl Hitting Backdrop: Item No. 26460.

        - Material: 18 oz. sq. yd., vinyl. Color: Black. Reinforced perimeter hem with grommets. Size: 6 x 5 ft (1.9 x1.5 m).

        - Includes C-Snaps.

** NOTE TO SPECIFIER ** The following is optional. Delete if not required.

        - White printed hitting target.

      - Douglas No. 42 Knotted Twisted HDPE Batting Tunnel Nets: Warranty: 3 Years.

** NOTE TO SPECIFIER ** Delete size options not required.

        - Size: Item No. 36042: 12 x 12 x 35 ft. (3.7 x 3.7 x 10.7 m).

        - Size: Item No. 36048: 12 x 12 x 55 ft. (3.7 x 3.7 x 16.75 m).

        - Size: Item No. 36043: 12 x 12 x 70 ft. (3.7 x 3.7 x 21.3 m).

        - Size: Item No. 36046: 12 x 14 x 55 ft. (3.7 x 4.3 x 16.75 m).

        - Size: Item No. 36045: 12 x 14 x 70 ft. (3.7 x 4.3 x 21.3 m).

        - Size: Item No. 36049: 12 x 14 x 75 ft. (3.7 x 4.3 x 22.9 m).

        - Netting: No. 42 knotted, twisted HDPE, 1-3/4 inch (44 mm) sq. mesh. Sewn Rope Border: 5/16 inch (8 mm). Rope Tails: 6 inch (152 mm). Sewn hanging loops. Top center rope and overlap-style entryways. UV, weather, and mildew resistant. Break Strength: 290 lbs. (131.54 kg).

      - Douglas BT-PRO 3.5 Knotted Braided HDPE Batting Tunnel Nets: Warranty: 4 Years.

** NOTE TO SPECIFIER ** Delete size options not required. Custom sizes are available .

        - Size: Item No. 36038: 12 x 12 x 55 ft. (3.7 x 3;7 x 16.75 m).

        - Size: Item No. 36033: 12 x 12 x 70 ft. (3.7 x 3.7 x 21.3 m).

        - Size: Item No. 36036: 12 x 14 x 55 ft. (3.7 x 4.3 x 16.75 m).

        - Size: Item No. 36035: 12 x 14 x 70 ft. (3.7 x 4.3 x 21.3 m).

        - Size: Item No. 36039: 12 x 14 x 75 ft. (3.7 x 4.3 x 22.9 m).

        - Size: __ x __ x __ ft. (__ x __ x ___ m)

        - Standards Compliance: NFHS, NCAA, IAAF, and ASBA.

        - Netting: Knotted, braided HDPE, 1-3/4 inch (44 mm) sq. mesh with 5/16 inch (8 mm) sewn rope border. Top center rope and overlap-style entryways. UV, weather, and mildew resistant. Break Strength: 325 lbs. (147.42 kg)

          - Reinforced vinyl top and center border.

      - Batting Tunnel Net Saver/Divider:

** NOTE TO SPECIFIER ** Delete size option not required.

        - Size: 12 x 12 ft. (3.7 x 3.7 m).

        - Size: 12 x 14 ft. (3.7 x 4.3 m).

        - Netting Panel: No. 48 knotted, twisted nylon netting, black. 1-3/4 inch (44 mm) sq. mesh with sewn rope border. UV and weather resistant. Break Strength: 459 lbs. (208.20 kg).

** NOTE TO SPECIFIER ** Delete paragraph if not required.

    - Outfield Products:

** NOTE TO SPECIFIER ** Delete products not required. Then delete size options not required.

      - Aluminum Foul Poles: Semi-permanent installation.

        - Size: 15 ft. (4.6 m) Above Ground: Item No. 36651.

          - Material: Aluminum, 4 inch (102 mm) OD pipe, with caps or base plate mounting system.

** NOTE TO SPECIFIER ** Pole is available with or without wing. Delete if not required.

          - Wing: 7 ft (2.1 m). Material: 1/8 inch (3 mm) stamped aluminum sheet wing with double reinforced bends, beginning at 8 ft. (2.5 m) above finished grade.

          - Finish: Powder coated. Color: Yellow.

          - Mounting: Ground sleeved.

          - Mounting: Plate.

        - Size: 20 ft. (6.1 m) Above Ground: Item No. 36652.

          - Material: Aluminum, 4 inch (102 mm) OD pipe, with caps or base plate mounting system.

** NOTE TO SPECIFIER ** Pole is available with or without wing. Delete if not required.

          - Wing: 12 ft. (3.7 m). Material: 1/8 inch (3 mm) stamped aluminum sheet wing with double reinforced bends, beginning at 8 ft. (2.5 m) above finished grade.

          - Finish: Powder coated. Color: Yellow.

          - Mounting: Ground sleeved.

          - Mounting: Plate.

        - Size: ___ ft. (___ mm) Above Ground.

          - Material: Aluminum, 4 inch (102 mm) OD pipe, with caps or base plate mounting system.

** NOTE TO SPECIFIER ** Pole is available with or without wing. Delete if not required.

          - Wing: ___ft., ___ m).Material: 1/8 inch (3 mm) stamped aluminum sheet wing with double reinforced bends, beginning at 8 ft. (2.5 m) above finished grade.

          - Finish: Powder coated. Color: Yellow.

          - Mounting: Ground sleeved.

          - Mounting: Plate.

        - Size: 30 ft. (9.1 m) Above Ground: Item No. 36653.

          - Material: Aluminum, 6.625 inch (168 mm) OD pipe, with caps or base plate mounting system.

** NOTE TO SPECIFIER ** Pole is available with or without wing. Delete if not required.

          - Wing: 22 ft. (6.7 m). Material: 1/8 inch (3 mm) stamped aluminum sheet wing with double reinforced bends, beginning at 8 ft. (2.5 m) above finished grade.

          - Finish: Powder coated. Color: Yellow.

          - Mounting: Ground sleeved.

          - Mounting: Plate.

        - Size: 40 ft. (12.2 m) Above Ground: Item No. 36654.

          - Material: Aluminum, 6.625 inch (168 mm) OD pipe, with caps or base plate mounting system.

** NOTE TO SPECIFIER ** Pole is available with or without wing. Delete if not required.

          - Wing: 32 ft. (9.8 m). Material: 1/8 inch (3 mm) stamped aluminum sheet wing with double reinforced bends, beginning at 8 ft. (2.5 m) above finished grade.

          - Finish: Powder coated. Color: Yellow.

          - Mounting: Ground sleeved.

          - Mounting: Plate.

        - Size: ___ ft. (___ m) Above Ground:

          - Material: Aluminum, 6.625 inch (168 mm) OD pipe, with caps or base plate mounting system.

** NOTE TO SPECIFIER ** Pole is available with or without wing. Delete if not required.

          - Wing: ___ ft. (___ m). 1/8 inch (3 mm) stamped aluminum sheet wing with double reinforced bends, beginning at 8 ft. (2.5 m) above finished grade.

          - Finish: Powder coated. Color: Yellow.

          - Mounting: Ground sleeved.

          - Mounting: Plate.

      - Protective Fence Guard for Chain Link Fences:

        - Item No. 36982. Premium. Warranty: 3 Years.

          - Size: Twelve, 4 x 2-5/8 inch (102 x 92 mm) x 7 ft. (2.1 m) sections. 84 ft. (25.6 m) total.

          - Material: Heavy-duty polyethylene, teardrop profile. UV resistant. Pre-drilled every 24 inch (610 mm). Wall thickness 0.10 inches

          - Zip-Ties: included.

** NOTE TO SPECIFIER ** Delete color options not required.

          - Color: Forest Green.

          - Color: Red.

          - Color: Blue.

          - Color: Orange.

          - Color: White.

          - Color: Black.

        - Item No. 36984. Standard. Warranty: 5 Years.

          - Size: Twelve, 4-1/2 x 3 inch (114 x 76 mm) x 7 ft. (2.1 m) sections. 84 ft. (25.6 m) total.

          - Material: Heavy-duty polyethylene, teardrop profile. UV resistant. Pre-drilled every 24 inch (610 mm). Wall thickness 0.07 inches

          - Zip-Ties: included.

** NOTE TO SPECIFIER ** Delete color options not required.

          - Color: Yellow.

          - Color: Red.

          - Color: Blue.

      - Poly-Cap, Protective Fence Cap for Chain-Link Fences: Warranty: 2 Years.

** NOTE TO SPECIFIER ** Delete roll option not required.

        - Unit Size: Roll, 100 ft.: Item No. 36960.

        - Unit Size: Roll, 250 ft.: Item No. 36966.

        - Standards Compliance: ASBA.

        - Material: 4-1/2 inch (114 mm) diameter and pre-slit. Polyethylene. UV and weather resistant.

** NOTE TO SPECIFIER ** Delete color options not required.

        - Color: Bright Safety Yellow.

        - Color: Black.

      - Professional Foul Pole Pairs: For direct bury or installation in ground sleeves. Heavy-Duty Steel. Warranty: 1 Year.

** NOTE TO SPECIFIER ** Delete size options not required.

        - Size Above Ground: 12 ft. (3.7 m). Wings: 6 ft. (1.9 m). Item No. 36650B.

        - Size Above Ground: 15 ft. (4.6 m). Wings: 8 ft. (2.5 m). Item No. 36650A.

        - Size Above Ground: 20 ft. (6.1 m) Wings: 12 ft. (3.7 m. Item No. 36650.

        - Ground Sleeves: Item No. 36650GS.

        - Standards Compliance: NFHS, NCAA, IAAF, and ASBA compliant.

        - Material: 3-1/2 inch (89 mm) OD. Wall thickness: 13 ga.

          - Wing: 18 inch (457 mm) wide expanded metal.

        - Finish: Powder coated. Color: Bright Safety Yellow.

      - Vinyl Mesh Outfield Distance Markers:

        - Windscreen/banner with reinforced, double-stitched perimeter and grommets.

** NOTE TO SPECIFIER ** Delete item options not required.

        - Item No. 36815A. Each.

        - Item No. 36815B. Set of Three.

        - Item No. 36815C. Set of Five.

        - Number Height: 16 inch (406 mm). Permanently UV printed.

        - Vertical Banner Dimensions: 60 x 24 inch (1524 x 610 mm).

        - Horizontal Banner Dimensions: 24 x 48 inch (610 x 1219 mm).

** NOTE TO SPECIFIER ** Delete color options not required.

        - Banner Color: Black.

        - Banner Color: Dark Green.

        - Banner Color: Royal Blue.

        - Banner Color: Navy Blue.

        - Banner Color: Purple.

        - Banner Color: Burgundy.

        - Banner Color: Red.

        - Banner Color: Brown.

        - Banner Color: Orange.

        - Banner Color: Yellow.

        - Banner Color: Gray.

        - Banner Color: ________.

        - Banner Color: As determined by the Architect or as detailed on the Drawings.

        - Printing Color: ________.

        - Printing Color: As determined by the Architect or as detailed on the Drawings.

        - Printed Numbers: ________.

        - Printed numbers: As determined by the Architect or as detailed on the Drawings.

      - Vinyl Outfield Distance Markers:

** NOTE TO SPECIFIER ** Delete item options not required.

        - Item No. 36816A. Each.

        - Item No. 36816B. Set of Three.

        - Item No. 36816C. Set of Five.

        - Material: 18 oz. viny. Reinforced, double-stitched perimeter and grommets.

        - Number Height: 16 inch (406 mm). Permanently UV printed.

        - Vertical Banner Dimensions: 60 x 24 inch (1524 x 610 mm).

        - Horizontal Banner Dimensions: 24 x 48 inch (610 x 1219 mm).

** NOTE TO SPECIFIER ** Delete color options not required.

        - Banner Color: Black.

        - Banner Color: Dark Green.

        - Banner Color: Royal Blue.

        - Banner Color: Navy Blue.

        - Banner Color: Purple.

        - Banner Color: Red.

        - Banner Color: Yellow.

        - Banner Color: Gray.

        - Banner Color: White.

        - Banner Color: ________.

        - Banner Color: As determined by the Architect or as detailed on the Drawings.

        - Printing Color: ________.

        - Printing Color: As determined by the Architect or as detailed on the Drawings.

        - Printed Numbers: ________.

        - Printed numbers: As determined by the Architect or as detailed on the Drawings.

** NOTE TO SPECIFIER ** Delete paragraph options not required.

    - Protective Screens:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Varsity Field Screen, 10 x 10 ft.: Item No. 36465.

        - Frame: Steel tube. 1-1/2 inch (38 mm) OD. Welded gussets on each leg. Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Varsity Field Screen, 6 x 4 ft.: Item No. 36468.

        - Frame: Steel tube, 1-1/2 inch (38 mm) OD. Welded gussets on each leg. Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Varsity Field Screen, 7 x 7 ft.: Item No. 36463.

        - Frame: Steel tube, 1-1/2 inch (38 mm) OD. Welded gussets on each leg, Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Varsity Field Screen, 8 x 10 ft.: Item No. 36464.

        - Frame: Steel tube, 1-1/2 inch (38 mm) OD. Welded gussets on each leg, Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Varsity L-Screen, 7 x 7 ft.: Item No. 36460.

        - Frame: Steel tube, 1-1/2 inch (38 mm) OD. Welded gussets on each leg. 3 x 3 ft (0.9 m) cutout. Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Varsity Mini L-Screen, 5 x 7 ft.: Item No. 36461.

        - Frame: Steel tube, 1-1/2 inch (38 mm) OD. Welded gussets on each leg. Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Varsity Sock Net Screen, 7 x 7 ft.: Item No. 36466.

        - Frame: Steel tube, 1-1/2 inch (38 mm) OD. Welded gussets on each leg. Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

        - Target: 49 x 53 inch (1245 x 1346 mm).

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Varsity Softball Pitching Screen, 7 x 7 ft.: Item No. 36467.

        - Frame: Steel tube, 1-1/2 inch (38 mm) OD. Welded gussets on each leg. Bolted installation. Finish: Powder coated. Color: Black.

        - Net: No. 60. Braided. Color: Black. UV, and weather resistant. Pillow case style.

        - Cutout: 30 x 24 inch (762 x 610 mm), 21 inch (533 mm) from ground.

** NOTE TO SPECIFIER ** The Varsity Wheel Kit is optional. Delete if not required.

        - Varsity Wheel Kit: Item No. 36469. Airless tires.

          - Slip on and slip off installation with tightening screw.

      - Douglas Replacement Slip-On Style Net: 7x5 ft. Item No. 36446.

        - 3.5 mm knotless HTPP, 1-3/4 inch (44 mm) sq. mesh, with bottom lace cord. UV and weather resistant. 325 lbs. (147.42 kg) break strength.

      - Douglas Replacement Slip-On Style Net: 7x7 ft. Item No. 36449.

        - 3.5 mm knotless HTPP, 1-3/4 inch (44 mm) sq. mesh with bottom lace cord, UV, and weather resistant. 325 lbs.(147.42 kg) break strength.

      - Douglas Replacement Slip-On Style Net: 7x7 ft. L-Screen. Item No. 36450.

        - 3.5 mm knotless HTPP, 1-3/4 inch (44 mm) sq. mesh with bottom lace cord, UV, and weather resistant. 325 lbs. (147.42 kg) break strength.

      - Douglas Replacement Slip-On Style Net: 7x7ft. Sock Net. Item No. 36457.

        - 3.5 mm knotless HTPP, 1-3/4 inch (44 mm) sq. mesh with bottom lace cord, UV, and weather resistant. 325 lbs. (147.42 kg) break strength.

      - Douglas Replacement Slip-On Style Net: 7x7 ft. Softball Pitching Net. Item No. 36462.

        - 3.5 mm knotless HTPP, 1-3/4 inch (44 mm) sq. mesh with bottom lace cord, UV, and weather resistant. 325 lbs. (147.42 kg) break strength.

      - Douglas Replacement Slip-On Style Net: 8x8ft. L-Screen. Item No. 36451H.

        - 3.5 mm knotless HTPP, 1-3/4 inch (44 mm) sq. mesh with bottom lace cord, UV, and weather resistant. 325 lbs. (147.42 kg) break strength.

      - Varsity Wheel Kit: Item No. 36469. Airless tires.

        - For use with 1-1/2 inch (38 mm) OD framed Varsity Screens.

        - Slip on and slip off installation with tightening screw.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until the substrates have been properly constructed and prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect in writing of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

  - FIELD QUALITY CONTROL

    - Field Inspection: Coordinate field inspection in accordance with appropriate sections in Division 01.

** NOTE TO SPECIFIER ** Include if manufacturer provides field quality control with onsite personnel for instruction or supervision of product installation, application, erection, or construction. Delete if not required.

    - Manufacturer's Services: Coordinate manufacturer's services in accordance with appropriate sections in Division 01.

  - CLEANING AND PROTECTION

    - Clean products in accordance with the manufacturers recommendations.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION