SECTION 08 14 00

WOOD DOORS - ARCHITECTURAL, COMMERCIAL, FLUSH, STILE AND RAIL, ATTACK RESISTANT

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2026 - 2026 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Forte Opening Solutions; Commercial door systems and components.
This section is based on the products of Forte Opening Solutions, which is located at:
4301 Anchor Plaza Pkwy., Suite 220
Tampa, FL 33634
Toll Free Tel: 877-332-4484
Email: request info (helpline@masonite.com)
Web: https://www.forteopenings.com  
 [ Click Here ] for additional information.
Masonite Architectural is Now Forte Opening Solutions!
Forte isn't just a new name, it's a reflection of our commitment to delivering premium value and showcasing our deep industry expertise. We meticulously craft our doors to ensure they're as secure as they are aesthetically pleasing. With over a century in the industry, our expertise is unmatched and our customer focus is unparalleled.
Our doors are engineered to withstand the passage of time, mirroring the same resilience that has defined our company's tradition. It's no coincidence that our doors are weaved throughout American history. Let us help your company build its legacy.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Flush Wood Doors:

      - Attack resistant.

      - Stile and rail, transparent finish.

      - Stile and rail, opaque finish.

      - Commercial.

      - Architectural.

  - RELATED SECTIONS

    - These sections may have data relevant to this Work Result but not available in this section.

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

      - Section 06 00 00 - Wood, Plastics and Composites

      - Section 08 11 00 - Hollow Metal Doors and Frames

      - Section 08 30 00 - Specialty Doors and Frames.

      - Section 08 71 00 - Door Hardware.

      - Section 08 80 00 - Factory Glazing for glass view panels in flush wood doors.

      - Section 09 00 00 - Finishes.

      - Section 09 91 23 - Interior Painting for field finishing doors.

      - Section 09 93 00 - Staining and Transparent Finishing for field finishing doors.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00.

      - Product Data: For each type of door. Include details of core and edge construction, vision kits for openings, louvers, and factory finishing.

      - Shop Drawings: Indicate location, size, and hand of each door; elevation of each kind of door; construction details not covered in Product Data; and the following:

        - Plans and elevations.

        - Hardware preparation.

        - Wiring chase preparation.

        - Glazed openings.

        - Blocking dimensions and locations.

        - Fire-protection ratings for fire-rated doors.

        - Surface grain directions.

        - Assembly methods.

        - Joint details.

        - Fastening methods.

        - Accessories.

        - Fire Rating: Hardware compliance.

          - Neutral Pressure - UL 10B. Positive Pressure - UL 10C.

      - Samples for Initial Selection: Actual materials; printed brochures are not acceptable.

        - Stain colors and gloss.

        - Paint colors and gloss.

        - Plastic Laminate: HPDL color and pattern. LPDL color and patterns.

        - Casing profiles for wood door frames.

      - Samples for Verification:

        - Factory finish applied to door face, 8x10 inches, for each material and finish.

        - Door Corner Sections: 8x10 inches. Faces and edges using actual materials.

        - Plastic Laminate: 5 inch square, for each color, texture and pattern selected.

        - Light Opening Frames: 6 inches long, for each material, type, and finish.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum of five years documented experience.

    - Installer Qualifications: Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

    - Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Construct a mock-up with actual materials in sufficient time for Architect's review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

      - Mock-up Intent: Demonstrate quality of workmanship and visual appearance.

      - If Mock-up is Not Acceptable: Rebuild mock-up until achieving satisfactory results.

      - Retain Mock-up: As a standard for comparison with completed work.

      - Do not alter or remove mock-up until work is completed or removal is authorized.

  - PRE-INSTALLATION CONFERENCE

    - Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees: Include Architect, Contractor, and trades involved. Agenda is to include schedule, responsibilities, critical path items, and approvals.

  - DELIVERY, STORAGE, AND HANDLING

    - Comply with requirements of referenced standard and manufacturer's written instructions.

      - Doors: Package individually in plastic. Wrap bundles of doors in plastic sheeting.

      - Door Marking: Each door on top rail with opening number used on Shop Drawings.

  - PROJECT CONDITIONS

    - Environmental Limitations: Deliver doors after spaces are enclosed and weather tight, wet work is complete and dry, HVAC system is operating and maintaining ambient temperature and humidity conditions at occupancy levels during remaining construction period.

  - WARRANTY

    - Warranty: Manufacturer agrees to repair or replace doors that fail in materials or workmanship within specified warranty period. Failures are not limited to the following:

      - Warping (bow, cup, or twist) more than 1/4 inch in a 42x84 inch section.

      - Telegraphing of core construction in face veneers exceeding 0.01 inch in a 3 inch span.

      - Warranty includes installation and finishing required due to repair or replacement of defective doors.

      - Warranty Period for Solid-Core Interior Doors: Life of installation.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Forte Opening Solutions, which is located at:
4301 Anchor Plaza Pkwy., Suite 220
Tampa, FL 33634
Toll Free Tel: 877-332-4484
Email: request info (helpline@masonite.com);Web: https://www.forteopenings.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs: coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for Substitutions: Considered in accordance with provisions of Section 01 60 00.

  - PERFORMANCE REQUIRMENTS

    - General: To be met unless specified otherwise.

      - Quality Standard: WDMA I.S.1-A, latest edition, "Industry Standard for Architectural Wood Flush Doors".

      - Security Standard: Filti Testing and Development (FTD), "Shooter Attack Test Method (SA)".

      - Match appearance grade and applicable construction and performance requirements of flush wood doors.

      - Performance Grade: Extra Heavy Duty.

      - Aesthetic Grade: Custom (A grade faces).

      - Aesthetic Grade: Premium (AA grade faces).

      - Security Standard: FTD-SA. Performance Class: 3.

      - Fire-Rated Wood Doors: Conform to NFPA 80. Listed and labeled for required ratings based on testing at pressure.

        - NFPA 252, UL 10C, or agency acceptable to authorities having jurisdiction.

          - Ratings: Category A positive pressure.

            - Vertical Edges: Integral intumescent seals concealed by outer stile where required.

          - Ratings: Category B positive pressure.

            - Vertical Edges: Intumescent seals applied to door frame per requirements of section 08 71 00 where required.

          - Cores: 20 minute fire rated: Extra heavy-duty particle board.

          - Cores: 20 minute fire rated: Structural composite lumber.

          - Cores: 45 minute fire rated: Fire-resistant wood-based particleboard.

          - Blocking: With improved screw holding capability approved for use in doors of fire protection ratings indicated as follows:

            - Not required in extra heavy duty particle board or SCL core doors.

        - Smoke and Draft Control Door Assemblies: Listed and labeled for smoke and draft control, based on testing according to UL 1784 for I occupancy projects.

      - Match appearance grade and applicable construction and performance requirements of flush wood doors.

  - FLUSH WOOD DOORS - ATTACK RESISTANT

    - Basis of Design: Aspiro Series Flush Wood Doors by Forte Opening Solutions.

      - A component of the Masonite Defendr Door System.

** NOTE TO SPECIFIER ** Delete paragraphs that are not required. Then delete options not required.

    - Veneer-Faced Doors for Transparent Finish: Interior Solid-Core Doors:

      - Aesthetic Grade: Custom, with Grade A faces. Wood Species: ________.

      - Aesthetic Grade: Premium, with Grade AA faces. Wood Species: ________.

      - Cut: Rotary cut

      - Cut: Plain sliced; flat sliced.

      - Cut: Quarter sliced.

      - Cut: Rift cut.

      - Match between Veneer Leaves: Book match.

      - Match between Veneer Leaves: Slip match.

      - Assembly of Veneer Leaves on Door Faces: Book and Running match.

      - Assembly of Veneer Leaves on Door Faces: Balanced match.

      - Assembly of Veneer Leaves on Door Faces: Center balanced match.

      - Pair and Set Match: Doors hung in same opening or separated by mullions.

      - Vertical Edges: Matching / Compatible Hardwood Lumber over SCL.

      - Vertical Edges: Matching / Compatible Veneer edge band over SCL.

      - Vertical Edges: Compatible high impact edge bonded to SCL.

      - Horizontal Edges: SCL. Clean edge band bonded to the SCL top rail.

      - Core: Extra Heavy-Duty Particle Board.

      - Core: Structural Composite Lumber.

      - Construction: Five plies. Stiles and rails are bonded to core, then entire unit is abrasive planed before veneering.

    - Plastic Laminate Faced Interior Solid-Core Doors: Aesthetic Grade: Premium.

      - Plastic Laminate Faces: HPDL. Comply with NEMA LD 3, Grade HGS.

      - Colors, Patterns, and Finishes: As indicated.

      - Colors, Patterns, and Finishes: Selected by Architect from manufacturer's products.

      - Vertical Edges: Matching HPDL edge band over structural composite lumber.

      - Vertical Edges: Matching high impact edge bonded to structural composite lumber.

      - Vertical Edges: Hardwood for staining or painting.

      - Vertical Edges: Compatible high impact edge bonded to structural composite lumber.

      - Horizontal Edges: SCL. Clean edge band bonded to the SCL top rail.

      - Core: Extra Heavy-Duty Particle Board.

      - Core: Structural Composite Lumber.

      - Construction: Five plies. Stiles and rails are bonded to core. Entire unit is abrasive planed before crossbands and laminate faces are applied.

    - MDO Faced Doors for Opaque Finish Paint: Interior Solid Core Doors.

      - Aesthetic Grade: Premium.

      - Faces: MDO, apply to high-density hardboard crossbands.

      - Vertical Edges: Matching / Compatible Hardwood Lumber over SCL.

      - Vertical Edges: Matching / Compatible Veneer edge band over SCL.

      - Vertical Edges: Compatible high impact edge bonded to SCL.

      - Horizontal Edges: SCL. Clean edge band bonded to the SCL top rail.

      - Core: Extra Heavy-Duty Particle Board.

      - Core: Structural Composite Lumber.

      - Construction: Five plies. Stiles and rails are bonded to core. Entire unit is abrasive planed before crossbands and MDO faces are applied.

    - Light Frames: Metal Vision Frames for Light Openings.

      - Material: Cold Rolled Steel: 18 gauge.

      - Factory Finish: Approved for use in doors for fire-protection rating indicated.

        - Finish: Factory baked-enamel. Color: As indicated.

        - Finish: Factory powder-coated. Color: As indicated.

        - Finish: Factory veneer wrapped, to match door faces.

    - Light Frames: MVF Manufacturer's: Subject to compliance, meeting Filti Testing and Development (FTD), "Shooter Attack Test Method (SA)".

      - All Metal Stamping: 110; non-rated.

      - All Metal Stamping: 110-D2; fire-rated.

    - Fabrication:

      - Factory Fit Doors: To suit frame-opening sizes indicated. Comply with clearance requirements of referenced quality standard for fitting unless otherwise indicated.

        - Comply with NFPA 80 requirements for fire-rated doors.

      - Factory Machine Doors: For hardware not surface applied. Locate hardware to comply with DHI-WDHS-3. Comply with final hardware schedules, door frame Shop Drawings, BHMA- 156.115-W, and hardware templates.

        - Coordinate with hardware mortises in metal frames to verify dimensions and alignment before factory machining.

      - Openings:

        - Light Openings: Factory cut and install with vision kits indicated.

        - Glass: Factory install glass in doors per manufacturer's instructions.

          - Glazing: Non-rated glass to be Armoured One AOTSG516.

          - Glazing: 20-minute glass to be Dual Glazed:

            - Armoured One AOTSG516.

            - Pyroswiss 20.

          - Glazing: 45 minute glass to be Dual Glazed: Armoured One AOTSG516.

            - 45 min: Keralite FR-F 3/16 inches or Firelite NT 3/16 inches.

            - 45 min: Keralite FR-L 5/16 inches or Firelite Plus 5/16 inches.

    - Factory Finishing: Comply with referenced quality standard for factory finishing.

      - Prior to Finishing: Complete fabrication, opening fitting, and hardware machining.

      - Transparent Finish: Grade: Premium. WDMA TR-8 UV Cured Acrylated Polyester/Urethane.

        - Staining: Match Architect's sample.

        - Staining: As selected by Architect from manufacturer's full range.

        - Sheen: Satin.

      - Factory Painted Finish: Grade: Premium. WDMA I.S. 1-A System OP-8.

        - Color: Matching Architect's paint sample.

        - Color: As selected by Architect from manufacturers full range.

        - Sheen: Satin.

  - STILE AND RAIL DOORS - TRANSPARENT FINISH

    - Basis of Design: Aspiro Series Authentic Stile Rail Wood Doors by Forte Opening Solutions.

** NOTE TO SPECIFIER ** Delete door type option not required.

      - Door Type: Interior. Aspiro Series. Model No.: ________.

** NOTE TO SPECIFIER ** Delete door thickness options not required.

        - Door Thickness: As indicated on the Drawings.

        - Door Thickness: 1-3/8 inches

        - Door Thickness: 1-3/4 inches.

        - Door Thickness: 2-1/4 inches.

        - Door Thickness, Fire-Rated: 1-3/4 inches.

        - Door Thickness, Fire-Rated: 2-1/4 inches.

        - Construction Type: Cope and Stick. Joinery: 1/2 x 4 inch dowels.

          - Joints to have minimum 1 dowel.

        - Construction Materials:

          - Stiles and Rails: SCL. Solid Hardwood Edge: 3/4 inch minimum.

          - Muntin Bars: 1/4 x 2-1/2 inch dowels at every joint.

          - Pressure Fit Joints: Use a Type II adhesive.

        - Veneers: 1/16 inch thick. Grade A. Wood Species: ________. Cut: ________.

          - Matching; Lay-up.

** NOTE TO SPECIFIER ** Delete top design, and panel types options not required.

        - Door Panel Top Design: Square top.

        - Door Panel Top Design: Arch.

        - Door Panel Top Design: Eyebrow.

        - Door Panel Top Design: Provence.

        - Door Panel Top Design: Custom.

        - Panel Types: Rim Banded Raised Panel: 1-1/8 inch thick.

        - Panel Types: Veneer Wrapped Raised Panel: 1-1/8 inch thick.

        - Panel Types: Flat Panel: 5/8 inches thick.

        - Panel Types: V-Groove Flat Panel, 5/8 inch thick.

        - Panel Types: V-Groove Flat Panel, 1-1/8 inch thick.

        - Panel Types: V-Groove Raised Panel: 1-1/8 inch thick.

        - Panel Types: V-Groove Raised Panel: 1-5/8 inch thick.

        - Panel Construction: Manufactured with face veneer directly to core material.

          - Raised Panels Thickness: 1-1/8 inch minimum of solid lumber material matching face species, rim banded, with mitered corners.

          - Flat Panels: 5/8 inch thick minimum.

        - Material for Rim Banded Raised Panel: Particle core.

        - Material for Veneer Wrapped Raised Panel: MDF.

        - Material for Flat Panel: Particle core.

** NOTE TO SPECIFIER ** Delete sticking types and louver types options not required.

        - Sticking Types: Ogee.

        - Sticking Types: Ovolo.

        - Sticking Types: Square.

        - Sticking Types: Quarter Round.

        - Sticking Types: No profile.

        - Sticking Types: Custom.

        - Louver Types: Standard.

        - Louver Types: False.

        - Louver Types: Chevron.

        - Louver Construction: 3/8 x 2-1/4 inch solid lumber.

          - Louver Slats: Mortised into stiles and assembled individually.

        - Fire-Rating Interior Doors: 20 minutes.

        - Fire-Rating Interior Doors: Fire Rating: 45 minutes.

        - Fire-Rating Interior Doors: Fire Rating: 60 minutes.

        - Fire-Rating Interior Doors: Fire Rating: 90 minutes.

        - Labeled Doors: Confirm to the following applicable local code.

** NOTE TO SPECIFIER ** Delete pressure option not required. Delete construction option not required.

          - Neutral Pressure Doors: Tested to UL 10B

          - Positive Pressure Doors: Tested to UL 10C (if selecting Positive Pressure, choose one of the below.)

            - Category A Construction: Doors without an edge-sealing system. Frame and doors with a sealing system incorporated, concealed or visible, into the door edge.

            - Category B Construction: Doors evaluated with an edge-sealing system field applied to labeled frame or door.

      - Door Type: Exterior. Aspiro Series. Model No. ________.

** NOTE TO SPECIFIER ** Delete thickness option not required.

        - Door Thickness: 1-3/4 inches.

        - Door Thickness: 2-1/4 inches.

        - Construction Type: Cope and Stick Joinery. Use 1/2 x 4 inch dowels.

          - Joints to have minimum 1 dowel.

        - Construction Materials:

          - Stiles: SCL, laminated with 1/8 inch thick veneer, concealing SCL.

          - Rails: Solid lumber construction.

          - Muntin Bars: 1/4 x 2-1/2 inch. Dowels at every joint.

          - Pressure Fit Joints: Use Type I Adhesive and silicone caulk to prevent moisture penetration.

          - Flat Panels: Exterior grade core. Overall Thickness: 5/8 inch.

          - Raised Panels: Exterior grade core. Overall Thickness: 1-5/8 inch.

        - Veneer: 1/8 inch thick. Grade: A

          - Species: ________.

          - Cut: ________

** NOTE TO SPECIFIER ** Delete top design and panel types options not required.

        - Door Panel Top Design: Square Top.

        - Door Panel Top Design: Arch.

        - Door Panel Top Design: Eyebrow.

        - Door Panel Top Design: Provence.

        - Door Panel Top Design: Custom.

        - Panel Types: Rim Banded Raised Panel: 1-5/8 inch.

        - Panel Types: Flat Panel: 5/8 inch.

        - Panel Types: V-Groove Flat Panel 1-5/8 inch.

        - Panel Types: V-Groove Raised Panel 1-5/8 inch.

        - Panel Construction: 5-ply with 1/32 inch crossbands, in addition to face veneer.

** NOTE TO SPECIFIER ** Delete panel material and sticking types options not required.

        - Panel Material: Rim Banded Raised Panel: Exterior MDF.

        - Panel Material: Flat Panel: Exterior MDF.

        - Sticking Types: Ogee.

        - Sticking Types: Ovolo.

        - Sticking Types: Square.

        - Sticking Types: Quarter round.

        - Sticking Types: No profile.

        - Sticking Types: Custom.

    - Fabrication:

** NOTE TO SPECIFIER ** Delete construction grade option not required.

      - In Accordance With Construction Grade: WDMA I.S.6A-Premium Grade.

      - In Accordance With Construction Grade: WDMA I.S.6A-Custom Grade.

      - Veneer: Grade A unless otherwise specified.

      - Prefit and Bevel Doors: 1/8 inch in 2 inches, ensuring proper gaps are maintained on fire doors to comply with NFPA 80 requirements.

      - Machined for specified hardware that is not surface applied.

      - Factory glazed with glass as specified unless otherwise indicated.

        - Glass Type: ________. Glass Thickness: ________.

    - Factory Finish: Manufacturers' Conversion Varnish and sheen unless otherwise indicated.

      - Finish Coating: Clear.

      - Finish Coating: Espresso.

      - Finish Coating: Nutmeg.

      - Finish Coating: Honey.

      - Finish Coating: Stout.

      - Finish Coating: Caramel.

      - Finish Coating: Cinnamon.

      - Finish Coating: Bourbon.

      - Finish Coating: Toast.

      - Finish Coating: Cane.

      - Finish Coating: Saffron.

      - Finish Coating: Cocoa Bean.

      - Finish Coating: Rolled Oats.

      - Finish Coating: Match Architect's sample.

  - STILE AND RAIL DOORS - OPAQUE FINISH

    - Basis of Design Product: Aspiro Series, Marshfield-Algoma by Forte Opening Solutions.

      - Performance and Design Requirements:

        - ANSI/WDMA I.S. 6A.

        - WDMA Quality Grade: Custom or Premium.

        - WDMA Performance Level: Heavy Duty.

        - Match appearance grade and applicable construction and performance requirements of flush wood doors.

      - Paint Grade Stile and Rail Doors:

        - Stiles and Rails:

          - Core: Structural composite lumber.

          - Faces: High Density Fiberboard (HDF) or paint grade hardwood veneer.

          - Faces: Paint grade hardwood veneer.

        - Panel Types; as Scheduled:

          - Medium Density Fiberboard (MDF).

          - Glazed panels.

          - Combination MDF and glass panels.

          - Wood louvers.

        - Wood Panels:

          - Profile: Raised.

          - Profile: Flat.

          - Profile: As indicated.

          - Panel Moldings: Paint-grade hardwood

        - Glazing: Fully tempered clear glass unless otherwise indicated.

          - Glazing Beads and Mullions: Paint-grade hardwood.

        - Louvers: Solid wood, paint-grade.

          - Configuration: Slat.

          - Configuration: Chevron.

          - Configuration: As scheduled.

      - Fire Rated Stile and Rail Doors:

        - Ratings: 20 minutes Category A positive pressure.

        - Ratings: 20 minutes Category B positive pressure.

        - Construction: SCL core. Listed and labeled per NFPA 80.

        - Safety Glazing: Fire-rated and approved for rating.

          - Glazing Frames: Paint-grade wood.

          - Glazing Frames: Listed metal.

      - Factory Finish:

        - Primer: Manufacturer's standard primer compatible with field-applied paint.

        - Painted: WDMA OP-4 finish or equivalent.

          - Color: As selected by Architect

          - Color: Matched sample provided by Architect.

          - Sheen: Satin.

  - COMMERCIAL WOOD DOORS

    - Basis of Design: Cendura Series Products, Mohawk as manufactured by Forte Opening Solutions.

    - Performance and Design Requirements:

      - Interior Wood Doors: Window and Door Manufacturers Association publication ANSI/WDMA I.S. 1A "Industry Standard for Interior Architectural Wood Flush Doors".

      - Fire-Rated Wood Doors: Conform to NFPA 80. Listed and labeled for required ratings based on testing at pressure.

        - NFPA 252, UL 10C, or agency acceptable to authorities having jurisdiction.

          - Ratings: Category A positive pressure.

            - Vertical Edges: Integral intumescent seals concealed by outer stile where required.

          - Ratings: Category B positive pressure.

            - Vertical Edges: Intumescent seals applied to door frame per requirements of section 08 71 00 where required.

** NOTE TO SPECIFIER ** Delete temperature-Rise limit option not required or both if not required. IBC allows an exception for buildings with fire-suppression sprinklers throughout.

        - Temperature-Rise Limit: Where indicated.

        - Temperature-Rise Limit: At vertical exit enclosures and exit passageways.

        - Maximum Transmitted Temperature End Point: Not more than 250 degrees F above ambient after 30 minutes of standard fire-test exposure.

        - Composite Blocking: Approved for use in doors of fire-protection ratings indicated as needed to maintain WDMA performance level and eliminate through-bolting hardware.

** NOTE TO SPECIFIER ** Retain pairs option not required.

        - Pairs: Fire-retardant stiles with concealed intumescent seals listed and labeled for applications indicated without formed-steel edges and astragals.

        - Pairs: Formed-steel edges and astragals with intumescent seals as required for ratings.

          - Steel edges and astragals factory primed for field painting.

          - Veneer wrapped steel edges in same species and finish as door faces.

          - Steel edges and astragals with baked enamel in color selected by Architect from manufacturer's standard offering.

          - Stainless steel edges and astragals, No. 4 satin polish.

** NOTE TO SPECIFIER ** Acoustical ratings up to STC 38 are available, depending on construction. Delete option not required or both if not required..

      - Acoustical Rating for Solid Core Doors: As indicated in door schedule on Drawings.

      - Acoustical Rating for Solid Core Doors: STC: _______. Doors supplied with seals and gaskets tested by manufacturer.

    - Sustainability Requirements:

** NOTE TO SPECIFIER ** Some states require specifications for publicly-funded projects to allow certification by agencies other than FSC.

      - HYPERLINK "http://www.arcomnet.com/sustainable_design.aspx?topic=155"Certified Wood: "FSC Pure" or "FSC Mixed Credit" per FSC STD-01-001 and FSC STD 40-004.

      - HYPERLINK "http://www.arcomnet.com/sustainable_design.aspx?topic=64"Adhesives: Meet California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

      - HYPERLINK "http://www.arcomnet.com/sustainable_design.aspx?topic=63"Composite Wood Products: Manufactured with ultra-low-emitting formaldehyde resins as defined in California Air Resources Board's "Airborne Toxic Control Measure to Reduce Formaldehyde Emissions from Composite Wood Products".

** NOTE TO SPECIFIER ** Delete product options not required.

    - Interior Solid Core Standard Veneer Flush Wood Doors for Transparent Finish

      - WDMA Quality Standard: Custom.

** NOTE TO SPECIFIER ** Delete Performance level option not required.

      - WDMA Performance Level: Heavy Duty.

      - WDMA Performance Level: Extra Heavy Duty. Surface Mounted Hardware: Attached with through-bolts.

      - Veneer Grade: A.

** NOTE TO SPECIFIER ** Delete, cut, and leaf match options not required.

      - Veneer Species: ________.

      - Veneer Cut: Rotary.

      - Veneer Cut: Plain sliced.

      - Veneer Cut: Rift cut.

      - Veneer Leaf Match: Book match.

      - Veneer Leaf Match: Slip match for rift cut oak veneers.

      - Veneer Face Match: Running match.

      - Pair Match: For doors hung in same opening or separated by mullions.

      - Horizontal Edges: Structural composite lumber.

** NOTE TO SPECIFIER ** Delete vertical edges, core, and thickness options not required.

      - Vertical Edges: Mill option softwood.

      - Vertical Edges: Low-density hardwood.

      - Core: Wood-based particleboard.

      - Core: Extra heavy duty wood-based particleboard.

      - Core: Structural composite lumber.

      - Thickness: 1-3/8 inch with heavy duty particleboard core.

      - Thickness: 1-3/4 inch.

      - Construction: Five plies. Stiles and rails are bonded to core, and then entire unit is abrasive planed before veneering.

    - Solid Core Standard Veneer Flush Wood Doors with Glazed Lites:

      - Factory Glazing: Refer to Section 08 80 00 - Glazing, for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

        - Glazing: Factory-installed fire-rated safety glass for door rating.

** NOTE TO SPECIFIER ** Delete factory install glass option not required.

        - Factory Install Glass: As required.

        - Factory Install Glass: In fire rated doors only.

      - Wood Glazing Accessories: Solid wood of same species as face veneer in standard profile selected by Architect.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profile: Straight Lip.

        - Bead Profile: Beveled Lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with Reveal.

        - Bead Profile: Straight Lip-insulated glass.

        - Applied Grille: MD Kit Lip Bead.

        - Applied Grille: MD Kit Beveled Bead

        - Applied Grille: Classic Grille Straight Bead.

        - Applied Grille: Classic Grille Beveled Bead.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete frame design and finish options not required.

        - Frame Design: 110. 1-1/4 inch flanges.

        - Frame Design: 110 Veneer Wrapped. 1-1/4 inch flanges.

        - Frame Design: 115-L1. For 2 glass lites with thermal spacer.

        - Frame Design: Metal Vision Frame 115-L2. 1-1/2 inch flanges.

        - Frame Design: Metal Vision Frame 115-L3. 1-1/2 inch flanges.

        - Finish: Factory primed for paint. Color: as indicated.

        - Finish: Baked enamel. Color: as indicated.

        - Finish: Powder-coated. Color: as indicated.

    - Solid Core Standard Veneer Flush Wood Doors with Louvers:

** NOTE TO SPECIFIER ** Delete louver and finish options not required.

      - Wood Louvers: Sight-proof.

      - Wood Louvers: Round slat.

      - Wood Louvers: Flat slat.

      - Metal Louvers: 600A1. Type: Chevron. Smaller blades with closer spacing.

      - Metal Louvers: 800A1. Type: Chevron.

      - Metal Louvers Finish: Factory primed for field painting.

      - Metal Louvers Finish: Painted. Color: As selected by Architect.

      - Metal Louvers Finish: Painted. Color: Match Architect's sample.

    - Fire-Rated Standard Wood Veneer Solid Core Flush Doors:

** NOTE TO SPECIFIER ** Delete rating, core, vertical and horizontal edges options not required.

      - Fire Rating: Category A positive pressure.

      - Fire Rating: Category B positive pressure.

      - Fire Rating: Category A neutral pressure.

      - Fire Rating: Category B neutral pressure.

      - Core: 20 Minute Doors. Wood-based particleboard core.

      - Core: 20 Minute Doors. Extra-heavy duty Wood-based particleboard core.

      - Core: 20 Minute Doors. Structural composite lumber core.

      - Core: 45 Minute Doors. Fire resistant particleboard core.

      - Core: 45 Minute Doors. Mineral core.

      - Core: 60 Minute Doors. Mineral core.

      - Core: 90 Minute Doors. Mineral core.

      - Vertical Edges: 20 Minute Doors: Mill option softwood or low-density hardwood.

      - Vertical Edges: 45 Minute Fire Resistant Particleboard Core Doors: Veneer edge-band over fire stile.

      - Vertical Edges: 45 Minute Mineral Core Doors: Hardwood composite fire stile.

      - Vertical Edges: 60 Minute Mineral Core Doors: Hardwood composite fire stile.

      - Vertical Edges: 90 Minute Mineral Core Doors: Hardwood composite fire stile.

      - Horizontal Edges: 20 Minute Doors: Structural composite lumber.

      - Horizontal Edges: 45 Minute. Manufacture's construction per label service listing.

      - Horizontal Edges: 60 Minute. Manufacture's construction per label service listing.

      - Horizontal Edges: 90 Minute. Manufacture's construction per label service listing.

    - Fire-Rated Standard Wood Veneer Solid Core Flush Wood Doors with Glazed Lites:

      - Factory Glazing: Refer to Section 08 80 00 - Glazing, for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

        - Glazing: Factory-installed fire-rated safety glass for door rating.

** NOTE TO SPECIFIER ** Delete factory install glass option not required.

        - Factory Install Glass: As required.

        - Factory Install Glass: In fire rated doors only.

      - Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profiles: Straight Lip.

        - Bead Profiles: Beveled Lip.

        - Bead Profiles: Straight.

        - Bead Profiles: Beveled.

        - Bead Profiles: Straight with Reveal.

        - Bead Profiles: Straight Lip-insulated glass.

        - Applied Grille: MD Kit Lip Bead.

        - Applied Grille: MD Kit Beveled Bead.

        - Applied Grille: Classic Grille Straight Bead.

        - Applied Grille: Classic Grille Beveled Bead.

      - Wood-Core Doors with 20 Minute Fire-Protection Ratings:

        - Provide wood beads and metal glazing clips as required and approved.

      - Veneer Wrap Bead for Light Openings in Fire-Rated Doors: Match veneer species of door face and approved for use in doors of fire-protection rating indicated.

        - Concealed Metal Glazing Clips: Include where required for opening size and fire-protection rating indicated.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete frame and finish options not required.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For two lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

        - Finish: Factory primed for paint approved for fire-protection rating indicated.

        - Finish: Baked enamel approved for fire-protection rating indicated.

        - Finish: Powder-coated approved for fire-protection rating indicated.

    - Fire-Rated Standard Wood Veneer Solid Core Flush Doors with Louvers:

      - Metal Louvers with Fusible Link and Closing Device. Listed and labeled for fire-protection door rating of 1-1/2 hours and less.

      - Material: Cold rolled steel. 18 gauge

** NOTE TO SPECIFIER ** Delete finish options not required.

      - Factory Finish: Primed for paint.

      - Factory Finish: Color baked-enamel.

      - Factory Finish: Powder-coated.

    - Interior Hollow Core Standard Veneer and Standard Painted Flush Wood Doors:

** NOTE TO SPECIFIER ** Hollow core doors are not available with lites or louvers.

      - Hollow Core Flush Wood Veneer and Tempered Hardboard Doors:

        - WDMA Quality Standard: Custom.

        - Faces: Wood Veneer. Grade: A.

** NOTE TO SPECIFIER ** Delete vcut, and leaf match options not required.

          - Veneer Species: ________.

          - Veneer Cut: Rotary.

          - Veneer Cut: Plain sliced.

          - Veneer Cut: Rift cut.

          - Veneer Leaf Match: Book match.

          - Veneer Leaf Match: Slip match.

          - Veneer Face Match: Running match.

        - Faces: Standard Painted. Tempered hardboard.

        - Vertical Edges, Stained: Full length softwood.

        - Vertical Edges, Painted: Finger joint softwood.

        - Horizontal Edges: Softwood.

        - Core: Kraft paper honeycomb with lumber lock blocks.

        - Construction: Hollow core with High Density Fiberboard (HDF) crossbands.

** NOTE TO SPECIFIER ** Delete thickness options not required.

        - Thickness: 1-3/8 inch.

        - Thickness: 1-3/4 inch.

    - Interior Solid Core Standard Painted Flush Wood Doors:

      - WDMA Quality Standard: Custom.

** NOTE TO SPECIFIER ** Construction meets Extra-Heavy Duty face screw pull values when surface mounted hardware is attached with through-bolts. Delete performance level, faces, edges, core, and thickness options not required.

      - WDMA Performance Level: Heavy duty.

      - WDMA Performance Level: Extra heavy duty.

      - Faces: Tempered hardboard.

      - Faces: Paint grade birch.

      - Vertical Edges: Finger-Jointed softwood/mill.

      - Vertical Edges: Finger-Jointed low density hardwood.

      - Horizontal Edges: SCL.

      - Core: Wood based particleboard.

      - Core: Extra heavy duty wood-based particleboard.

      - Core: Structural composite lumber.

      - Thickness: 1-3/8 inch.

      - Thickness: 1-3/4 inch.

      - Construction: Five plies. Stiles and rails are bonded to core, and then entire unit is abrasive planed before veneering.

    - Solid Core Standard Painted Flush Doors with Glazed Lites:

      - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

        - Glazing: Safety glass for door rating.

** NOTE TO SPECIFIER ** Delete install option not required.

        - Factory Install Glass: As required.

        - Factory Install Glass: In fire rated doors only.

      - Wood Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profile: Straight lip.

        - Bead Profile: Beveled lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with reveal.

        - Bead Profile: Straight lip-insulated glass.

        - Applied Grille: MD Kit lip bead.

        - Applied Grille: MD Kit beveled bead.

        - Applied Grille: Classic grille straight bead.

        - Applied Grille: Classic grille beveled bead.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete finish and frame design options not required.

        - Factory Finish: Factory primed for painting. Color: As indicated.

        - Factory Finish: Baked enamel. Color: As indicated.

        - Factory Finish: Powder-coated. Color: As indicated.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For two lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

    - Solid Core Standard Painted Flush Doors with Louvers:

** NOTE TO SPECIFIER ** Delete lovers and finish options not required.

      - Wood Louvers: Sight-proof.

      - Wood Louvers: Round slat.

      - Wood Louvers: Flat slat.

      - Metal Louvers: 600A1. Type: Chevron. Smaller bladed with closer spacing.

      - Metal Louvers: 800A1. Type: Chevron.

      - Louver Finish: Factory primed for field painting.

      - Louver Finish: Paint. Color: As selected by Architect.

      - Louver Finish: Paint. Color: To match Architect's color sample.

    - Fire-Rated Standard Painted Solid-Core Flush Doors:

** NOTE TO SPECIFIER ** Delete rating, core, and edges options not required.

      - Rating: Category A positive pressure.

      - Rating: Category B positive pressure.

      - Rating: Neutral pressure.

      - Core, 20 Minute: Wood-based particleboard.

      - Core, 20 Minute: Extra-heavy duty Wood-based particleboard.

      - Core, 20 Minute: Structural composite lumber.

      - Core, 45 Minute: Fire resistant particleboard.

      - Core, 45 Minute: Mineral with blocking options.

      - Core, 60 Minute: Mineral with blocking options.

      - Core, 90 Minute: Mineral with blocking options.

      - Vertical Edges, 20 Minute: Mill option softwood or low-density hardwood.

      - Vertical Edges, 45 Minute Fire Resistant Particleboard Core: Veneer edge-band over fire stile.

      - Vertical Edges, 45 Minute Mineral Core: Hardwood composite fire stile.

      - Vertical Edges, 60 Minute Mineral Core: Hardwood composite fire stile.

      - Vertical Edges, 90 Minute Mineral Core: Hardwood composite fire stile.

      - Horizontal Edges, 20 Minutes: Structural composite lumber.

      - Horizontal Edges, 45 Minute: Manufacture's construction per label service listing.

      - Horizontal Edges, 60 Minute: Manufacture's construction per label service listing.

      - Horizontal Edges, 90 Minute: Manufacture's construction per label service listing.

    - Fire-Rated Standard Painted Solid Core Flush Doors with Glazed Lites:

    - Factory Glazing: Refer to Section 08 80 00 - Glazing, for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

        - Glazing: Factory-installed fire-rated safety glass for door rating.

** NOTE TO SPECIFIER ** Delete factory install glass option not required.

        - Factory Install Glass: As required.

        - Factory Install Glass: In fire rated doors only.

      - Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profile: Straight lip.

        - Bead Profile: Beveled lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with reveal.

        - Bead Profile: Straight lip-insulated glass.

        - Applied Grille: MD Kit lip bead.

        - Applied Grille: MD Kit beveled bead.

        - Applied Grille: Classic grille straight bead.

        - Applied Grille: Classic grille beveled bead.

      - Wood-Core Doors with 20 Minute Fire-Protection Ratings:

        - Provide wood beads and metal glazing clips as required and approved.

      - Veneer Wrap Bead for Light Openings in Fire-Rated Doors: Match veneer species of door face and approved for use in doors of fire-protection rating indicated.

        - Concealed Metal Glazing Clips: Include where required for opening size and fire-protection rating indicated.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete frame and finish options not required.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For two lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

        - Finish: Factory primed for paint approved for fire-protection rating indicated.

        - Finish: Baked enamel approved for fire-protection rating indicated.

        - Finish: Powder-coated approved for fire-protection rating indicated.

    - Fire-Rated Standard Painted Solid Core Flush Doors with Louvers:

      - Metal Louvers with Fusible Link and Closing Device. Listed and labeled for fire-protection door rating of 1-1/2 hours and less.

      - Material: Cold rolled steel. 18 gauge

** NOTE TO SPECIFIER ** Delete finish options not required.

      - Factory Finish: Primed for paint.

      - Factory Finish: Color baked-enamel.

      - Factory Finish: Powder-coated.

    - Interior Solid Core Everyday Laminate Faced Flush (hpdl) Doors:

      - WDMA Quality Standard: Standard construction.

** NOTE TO SPECIFIER ** Delete performance, color, edges, core, and thickness options not required.

      - WDMA Performance Level: Heavy duty.

      - WDMA Performance Level: Extra heavy duty.

      - Faces: High-pressure decorative laminates complying with NEMA LD 3,Grade HGS.

        - Vertical and post formable grade laminates are not acceptable.

        - Colors, Patterns: As selected by Architect from manufacturer's offering.

        - Colors, Patterns: Match Architect's sample.

      - Vertical Edges: HPDL Edge Band applied after faces over SCL.

      - Vertical Edges: Hardwood for paint finish.

      - Vertical Edges: Hardwood for stain finish.

      - Horizontal Edges: Structural Composite Lumber

      - Core: Wood based particleboard.

      - Core: Extra heavy duty wood-based particleboard.

      - Core: Structural composite lumber (SCLC) for full lite doors.

      - Thickness: 1-3/8 inch.

      - Thickness: 1-3/4 inch.

      - Construction: Five plies. Stiles and rails are bonded to core. Then entire unit is abrasive planed before veneering.

    - Solid Core Everyday Laminate Flush Doors with Glazed Lites:

      - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

        - Glazing: Safety glass for door rating.

** NOTE TO SPECIFIER ** Delete install option not required.

        - Factory Install Glass: As required.

        - Factory Install Glass: In fire rated doors only.

      - Wood Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profile: Straight lip.

        - Bead Profile: Beveled lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with reveal.

        - Bead Profile: Straight lip-insulated glass.

        - Applied Grille: MD Kit lip bead.

        - Applied Grille: MD Kit beveled bead.

        - Applied Grille: Classic grille straight bead.

        - Applied Grille: Classic grille beveled bead.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete finish and frame design options not required.

        - Factory Finish: Factory primed for paint. Color: As indicated.

        - Factory Finish: Baked enamel. Color: As indicated.

        - Factory Finish: Powder-coated. Color: As indicated.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For two lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

    - Solid Core Standard Veneer Flush Wood Doors with Louvers:

** NOTE TO SPECIFIER ** Delete lovers and finish options not required.

      - Wood Louvers: Sight-proof.

      - Wood Louvers: Round slat.

      - Wood Louvers: Flat slat.

      - Metal Louvers: 600A1. Type: Chevron. Smaller bladed with closer spacing.

      - Metal Louvers: 800A1. Type: Chevron.

      - Louver Finish: Factory primed for field painting.

      - Louver Finish: Paint. Color: As selected by Architect.

      - Louver Finish: Paint. Color: To match Architect's color sample.

    - Fire-Rated Everyday Laminate Solid-Core Flush Doors:

** NOTE TO SPECIFIER ** Delete rating, core, and edges options not required.

      - Rating: Category A positive pressure.

      - Rating: Category B positive pressure.

      - Rating: Neutral pressure.

      - Core, 20 Minute Doors: Wood-based particleboard core.

      - Core, 20 Minute Doors: Extra-heavy duty Wood-based particleboard core.

      - Core, 45 Minute Doors: Fire resistant particleboard core.

      - Core, 45 Minute Doors: Mineral core.

      - Core, 60 Minute Doors: Mineral core.

      - Core, 90 Minute Doors: Mineral core.

      - Vertical Edges, 20 Minute Doors: HPDL edge band applied after faces over SCL.

      - Vertical Edges, 20 Minute Doors: Hardwood for paint finish.

      - Vertical Edges, 20 Minute Doors: Hardwood for stain finish.

      - Vertical Edges, 45 Minute Fire Resistant Particleboard Core Doors: HPDL edge band applied after faces over HDF fire stile.

      - Vertical Edges, 45 Minute Mineral Core Doors: Hardwood composite fire stile.

      - Vertical Edges, 60 Minute Mineral Core Doors: Hardwood composite fire stile.

      - Vertical Edges, 490 Minute Mineral Core Doors: Hardwood composite fire stile.

      - Horizontal Edges, 20 Minute Doors: SCL.

      - Horizontal Edges, 45 Minute Fire-rated Doors: Manufactures standard construction.

      - Horizontal Edges, 60 Minute Fire-rated Doors: Manufactures standard construction.

      - Horizontal Edges, 90 Minute Fire-rated Doors: Manufactures standard construction.

    - Fire-Rated Everyday Laminate Solid-Core with Glazed Lites:

      - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

        - Glazing: Fire-rated safety glass for door rating.

** NOTE TO SPECIFIER ** Delete factory install option not required.

        - Factory Install Glass: As required.

        - Factory Install Glass: In fire rated doors only.

      - Wood Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profile: Straight lip.

        - Bead Profile: Beveled lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with reveal.

        - Bead Profile: Straight lip-insulated glass.

        - Applied Grille: MD Kit lip bead.

        - Applied Grille: MD Kit beveled bead.

        - Applied Grille: Classic grille straight bead.

        - Applied Grille: Classic grille beveled bead.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete Frame and finish options not required.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For two lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

        - Factory Finish: Factory primed for paint. Color: As indicated.

        - Factory Finish: Baked enamel. Color: As indicated.

        - Factory Finish: Powder-coated. Color: As indicated.

    - Fire-Rated Everyday Laminate Solid-Core with Louvers:

** NOTE TO SPECIFIER ** Delete lovers and finish options not required.

      - Wood Louvers: Sight-proof.

      - Wood Louvers: Round slat.

      - Wood Louvers: Flat slat.

      - Metal Louvers: 600A1. Type: Chevron. Smaller bladed with closer spacing.

      - Metal Louvers: 800A1. Type: Chevron.

      - Louver Finish: Factory primed for field painting.

      - Louver Finish: Paint. Color: As selected by Architect.

      - Louver Finish: Paint. Color: To match Architect's color sample.

** NOTE TO SPECIFIER ** LPDL doors are only available as non-rated.

    - Interior Solid Core Everyday Low Pressure Decorative Laminate Faced (Lpdl) Flush Doors:

      - Performance Requirements:

        - WDMA Quality Standard: Standard construction.

** NOTE TO SPECIFIER ** Construction meets Extra-Heavy Duty face screw pull values when surface mounted hardware is attached with through-bolts.

        - WDMA Performance Level: Heavy duty.

        - WDMA Performance Level: Extra heavy duty.

      - Flush Wood Doors:

        - Faces: Low Pressure Decorative Laminate complying with Laminating Materials Association (LMA), Product Standard and Typical Physical Properties of Decorative Overlays; latest edition.

          - Colors, Patterns: As selected by Architect from manufacturer's offerings.

          - Vertical Edges: 0.024 inch PVC edge band over SCL.

          - Horizontal Edges: SCL.

          - Construction: Five plies. Stiles and rails are bonded to core. Then entire unit is abrasive planed before faces and (HDF) crossbands are applied.

          - Thickness: 1-3/4 inch.

      - Flush Wood Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

** NOTE TO SPECIFIER ** Delete factory install option not required.

          - Factory Install Glass: As required.

          - Factory Install Glass: In fire rated doors only.

      - Wood Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profile: Straight lip.

        - Bead Profile: Beveled lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with reveal.

        - Bead Profile: Straight lip-insulated glass.

        - Applied Grille: MD Kit lip bead.

        - Applied Grille: MD Kit beveled bead.

        - Applied Grille: Classic grille straight bead.

        - Applied Grille: Classic grille beveled bead.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete frame and finish design options not required.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For two lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

        - Factory Finish: Factory primed for paint. Color: As indicated.

        - Factory Finish: Baked enamel. Color: As indicated.

        - Factory Finish: Powder-coated. Color: As indicated.

    - Interior Medium Density Fiberboard Simulated Stile and Rail Wood Doors:

** NOTE TO SPECIFIER ** Doors are solid MDF. WDMA Quality Standards and Performance Ratings are not applicable to this type of door. Delete door type options not required.

      - Door Type: MDF Simulated Stile and Rail Doors: Routed faces.

        - Vertical Edges: Medium density fiberboard.

        - Horizontal Edges: Medium density fiberboard.

        - Core: Medium density fiberboard.

        - Construction: Solid panel medium density fiberboard (MDF).

** NOTE TO SPECIFIER ** Delete thickness options not required.

        - Thickness: 1-3/8 inch.

        - Thickness: 1-3/4 inch.

      - Door Type: MDF Simulated Stile and Rail Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

          - Glazing: Factory-installed tempered safety glass.

** NOTE TO SPECIFIER ** Delete factory install glass, glazing beads, and finish options not required. S4S is for rectangular profile. True Divided Lite is for multiple lites.

            - Factory Install Glass: As required.

            - Factory Install Glass: In fire rated doors only.

            - Glazing Beads, MDF: Qtr Round profile.

            - Glazing Beads, MDF: S4S profile.

            - Glazing Beads, MDF: True Divided Lite profile and muntins.

            - Glazing Beads, MDF: Applied Grille.

        - Metal Frame: Standard metal vision frame.

          - Factory Finish: Primer.

          - Factory Finish: Paint. Color: As selected by Architect.

      - Door Type: MDF Simulated Stile and Rail Doors with Louvers:

** NOTE TO SPECIFIER ** Delete design options not required.

        - Design: False Louver. MDF panel molded on sides to simulate slat louvers.

        - Design: Half False Louver. molded on one side to simulate slat louvers.

        - Design: Vented Louver. True slat design louvers

        - Design: Chevron Louver. Sight-proof inverted-V louvers.

      - Door Type: Fire Rated MDF Simulated Stile and Rail Doors.

** NOTE TO SPECIFIER ** Delete rating and edges options not required.

        - Rating: Category A, positive pressure.

        - Rating: Neutral pressure.

        - 20 Minute Doors: MDF.

        - 45 Minute Doors: Multi-layer composite.

        - 60 Minute Doors: Multi-layer composite.

        - 90 Minute Doors: Multi-layer composite.

        - Vertical Edges: 20-Minute Doors: MDF.

        - Vertical Edges: 45 Minute Doors: Hardwood composite fire stile.

        - Vertical Edges: 60 Minute Doors: Hardwood composite fire stile.

        - Vertical Edges: 90 Minute Doors: Hardwood composite fire stile.

      - Door Type: Fire Rated MDF Simulated Stile and Rail Doors with Glazed Lites:

** NOTE TO SPECIFIER ** Delete rating and edges options not required.

        - Rating: Category A, positive pressure.

        - Rating: Neutral pressure.

        - 20 Minute Doors: MDF.

        - 45 Minute Doors: Multi-layer composite.

        - 60 Minute Doors: Multi-layer composite.

        - 90 Minute Doors: Multi-layer composite.

        - Vertical Edges, 20-Minute Doors: MDF.

        - Vertical Edges, 45 Minute Doors: Hardwood composite fire stile.

        - Vertical Edges, 60 Minute Doors: Hardwood composite fire stile.

        - Vertical Edges, 90 Minute Doors: Hardwood composite fire stile.

        - Horizontal Edges, 20-Minute Doors: MDF.

        - Horizontal Edges, 45 Minute Doors: Hardwood composite fire stile.

        - Horizontal Edges, 60 Minute Doors: Hardwood composite fire stile.

        - Horizontal Edges, 90 Minute Doors: Hardwood composite fire stile.

** NOTE TO SPECIFIER ** Delete glazed lites options not required.

        - Glazed Lites, Maple Vision Frame 7.

          - Glazing: 1/4 inch Pyroswiss glass.

          - 20 Minute Doors: Factory-installed rated safety glass.

        - Glazed Lites: Formed steel frame.

          - Glazing: 3/16 inch Keralite FR-F.

          - 45 Minute Doors: Factory-installed rated safety glass.

          - 60 Minute Doors: Factory-installed rated safety glass.

          - 90 Minute Doors: Factory-installed rated safety glass.

        - Factory Finish: Primer for field painting. Color: As indicated.

        - Factory Finish: Baked enamel. Color: As indicated.

        - Factory Finish: Powder coat. Color: As indicated.

    - Interior Molded Panel and West End Wood Doors:

** NOTE TO SPECIFIER ** Includes hollow core and bifold; 2-panel and 6-panel design. West End doors include, hollow core and bifold; Hamel design only. Vision panels or louvers are not available.

      - Molded Panel Doors and West End:

        - WDMA Quality Standard: Standard Construction.

        - Specifier Note: WDMA Performance Level is only available on doors with solid core.

        - WDMA Performance Level: Heavy Duty. Note: Construction meets Extra Heavy Duty face screw pull values when surface mounted hardware is attached with through-bolts Heavy Duty.

** NOTE TO SPECIFIER ** Delete core, style, panel design, molded panel, west end design, thickness, and finish options not required.

        - Core Construction: Hollow core Kraft paper honeycomb with 2 lock blocks.

        - Core Construction: Wood-based particleboard with bonded faces.

        - Core Construction: Wood-based particleboard with non-bonded faces.

        - Style: Standard style selected by Architect.

        - Style: See door elevation types on Drawings.

        - Style: ________.

        - Faces: Molded medium density fiberboard.

        - Molded Panel Design: 2 panel.

        - Molded Panel Design: 6-panel.

        - West End Design: Hamel.

        - West End Design: Berkley.

        - West End Design: Melrose.

        - Vertical Edges: Finger-jointed softwood.

        - Thickness: 1-3/8 inch.

        - Thickness: 1-3/4 inch.

        - Factory Finish: Primer for field painting.

        - Factory Finish: Paint. Color: As indicated.

      - Fire Rated Molded Panel and West End Doors.

** NOTE TO SPECIFIER ** Delete rating, and edge options not required.

        - Rating: Category A positive pressure.

        - Rating: Category B positive pressure.

        - Rating: Neutral pressure.

        - 20 minute, Doors: Wood-based particleboard.

        - 45 minute, Doors: Mineral core.

        - 60 minute, Doors: Mineral core.

        - Vertical Edges, 20 Minute Doors: Finger Jointed softwood.

        - Vertical Edges, 45 Minute Doors: Hardwood composite fire stile.

        - Vertical Edges, 60 Minute Doors: Hardwood composite fire stile.

        - Horizontal Edges, 20 Minute Doors: Structural composite lumber.

        - Horizontal Edges, 45 Minute Doors: Manufactures standard.

        - Horizontal Edges, 60 Minute Doors: Manufactures standard.

    - Transparent Finish Wood Door Frames: Solid lumber matching door veneer species.

** NOTE TO SPECIFIER ** Delete frame profile options not required.

      - Frame Profile: Standard double rabbet.

      - Frame Profile: Custom double rabbet.

      - Frame Profile: Standard single rabbet.

      - Frame Profile: Custom single rabbet.

      - Frame Profile: T-Stop Jamb. A double rabbet frame with applied stop.

      - Frame Profile: T-Stop Jamb.

** NOTE TO SPECIFIER ** Delete trim and trim stile options not required. Wood frames can be supplied without casing trim.

      - Trim: Solid lumber that matches species of door veneer.

      - Trim: Door face veneer applied to mill option lumber or medium density fiberboard.

      - Trim Stile: Colonial Casing.

      - Trim Stile: Contemporary Casing.

      - Trim Stile: Sleek Casing.

      - Trim Stile: Traditional Casing.

      - Finish: Match door finish.

** NOTE TO SPECIFIER ** Wood frames are available in 20, 45, 60, and 90 minute ratings.

      - Fire Rating: Match door rating.

    - Paint Finish Wood Door Frames: Paint grade hardwood.

** NOTE TO SPECIFIER ** Delete frame profile not required.

      - Frame Profile: Standard double rabbet.

      - Frame Profile: Custom double rabbet.

      - Frame Profile: Standard single rabbet.

      - Frame Profile: Custom single rabbet.

      - Frame Profile: T-Stop Jamb. A double rabbet frame with applied stop.

** NOTE TO SPECIFIER ** Wood frames can be supplied without casing trim.

      - Trim: Paint grade softwood or medium density fiberboard.

      - Trim Stile: Colonial Casing.

      - Trim Stile: Contemporary Casing.

      - Trim Stile: Sleek Casing.

      - Trim Stile: Traditional Casing.

      - Finish: Factory primed for field painting.

      - Finish: Factory painted. Color: As indicated.

** NOTE TO SPECIFIER ** Wood frames are available in 20, 45, 60, and 90 minute ratings.

      - Fire Rating: Match door rating.

    - Fabrication:

      - Door Pairs and Fixed Side Panels: Veneer Matching: Pair match.

      - Factory Fitting: Fit to frame openings with clearances specified in WDMA I.S. 1A.

        - Undercut: Maximum 3/8 inch above thresholds.

        - Fire-Rated Doors: Comply with NFPA 80.

      - Factory Machining of Doors: For hardware not surface applied.

        - Verify dimensions for hardware mortises in metal frames before machining.

      - Cut Openings: At factory. Glazing: Trim and install.

      - Cut Openings: At factory. Louvers: Install.

      - Doors Supplied with Wood Frames: Hardware: Factory installed.

      - Doors Supplied with Wood Frames: Hardware: Contractor field installed.

      - Door Frames: Supply frames with temporary spreader bars at base.

** NOTE TO SPECIFIER ** Delete either or both of the following paragraphs. Whichever is not required.

        - Provide matching sidelite frames with rectangular wood beads for glazing.

        - Provide matching transom frames with rectangular wood beads for glazing.

    - Factory Finish: Grade: Match grade of door.

** NOTE TO SPECIFIER ** Delete finish and color options not required.

      - Finish: Stain, Transparent: WDMA TR-8, UV-Cured Acrylated Polyester / Urethane.

        - Color: Standard color selected by Architect.

        - Color: Match Architect's sample.

        - Color: None.

        - Sheen: Satin.

      - Finish: Primed. Primer for finish paint specified in the interior painting section.

      - Finish: Painted. WDMA I.S. 1-A System OP-8, UV-Cured Acrylated Polyester / Urethane.

        - Color: As selected by Architect from manufacturer's standard line.

        - Color: Match Architects' paint sample.

        - Sheen: Satin.

  - ARCHITECTURAL WOOD DOORS

    - Basis of Design: Products of Aspiro Series, Marshfield-Algoma by Forte Opening Solutions

    - Performance and Design Requirements:

** NOTE TO SPECIFIER ** WDMA standards are recommended for performance testing and have equivalent appearance requirements to AWS. Do not use both standards. If flush or stile and rail doors must match adjacent wood paneling or the unusual requirements, use AWS standards with other architectural woodwork in Division 06.

      - Interior Stile and Rail Wood Doors: ANSI/WDMA I.S. 6A "Industry Standard for Interior Architectural Wood Stile and Rail Doors".

      - Fire-Rated Wood Doors:

** NOTE TO SPECIFIER ** Delete temperature rise limit if not required. The IBC allows an exception for buildings equipped throughout with fire-suppression sprinklers.

        - Temperature-Rise Limit: Doors to have a maximum transmitted temperature end point of not more than 250 degrees F above ambient after 30 minutes of standard fire-test exposure.

** NOTE TO SPECIFIER ** Delete location option not required.

          - Location: Where indicated.

          - Location: At vertical exit enclosures and exit passageways.

        - Composite Blocking: Approved for use in doors of fire-protection ratings indicated as needed to maintain WDMA performance level and eliminate through-bolting hardware.

** NOTE TO SPECIFIER ** Delete pairs option not required.

          - Pairs: Fire-retardant stiles. Concealed intumescent seals listed and labeled for applications indicated without formed-steel edges and astragals.

          - Pairs: Formed-steel edges and astragals with intumescent seals as required for ratings.

          - Steel Edges and Astragals: Factory primed for field painting.

          - Veneer Wrapped Steel Edges: Same species and finish as door faces.

          - Steel Edges and Astragals: Baked enamel. Color: As indicated.

          - Stainless Steel Edges and Astragals: No. 4 satin polish.

** NOTE TO SPECIFIER ** Following requirement applies only to ' I' occupancy projects.

      - Smoke and Draft Control Door Assemblies: Listed and labeled per UL 1784.

** NOTE TO SPECIFIER ** Doors are available with STC rating of 30 to 47, depending on construction.

      - Acoustical Rating for Solid Core Doors: Where indicated in door schedule, provide STC ___ doors supplied with seals and gaskets tested by manufacturer.

    - Sustainability Requirements:

** NOTE TO SPECIFIER ** Some states require specifications for publicly-funded projects to allow certification by agencies other than FSC.

      - HYPERLINK "http://www.arcomnet.com/sustainable_design.aspx?topic=155"Certified Wood: "FSC Pure"or "FSC Mixed Credit" per FSC STD-01-001 and FSC STD-40-004.

      - HYPERLINK "http://www.arcomnet.com/sustainable_design.aspx?topic=64"Adhesives: Meet test and product requirements of California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

      - HYPERLINK "http://www.arcomnet.com/sustainable_design.aspx?topic=63"Composite Wood: Ultra-low-emitting formaldehyde resins as defined in California Air Resources Board's "Airborne Toxic Control Measure to Reduce Formaldehyde Emissions from Composite Wood Products" or no added formaldehyde.

    - Interior Solid Core Flush Wood Doors for Transparent Finish:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Solid Core Select Wood Veneer Flush Doors:

** NOTE TO SPECIFIER ** Delete grade, performance level, faces, species, cut, and match options not required.

        - WDMA Quality grade: Custom.

        - WDMA Quality grade: Premium.

        - WDMA Performance Level: Extra Heavy Duty.

        - WDMA Performance Level: Heavy Duty.

        - Faces: Veneer Grade: AA.

        - Faces: Veneer Grade: A.

        - Veneer Species: __________.

        - Veneer Cut: Rotary cut.

        - Veneer Cut: Plain sliced. Flat cut.

        - Veneer Cut: Quarter sliced

        - Veneer Cut: Rift cut for Oak veneers.

        - Veneer Leaf Match: Book match.

        - Veneer Leaf Match: Slip match.

        - Veneer Face Match/Assembly: Running.

        - Veneer Face Match/Assembly: Balance.

        - Veneer Face Match/Assembly: Center-balance.

** NOTE TO SPECIFIER ** Specify veneer designs with more than one species in next subparagraphs. Delete preceding "matching" and "assembly" subparagraphs.

        - Decorative Arrangement: See door elevations on Drawings.

        - Decorative Arrangement, Manufacturer Designation: __________.

        - Pair Match: For doors hung in same opening or separated only by mullions.

** NOTE TO SPECIFIER ** Delete edges, core, thickness, and pattern options not required. 1-3/8 thick Solid-Core Doors are only available with Heavy Duty Particleboard core.

        - Vertical Edges, SCL: Matching/Compatible Veneer edge band over SCL.

        - Vertical Edges, SCL: Matching/Compatible Hardwood lumber. Match face veneer.

        - Vertical Edges, SCL: Matching 1/8 inch high impact edge bonded to SCL.

        - Horizontal Edges, SCL: Hardwood.

        - Horizontal Edges, SCL: Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

        - Horizontal Edges, SCL: Veneer edge band-top rail only.

        - Core: Wood-based Particleboard (PC).

        - Core: Extra heavy duty wood-based particleboard (PC).

        - Core: Structural composite lumber (SCLC).

        - Thickness: 1-3/8 inch.

        - Thickness: 1-3/4 inch.

        - Pattern: See door elevations on Drawings.

        - Pattern, Manufacturer designation: ________.

        - Construction: Five Plies; stiles and rails bonded to core. Unit is abrasive planed before veneering.

** NOTE TO SPECIFIER ** . Moldings Design options are A, B, C, D, E, F, G, H, and J.

        - Wood Applied Moldings Design: ____.

      - Solid Core Select Wood Veneer Flush Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

** NOTE TO SPECIFIER ** Delete factory install option not required.

          - Factory Install Glass: As required.

          - Factory Install Glass: In fire rated doors only.

        - Wood Glazing Accessories: Solid wood of same species as face veneer in standard profile selected by Architect.

** NOTE TO SPECIFIER ** Delete bead, grille, finish, and frame options not required.

        - Bead Profile: Straight Lip.

        - Bead Profile: Beveled Lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with Reveal.

        - Bead Profile: Straight Lip-insulated glass.

        - Applied Grille: MD Kit Lip Bead.

        - Applied Grille: MD Kit Beveled Bead.

        - Applied Grille: Classic Grille Straight Bead.

        - Applied Grille: Classic Grille Beveled Bead.

        - Metal Glazing Frames: Formed 0.048 inch thick, cold-rolled steel sheet.

          - Factory Finish: Primed for paint.

          - Factory Finish: Baked-enamel. Color: As indicated.

          - Factory Finish: Powder-coated. Color: As indicated.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For 2 lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

      - Solid Core Select Wood Veneer Flush Doors with Louvers:

** NOTE TO SPECIFIER ** Delete louver and finish options not required.

        - Wood Louvers: Sight-proof.

        - Wood Louvers: Round slat.

        - Wood Louvers: Flat slat.

        - Metal Louvers: 600A1. Type: Chevron. Smaller blades with closer spacing.

        - Metal Louvers: 800A1. Type: Chevron.

        - Metal Louvers Finish: Factory primed for field painting.

        - Metal Louvers Finish: Painted. Color: As selected by Architect.

        - Metal Louvers Finish: Painted. Color: Match Architect's sample.

      - Fire-Rated Select Wood Veneer Flush Doors:

** NOTE TO SPECIFIER ** Delete fire rating and core options not required.

        - Fire Rating: Category A positive pressure.

        - Fire Rating: Category B positive pressure.

        - Fire Rating: Category A neutral pressure.

        - Fire Rating: Category B neutral pressure.

        - Core: 20-Minute Doors. Wood-based particleboard core.

        - Core: 20-Minute Doors. Extra-heavy duty Wood-based particleboard core.

        - Core: 20-Minute Doors. Structural composite lumber core.

        - Core: 45-Minute Doors. Fire resistant particleboard core.

        - Core: 45-Minute Doors. Mineral core with blocking options.

        - Core: 60-Minute Doors. Mineral core with blocking options.

        - Core: 90-Minute Doors. Mineral core with blocking options.

** NOTE TO SPECIFIER ** Delete rating and matching options not required.

        - 20-Minute Doors: Matching/Compatible Veneer edge-band over SCL.

          - Matching: Compatible hardwood.

          - Matching: 1/8 inch high impact edge bonded to SCL.

        - 45-Minute Particleboard Core Doors: Veneer edge-band over fire stile.

          - Matching: 1/8 inch high impact edge bonded to SCL.

        - 45 Minute Mineral Core Doors: Veneer edge-band over fire stile.

          - Hardwood composite fire stile.

          - Matching: 1/8 inch high impact edge bonded to SCL.

        - 60 Minute Mineral Core Doors: Veneer edge-band over fire stile.

          - Hardwood composite fire stile.

          - Matching: 1/8 inch high impact edge bonded to SCL.

        - 90 Minute Mineral Core Doors: Veneer edge-band over fire stile.

          - Hardwood composite fire stile.

          - Matching: 1/8 inch high impact edge bonded to SCL.

** NOTE TO SPECIFIER ** Delete edges options not required.

        - Horizontal Edges, 20 Minute Doors: SCL. Hardwood.

        - Horizontal Edges, 20 Minute Doors: SCL. Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

        - Horizontal Edges, 20 Minute Doors: SCL. Veneer edge band-top rail only.

        - Horizontal Edges, 45 Minute Fire Resistant Particleboard Core Doors:

          - Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

          - Veneer edge band-top rail only.

        - Horizontal Edges: 45 Minute Doors: Per label service listing.

          - Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

          - Veneer edge band-top rail only..

        - Horizontal Edges: 60 Minute Doors: Per label service listing.

          - Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

          - Veneer edge band-top rail only..

        - Horizontal Edges: 90 Minute Doors: Per label service listing.

          - Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

          - Veneer edge band-top rail only..

      - Fire-Rated Select Wood Veneer Flush Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

          - Glazing: Fire-rated safety glass for door rating.

** NOTE TO SPECIFIER ** Delete factory install option not required.

          - Factory Install Glass: As required.

          - Factory Install Glass: In fire rated doors only.

        - Wood Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

          - Bead Profile: Straight lip.

          - Bead Profile: Beveled lip.

          - Bead Profile: Straight.

          - Bead Profile: Beveled.

          - Bead Profile: Straight with reveal.

          - Bead Profile: Straight lip-insulated glass.

          - Applied Grille: MD Kit lip bead.

          - Applied Grille: MD Kit beveled bead.

          - Applied Grille: Classic grille straight bead.

          - Applied Grille: Classic grille beveled bead.

        - Wood-Core Doors with 20 minute Fire-Protection Ratings: Provide wood beads and metal glazing clips as required and approved for such use.

        - Veneer Wrap Bead for Light Openings in Fire-Rated Doors: Match veneer species of door face and approved for use in doors of fire-protection rating indicated. Include concealed metal glazing clips where required for opening size and fire-protection rating indicated.

        - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet approved for use in doors of fire-protection rating indicated.

** NOTE TO SPECIFIER ** Delete frame and finish design options not required.

          - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

          - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

          - Metal Vision Frame: 115-L1. For 2 lites of glass with thermal spacer.

          - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

          - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

          - Factory Finish: Factory primed for paint.

          - Factory Finish: Baked enamel. Color: As indicated.

          - Factory Finish: Powder-coated. Color: As indicated.

      - Fire-Rated Select Wood Veneer Flush Doors with Louvers:

        - Fire-Rated Door, Metal Louvers: With fusible link and closing device, listed and labeled for use in doors with fire-protection rating of 1-1/2 hours and less.

        - Metal: 18 gauge cold rolled steel,

          - Finish: Factory primed for paint,

          - Finish: Baked enamel. Color: As indicated.

          - Finish: Powder coated. Color: As indicated.

    - Interior Solid Core Flush Wood Doors for Opaque Finish - Premium Painted.

      - Solid Core Premium Painted Flush Doors

** NOTE TO SPECIFIER ** Delete options not required.

        - WDMA Quality Grade: Custom.

        - WDMA Quality Grade: Premium.

        - WDMA Performance Level: Heavy Duty.

        - WDMA Performance Level: Extra Heavy Duty.

        - Faces: Medium density overlay (MDO).

        - Vertical Edges: Hardwood over Structural composite lumber.

          - Matching 1/8 inch high impact edge bonded to SCL.

        - Horizontal Edges: SCL. Hardwood.

        - Horizontal Edges: SCL. Clean edge-Bond smooth PVC edge band (top), (bottom), (top and bottom).

** NOTE TO SPECIFIER ** SCLC does not require blocking for hardware as PC does

        - Core: Wood-based Particleboard (PC).

        - Core: Extra heavy duty wood-based particleboard (PC).

        - Core: Structural composite lumber (SCLC).

        - Thickness: 1-3/8 inch.

        - Thickness: 1-3/4 inch.

        - Thickness: 2 inch.

        - Thickness: 2-1/4 inch.

        - Pattern: See door elevations on Drawings.

        - Pattern, Manufacturer designation: ________.

** NOTE TO SPECIFIER ** Moldings Design options are A, B, C, D, E, F, G, H, J

        - Wood Applied Moldings Design: ____.

        - Construction: Five Plies; stiles and rails bonded to core, and entire unit is abrasive planed before veneering.

      - Solid Core Premium Painted Flush Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

** NOTE TO SPECIFIER ** Delete factory install option not required.

          - Factory Install Glass: As required.

          - Factory Install Glass: In fire rated doors only.

        - Wood Glazing Accessories: Solid wood of same species as face veneer in standard profile selected by Architect.

** NOTE TO SPECIFIER ** Delete bead and grill options not required.

          - Bead Profile: Straight Lip.

          - Bead Profile: Beveled Lip.

          - Bead Profile: Straight.

          - Bead Profile: Beveled.

          - Bead Profile: Straight with Reveal.

          - Bead Profile: Straight Lip-insulated glass.

          - Applied Grille: MD Kit Lip Bead.

          - Applied Grille: MD Kit Beveled Bead

          - Applied Grille: Classic Grille Straight Bead.

          - Applied Grille: Classic Grille Beveled Bead.

        - Metal Glazing Frames: Formed 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete frame and finish options not required.

          - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

          - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

          - Metal Vision Frame: 115-L1. For 2 lites of glass with thermal spacer.

          - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

          - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

          - Factory Finish: Primed for paint.

          - Factory Finish: Baked-enamel. Color: As indicated.

          - Factory Finish: Powder-coated. Color: As indicated.

      - Solid Core Premium Painted Flush Doors with Louvers:

** NOTE TO SPECIFIER ** Delete lovers and finish options not required.

        - Wood Louvers: Sight-proof.

        - Wood Louvers: Round slat.

        - Wood Louvers: Flat slat.

        - Metal Louvers: 600A1. Type: Chevron. Smaller bladed with closer spacing.

        - Metal Louvers: 800A1. Type: Chevron.

        - Louver Finish: Factory primed for field painting.

        - Louver Finish: Paint. Color: As selected by Architect.

        - Louver Finish: Paint. Color: To match Architect's color sample.

      - Fire-Rated Premium Painted Flush Doors:

** NOTE TO SPECIFIER ** Delete ratings, core, and edges options not required.

        - Ratings: Category A positive pressure.

        - Ratings: Category B positive pressure.

        - Ratings: Neutral pressure.

        - Core: 20-Minute Doors. Wood-based particleboard core.

        - Core: 20-Minute Doors. Extra-heavy duty Wood-based particleboard core.

        - Core: 20-Minute Doors. Structural composite lumber core.

        - Core: 45-Minute Doors. Fire resistant particleboard core.

        - Core: 45-Minute Doors. Mineral core.

        - Core: 45-Minute Doors. Mineral core.

        - Core: 60-Minute Doors. Mineral core with blocking options.

        - Core: 90-Minute Doors. Mineral core with blocking options.

        - Vertical Edges, 20-Minute Doors: Hardwood Over SCL.

          - Matching 1/8 inch high impact acrylic edge bonded to SCL.

        - Vertical Edges, 45-Minute Fire Resistant Particleboard Core Doors:

          - Matching 1/8 inch high impact edge bonded to SCL.

        - Vertical Edges, 45 Minute Mineral Core Doors: Hardwood composite fire stile.

          - Matching 1/8 inch high impact acrylic edge bonded to SCL.

        - Vertical Edges, 60 Minute Mineral Core Doors: Hardwood composite fire stile.

          - Matching 1/8 inch high impact acrylic edge bonded to SCL.

        - Vertical Edges, 90 Minute Mineral Core Doors: Hardwood composite fire stile.

          - Matching 1/8 inch high impact acrylic edge bonded to SCL.

        - Horizontal Edges, 20-Minute Doors: SCL. Hardwood.

        - Horizontal Edges, 20-Minute Doors: Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

        - Horizontal Edges, 45-Minute Fire Resistant Particleboard Core Doors:

          - Veneer edge band-top rail only.

        - Horizontal Edges, 45-Minute Fire Resistant Particleboard Core Doors:

          - Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

        - Horizontal Edges: Manufactures construction per label service listing. Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

          - 45 Minute Doors.

          - 60 Minute Doors.

          - 90 Minute Doors.

      - Fire-Rated Premium Painted Flush Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

          - Glazing: Factory-installed fire-rated safety glass for door rating.

** NOTE TO SPECIFIER ** Delete factory install option not required.

          - Factory Install Glass: As required.

          - Factory Install Glass: In fire rated doors only.

        - Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead and grille options not required.

          - Bead Profiles: Straight Lip.

          - Bead Profiles: Beveled Lip.

          - Bead Profiles: Straight.

          - Bead Profiles: Beveled.

          - Bead Profiles: Straight with Reveal.

          - Bead Profiles: Straight Lip-insulated glass.

          - Applied Grille: MD Kit Lip Bead.

          - Applied Grille: MD Kit Beveled Bead.

          - Applied Grille: Classic Grille Straight Bead.

          - Applied Grille: Classic Grille Beveled Bead.

        - Wood-Core Doors with 20 Minute Fire-Protection Ratings: Use wood beads and metal glazing clips as required and approved for such use.

        - Veneer Wrap Bead for Light Openings in Fire-Rated Doors: Match veneer species of door face and approved for use in doors of fire-protection rating indicated. Include concealed metal glazing clips where required for opening size and fire-protection rating indicated.

        - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete frame and finish options not required.

          - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

          - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

          - Metal Vision Frame: 115-L1. For 2 lites of glass with thermal spacer.

          - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

          - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

          - Finish: Factory primed for paint.

          - Finish: Baked enamel. Color: As indicated.

          - Finish: Powder-coated. Color: As indicated.

      - Fire-Rated Select Wood Veneer Flush Doors with Louvers:

        - Metal Louvers for Fire-Rated Doors: With fusible link and closing device, listed and labeled for use in doors with fire-protection rating of 1-1/2 hours and less.

        - Metal and Finish: 18 gauge cold rolled steel,

** NOTE TO SPECIFIER ** Delete finish options not required.

          - Finish: Factory primed for paint.

          - Finish: Baked enamel. Color: As indicated.

          - Finish: Powder-coated. Color: As indicated.

    - Laminate Finish Wood Doors:

      - Interior Solid Core Choice Laminate (HPDL) Flush Doors:

** NOTE TO SPECIFIER ** Delete grade, performance, colors, edges, core, and thickness options not required.

        - WDMA Quality Grade: Custom.

        - WDMA Quality Grade: Premium.

        - WDMA Performance Level: Heavy Duty.

        - WDMA Performance Level: Extra Heavy Duty.

        - Faces: High-pressure decorative laminates complying with NEMA LD 3, Grade HGS. Vertical and post formable grade laminates are not acceptable.

        - Colors, Patterns: Selected by Architect from manufacturer's standards.

        - Colors, Patterns: Match Architect sample.

        - Vertical Edges: Matching HPDL Edge Band applied after faces over SCL.

          - Finish: Hardwood for paint finish.

          - Finish: Hardwood for stain finish.

        - Vertical Edges: Matching 1/8 inch high impact edge, bonded to SCL.

          - Finish: Hardwood for paint finish.

          - Finish: Hardwood for stain finish.

        - Horizontal Edges: SCL. Hardwood.

        - Horizontal Edges: SCL. HPDL edge band (top rail only).

        - Horizontal Edges: SCL. Clean edge-Bond smooth PVC edge band to structural composite lumber (top), (bottom), (top and bottom).

        - Core: Wood-based Particleboard (PC).

        - Core: Extra heavy duty wood-based particleboard (PC).

        - Core: Structural composite lumber (SCLC).

        - Thickness: 1-3/4 inch.

        - Thickness: 1-3/8 inch, for SCL and wood-based particleboard cores.

        - Construction: Five Plies; stiles and rails bonded to core, and entire unit is abrasive planed. High pressure decorative laminate with high density fiberboard (HDF) crossbands.

      - Solid Core Choice Laminate (HPDL) Flush Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

** NOTE TO SPECIFIER ** Delete factory install options not required.

          - Factory Install Glass: As required.

          - Factory Install Glass: In fire rated doors only.

        - Wood Glazing Accessories: Solid wood of same species as face veneer in standard profile selected by Architect.

** NOTE TO SPECIFIER ** Delete bead and grille options not required.

          - Bead Profile: Straight Lip.

          - Bead Profile: Beveled Lip.

          - Bead Profile: Straight.

          - Bead Profile: Beveled.

          - Bead Profile: Straight with Reveal.

          - Bead Profile: Straight Lip-insulated glass.

          - Applied Grille: MD Kit Lip Bead.

          - Applied Grille: MD Kit Beveled Bead,

          - Applied Grille: Classic Grille Straight Bead.

          - Applied Grille: Classic Grille Beveled Bead.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet.

** NOTE TO SPECIFIER ** Delete frame and finish options not required.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For 2 lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

        - Finish: Factory primed for paint.

        - Finish: Baked enamel. Color: As indicated.

        - Finish: Powder-coated. Color: As indicated.

      - Solid Core Premium Painted Flush Doors with Louvers:

** NOTE TO SPECIFIER ** Delete louvers and finish options not required.

        - Wood Louvers: Sight-proof.

        - Wood Louvers: Round slat.

        - Wood Louvers: Flat slat.

        - Metal Louvers: 600A1. Type: Chevron. Smaller bladed with closer spacing.

        - Metal Louvers: 800A1. Type: Chevron.

        - Louver Finish: Factory primed for field painting.

        - Louver Finish: Paint. Color: As selected by Architect.

        - Louver Finish: Paint. Color: To match Architect's color sample.

      - Fire-Rated Choice Laminate (HPDL) Flush Doors:

** NOTE TO SPECIFIER ** Delete rating, core, and edges options not required.

        - Rating: Category A positive pressure.

        - Rating: Category B positive pressure.

        - Core, 20 Minute Doors: Wood-based particleboard.

        - Core, 20 Minute Doors: Structural composite lumber.

        - Core, 45 Minute Doors: Fire resistant particleboard.

        - Core, 45 Minute Doors: Mineral with blocking options.

        - Core, 60 Minute Doors: Mineral with blocking options.

        - Core, 90 Minute Doors: Mineral with blocking options.

        - Vertical Edges: High pressure decorative laminate over hardwood composite fire stile.

        - Vertical Edges: Hardwood composite fire stile with transparent finish.

        - Vertical Edges: Impact resistant edges.

      - Fire-Rated Choice Laminate (HPDL) Flush Doors with Glazed Lites:

        - Factory Glazing: Refer to Section 08 80 00 "Glazing" for glass view panels in flush wood doors. Fill glazing bead nail holes in factory finished doors.

          - Glazing: Fire-rated safety glass for door rating.

** NOTE TO SPECIFIER ** Delete factory install option not required.

          - Factory Install Glass: As required.

          - Factory Install Glass: In fire rated doors only.

      - Wood Glazing Accessories: Solid wood of same species as face veneer.

** NOTE TO SPECIFIER ** Delete bead profile and applied grille options not required.

        - Bead Profile: Straight lip.

        - Bead Profile: Beveled lip.

        - Bead Profile: Straight.

        - Bead Profile: Beveled.

        - Bead Profile: Straight with reveal.

        - Bead Profile: Straight lip-insulated glass.

        - Applied Grille: MD Kit lip bead.

        - Applied Grille: MD Kit beveled bead.

        - Applied Grille: Classic grille straight bead.

        - Applied Grille: Classic grille beveled bead.

        - Wood-Core Doors with 20 Minute Fire-Protection Ratings: Wood beads and metal glazing clips as required and approved for such use.

      - Veneer Wrap Bead for Light Openings in Fire-Rated Doors: Matching veneer species of door face and approved for use in doors of fire-protection rating indicated.

        - Concealed Metal Glazing Clips: As required for opening and fire protection rating.

      - Metal Glazing Frames: Formed of 0.048 inch thick, cold-rolled steel sheet and approved for use in doors of fire-protection rating indicated.

** NOTE TO SPECIFIER ** Delete frame and finish design options not required.

        - Metal Vision Frame: 110. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 110 Veneer Wrapped. Flanges: 1-1/4 inches.

        - Metal Vision Frame: 115-L1. For 2 lites of glass with thermal spacer.

        - Metal Vision Frame: 115-L2. Flanges: 1-1/2 inches.

        - Metal Vision Frame: 115-L3. Flanges: 1-1/2 inches.

        - Factory Finish: Factory primed for paint.

        - Factory Finish: Baked enamel. Color: As indicated.

        - Factory Finish: Powder-coated. Color: As indicated.

      - Fire-Rated Select Wood Veneer Flush Doors with Louvers:

        - Louvers for Fire-Rated Doors: Metal louvers with fusible link and closing device, listed and labeled for use in doors with fire-protection rating of 1-1/2 hours and less.

        - Metal and Finish: 18 gauge cold rolled steel.

** NOTE TO SPECIFIER ** Delete finish options not required.

          - Factory Finish: Factory primed for paint.

          - Factory Finish: Baked enamel. Color: As indicated.

          - Factory Finish: Powder-coated. Color: As indicated.

    - Integrated Functional Surface Doors:

      - Recessed functional surface to be flush with door face.

** NOTE TO SPECIFIER ** Delete surface options not required.

        - Functional Surface: Tackable bulletin board. Color: Selected by Architect.

        - Functional Surface: White dry erase marker board.

        - Functional Surface: White magnet/dry erase marker board.

        - Functional Surface: Tempered glass mirror.

        - Size: Selected by Architect.

    - Radiation Shielded Interior Flush Wood Doors:

** NOTE TO SPECIFIER ** Delete thickness and astragals finish options not required.

      - Lead Thickness: 1/16 inch.

      - Lead Thickness: 1/8 inch.

      - Lead Thickness: See door schedule on Drawings.

      - Astragals: Wood/lead astragals on radiation shielded pairs in lead thickness indicated.

        - Finish: Match veneer face.

        - Finish: Match laminate face.

    - Acoustic (STC) Rated Interior Flush Wood Doors:

      - STC Ratings: Certified by manufacturer on basis of testing per ASTM E90 and E413.

        - Rating Requirements: Stated in the door schedule on the Drawings.

      - Gasketing: As required to meet manufacturers tested acoustic rating.

      - Door Shoe: As required to meet manufacturers tested acoustic rating.

      - Mortise Door Bottom: As required to meet manufacturers tested acoustic rating.

    - Ballistic Rated Interior Wood Doors: UL Ratings: Stated in door schedule on the Drawings.

    - Stile and Rail Doors:

      - Basis of Design: Aspiro Series Marshfield-Algoma by Forte Opening Solutions - Authentic Stile and Rail.

      - Interior Stile and Rail Doors with Transparent Finish:

** NOTE TO SPECIFIER ** Delete grade, performance, type, and arrangements options not required.

        - WDMA Quality Grade: Custom.

        - WDMA Quality Grade: Premium.

        - WDMA Performance Level: Heavy Duty.

        - WDMA Performance Level: Extra Heavy Duty.

        - Panel Type: Single glass panel.

        - Panel Type: Multiple glass panels.

        - Panel Type: Wood panels.

        - Panel Type: Combination glass and wood panels.

        - Panel Type: Wood louvers.

        - Panel Type: As indicated in door elevations on Drawings.

        - Panel Arrangement: As indicated in door elevations on Drawings.

        - Panel Arrangement: ________.

        - Stiles and Rails: Veneer on structural composite lumber.

** NOTE TO SPECIFIER ** Delete species options not required.

        - Veneer Species: __________.

        - Assembly: Center balance.

      - Glazed Panels:

        - Glazing Moldings: Same species as face veneer in standard profile selected by Architect.

        - Mullions: Same species as face veneer in standard profile selected by Architect.

** NOTE TO SPECIFIER ** Delete glass option not required.

        - Glass: Fully tempered 6 mm clear glass.

      - Interior Stile and Rail Doors with Opaque Finish:

** NOTE TO SPECIFIER ** Delete grade, performance, type, arrangement, and stile and rail options not required.

        - WDMA Quality Grade: Custom.

        - WDMA Quality Grade: Premium.

        - WDMA Performance Level: Heavy Duty.

        - WDMA Performance Level: Extra Heavy Duty.

        - Panel Type: Single glass.

        - Panel Type: Multiple glass.

        - Panel Type: Wood.

        - Panel Type: Combination of glass and wood.

        - Panel Type: Wood louvers.

        - Panel Type: As indicated in door elevations on Drawings.

        - Panel Arrangement: See door schedule on Drawings.

        - Panel Arrangement: _______.

        - Stiles and Rails: Medium density overlay (MDO) on SCL.

        - Stiles and Rails: Paint grade hardwood veneer on SCL.

        - Glass: Laminated glass comprised of two layers of 3 mm clear glass and clear, colorless interlayer.

      - Wood Panels:

** NOTE TO SPECIFIER ** Delete design and louvers options not required.

        - Design: Raised panels in standard profile selected by Architect.

        - Design: Flat panels.

        - Panel Moldings: Same species as face veneer in standard profile selected by Architect.

        - Louvers: Solid wood in same species as door face. Design: Slat.

        - Louvers: Solid wood in same species as door face. Design: Chevron.

      - Glazed Panels:

        - Glazing Moldings: Same species as face veneer. Profile selected by Architect.

        - Mullions: Same species as face veneer. Profile selected by Architect.

** NOTE TO SPECIFIER ** Delete glass options not required.

        - Glass: Fully tempered 6 mm clear.

        - Glass: Laminated. 2 layers of 3 mm clear glass. Clear, colorless interlayer.

      - Wood Panels:

** NOTE TO SPECIFIER ** Delete design, louvers, and finish options not required.

        - Design: Raised panels in standard profile selected by Architect.

        - Design: Flat panels.

        - Panel Moldings: Same species as face veneer. Profile selected by Architect.

        - Louvers: Paint grade hardwood. Design: Slat.

        - Louvers: Paint grade hardwood. Design: Chevron.

        - Finish: Factory-applied primer.

        - Finish: Factory-applied primer and finish paint.

      - Interior Fire-Rated Stile and Rail Wood Doors:

** NOTE TO SPECIFIER ** Delete grade, performance, ratings, and glazing frame options not required.

        - WDMA Quality Grade: Custom.

        - WDMA Quality Grade: Premium.

        - WDMA Performance Level: Heavy Duty.

        - WDMA Performance Level: Extra Heavy Duty.

        - Ratings: Category A positive pressure.

        - Ratings: Category B positive pressure.

        - 20-Minute Doors: Structural composite lumber core.

          - Glazing: Fire-rated safety glass conforming to Section 08 80 00.

          - Glazing Frame: Solid wood of same species as face veneer.

          - Glazing Frame: Metal. Finish: Baked enamel. Color: as indicated.

          - Glazing Frame: Metal. Finish: Powder-coat. Color: as indicated.

      - Exterior Stile and Rail Doors with Transparent Finish:

** NOTE TO SPECIFIER ** Delete grade, performance, panel type, arrangement, species, cut, and stile and rail door options not required.

        - WDMA Quality Grade: Custom.

        - WDMA Quality Grade: Premium.

        - WDMA Performance Level: Heavy Duty.

        - WDMA Performance Level: Extra Heavy Duty.

        - Panel Type: Single glass panel.

        - Panel Type: Multiple glass panels.

        - Panel Type: Wood panels.

        - Panel Type: Combination glass and wood panels.

        - Panel Type: As indicated in door elevations on Drawings.

        - Panel Arrangement: As indicated in door elevations on Drawings]

        - Panel Arrangement: ________.

        - Exposed Faces: Species: ________.

        - Cut: Flat cut.

        - Cut: Quarter cut.

        - Cut: Rift cut. For red and white oak.

        - Stiles and Rails: Clear lumber; top and bottom rails may be edge glued to achieve required width.

        - Stiles and Rails: Veneer on structural composite lumber core (SCLC).

      - Glazed Panels:

        - Glazing Moldings: Same species as face veneer. Profile selected by Architect.

** NOTE TO SPECIFIER ** Delete glass options not required.

        - Glass: Tempered 6 mm clear glass.

        - Glass: Laminated glass of two layers of 3 mm clear glass.

        - Glass: Tempered insulating glass conforming to Section 08 80 00.

** NOTE TO SPECIFIER ** Delete panels option not required.

      - Wood Panels: Raised panels. Profile selected by Architect.

      - Wood Panels: Flat panels.

      - Panel Moldings: Same species as face veneer. Profile selected by Architect.

      - Sidelites: Match appearance of adjacent door.

    - Door Core Materials:

      - Particleboard: Wood-based particleboard; ANSI A208.4, Grade LD-2 as required to meet WDMA Performance Duty level specified without added blocking..

      - Structural Composite Lumber (SLC): WDMA T.M.10.

    - Transparent Finish Wood Frame:

** NOTE TO SPECIFIER ** Delete material, profile, trim, and trim stile options not required.

      - Material: Solid lumber that matches species of door veneer.

      - Material: Door face veneer applied to structural composite lumber.

      - Frame Profile: Standard double rabbet.

      - Frame Profile: Custom double rabbet.

      - Trim: Solid lumber that matches species of door veneer.

      - Trim: Door face veneer applied to lumber or medium density fiberboard.

      - Trim Stile: Colonial Casing.

      - Trim Stile: Contemporary Casing.

      - Trim Stile: Sleek Casing.

      - Trim Stile: Traditional Casing.

** NOTE TO SPECIFIER ** Wood frames are available in 20, 45, 60, and 90 minute ratings.

      - Fire Rating: Match rating of door.

    - Paint finish Wood Door Frames:

      - Material: Paint grade hardwood.

** NOTE TO SPECIFIER ** Delete profile, trim, and trim stile options not required. Wood frames can be supplied without casing trim.

      - Frame Profile: Standard double rabbet.

      - Frame Profile: Custom double rabbet.

      - Trim: Paint grade softwood.

      - Trim: Paint grade medium density fiberboard.

      - Trim Stile: Colonial Casing.

      - Trim Stile: Contemporary Casing.

      - Trim Stile: Sleek Casing.

      - Trim Stile: Traditional Casing.

** NOTE TO SPECIFIER ** Wood frames are available in 20, 45, 60, and 90 minute ratings.

      - Fire Rating: Match rating of door.

    - Fabrication:

      - Door Pairs and Fixed Side Panels:

** NOTE TO SPECIFIER ** Delete veneer matching options not required. First two options are for AA veneer. The second is for installations where there is more than one pair. Running match is the most common assembly method for A grade veneer.

        - Veneer Matching: Pair match.

        - Veneer Matching: Set match.

        - Veneer Matching: Running match.

      - Transoms and Side Panels: Same construction as adjacent doors.

** NOTE TO SPECIFIER ** Delete transom veneer option not required.

        - Transom Veneer: Continuous match. From floor to top of the transom.

        - Transom Veneer: End match. Transoms have mirror match at top of door.

      - Flush Dutch Doors:

** NOTE TO SPECIFIER ** Delete finish and shelf options not required.

        - Finish: Transparent.

        - Finish: Opaque.

        - Finish: Laminate.

        - Shelf: Not required.

        - Shelf: 8 inches wide.

        - Shelf: 10 inches wide.

        - Shelf: 12 inches wide.

      - Factory Fitting: Fit to frame openings with clearances specified in WDMA I.S. 1A.

        - Undercut: Maximum 3/8 inch above thresholds.

        - Fire-Rated Doors: Comply with NFPA 80.

      - Factory Machining: Machine doors for hardware that is not surface applied.

        - Verify dimensions for hardware mortises in metal frames before machining.

      - Openings: Cut and trim openings at factory. Install glazing at factory.

        - Cut openings and install louvers at factory.

      - Exterior Doors: Treat with water-repellent preservative per WDMA I.S.4.

        - Flash tops of out swinging doors with stainless steel.

      - Doors supplied with wood frames may have hardware installed at factory.

      - Door Frames: Supply frames with temporary spreader bars at base.

        - Where Indicated Provide:

          - Matching sidelite frames with rectangular wood beads for glazing.

          - Matching transom frames with rectangular wood beads for glazing.

    - Finishes: Match door grade.

      - Transparent: WDMA TR-8, UV-Cured Acrylated Polyester/Urethane.

        - Staining: As indicated.

        - Sheen: Satin.

      - Factory-Primed: For finish paint specified in the interior painting section.

** NOTE TO SPECIFIER ** Factory finish painting is available only for interior doors.

      - Factory-Painted: WDMA I.S. 1-A System OP-8, UV-Cured Acrylated Polyester / Urethane.

        - Color: As indicated.

        - Sheen: Satin.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until the substrates have been properly constructed and prepared.

      - If substrate preparation is the responsibility of another installer, notify Architect in writing of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

      - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

  - FIELD QUALITY CONTROL

    - Field Inspection: Coordinate in accordance with appropriate sections in Division 01.

  - CLEANING AND PROTECTION

    - Clean products in accordance with the manufacturers recommendations.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION