SECTION 07 46 46.13

FIBER CEMENT SIDING - HARDIE ARTISAN SIDING, SOFFIT, AND TRIM

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2026 - 2026 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** James Hardie Building Products, Inc.; HardieTM Architectural Collection.
This section is based on the products of James Hardie Building Products, Inc., which is located at:
303 E. Wacker Dr.
Chicago, IL 60601 
Toll Free Tel: 877-236-7526
Email: request info (info@jameshardie.com)
Web: https://www.jameshardiepros.com | https://www.jameshardie.com  

 [ Click Here ] for additional information.

Our operations encompass the North America, Australia, and New Zealand. James Hardie Building Products are also now available in Asia, with a manufacturing plant in the Philippines; in Europe, our operations are headquartered in Amsterdam. In every country in which we operate, our goal is to remain at the forefront of the fiber-cement industry, capitalizing on our global leadership in products, manufacturing, and technology for both new home construction and remodeling.
Innovation is vital to maintaining our pre-eminent position in the fiber-cement industry. Consequently, we invest heavily in research and development, with a special emphasis on product quality and production efficiency to develop durable and low maintenance siding products and backerboard.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Fiber cement lap, v-groove, shiplap, and square channel siding.

    - Fiber cement non-vented soffit panels.

    - Fiber cement vented soffit panels.

    - Fiber cement vented plus soffit panels.

    - Trim, Fascia, Molding, and Accessories.

  - RELATED REQUIREMENTS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 05 12 00 - Structural Steel Framing.

    - Section 05 40 00 - Cold-Formed Metal Framing.

    - Section 06 10 00 - Rough Carpentry.

    - Section 07 21 00 - Thermal Insulation.

    - Section 07 25 00 - Weather Barriers.

    - Section 07 92 00 - Joint Sealants.

    - Section 09 21 16 - Gypsum Board Assemblies.

    - Section 09 91 13 - Exterior Painting.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ASTM International (ASTM):

      - ASTM C1185, Standard Test Methods for Sampling and Testing Fiber-Cement Flat Sheet, Roofing and siding Shingles, and Clapboards; 2023.

      - ASTM C1186., Standard Specification for Flat Fiber-Cement Sheets; 2022, with 2023 Editorial Revision.

      - ASTM D3359, Standard Test Methods for Rating Adhesion by Tape Test; 2023.

      - ASTM E84, Standard Test Method for Surface Burning Characteristics of Building Materials; 2020.

      - ASTM E119, Standard Test Methods for Fire Tests of Building Construction and Materials; 2024.

      - ASTM E136, Standard Test Method for Assessing Combustibility of Materials Using a Vertical Tube Furnace at 750 Degrees C; 2022.

      - ASTM E330/E330M, Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference; 2021.

    - International Cose Council Evaluation Reports (ICC-ES):

      - ICC-ES Evaluation Reports ESR-2290 and ESR-2273 (IBC, IRC, CBC, CRC); Current Editions.

** NOTE TO SPECIFIER ** Other References you may want to include based on location of project.

    - CAL-FIRE, Fire Engineering Division Building Materials Listing - Wildland Urban Interface (WUI) Listed Product.

    - Florida State Product Approval -FL10477 and FL13265.

    - Miami Dade County, -Notice of Acceptance -23-1012.05 and 24-0221.09.

    - Texas Department of Insurance Product Evaluation EC-55.

    - US Department of Housing and Urban Development Materials Release -1263h.

  - SUBMITTALS.

    - See Section 01 30 00, Administrative Requirements.

    - Product Data:

      - Manufacturer's product data sheets.

      - Manufacturer's storage and handling requirements.

      - Manufacturer's preparation instructions.

      - Manufacturer's installation instructions.

      - Manufacturer's operation and maintenance instructions.

    - Shop Drawings:

      - Provide detailed drawings showing dimensions, layout, joints, construction details, and attachments.

      - Indicate atypical non-standard applications outside of manufacturer's standard details and specifications.

    - Selection Samples:

      - Provide two complete sets of color chips representing manufacturer's full range of available colors and patterns for each finish product specified.

    - Verification Samples:

      - Provide two samples, minimum 4-inch x 6-inch (100 mm x 150 mm) representing selected product, color, texture, and patterns for each finish product specified.

    - Installers qualification statement.

    - Warranty:

      - Manufacturer's sample warranty.

      - Manufacturer's executed written warranty, completed in Owner's name, and registered with manufacturer.

  - QUALITY ASSURANCE

    - Installer Qualifications: Company specializing in work specified in this section with a minimum of ___ years of experience with installation of products of this type.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up:

      - Mock-Up: Build mock-up for evaluation of surface preparation techniques, installation workmanship, and to verify selections made from sample submittals to demonstrate aesthetic effects and set quality standards for fabrication and installation.

        - Minimum 48 inches wide x 60 inches high or similar area as fits the layout of the building exterior.

        - Include outside corner on one end of mockup and inside corner on the other end.

        - Finish areas designated by Architect.

        - Approval of mock-up does not constitute approval of deviations from the Contract Documents unless approved in writing.

        - Approved Mock-Up: May remain as a part of the completed work.

        - Approved Mock-Up: May not remain as a part of the completed work.

      - Provide a minimum of ___ days notification to Architect, Installer, and Manufacturer's Representative prior to application of mock-up for review.

      - Obtain approval of mock-up from Architect prior to commencement of Work.

  - PRE-INSTALLATION CONFERENCE

    - Pre-Installation Conference:

      - A minimum of 2 weeks prior to the commencement of the Work of this section, conduct a preliminary pre-installation conference at project site.

      - Meet with Owner, Architect, and Installer, and Subcontractors completing related work.

      - Review the Following:

        - Methods and procedures to complete the Work.

        - Substrate and installation requirements.

        - Temporary protection requirements.

        - Repair procedures.

        - Schedule.

        - Other pertinent topics.

      - Provide record of discussion with meeting minutes distributed to all attendees.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver and store products in manufacturer's unopened packaging with labels intact until ready for installation.

    - Store siding flat on a smooth level surface and carry on its side.

    - Protect edges and corners from chipping.

    - Store materials under dry, waterproof, well-ventilated cover, elevated above grade, and kept dry prior to installation.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of the Authorities Having Jurisdiction.

    - Protect materials from damage, dust, or other detrimental conditions.

  - PROJECT FIELD CONDITIONS

    - Maintain environmental conditions including temperature, humidity, and ventilation in accordance with the limits indicated in the manufacturer's written instructions.

    - Do not install products in conditions outside of the manufacturer's stated limits.

  - WARRANTY

    - Manufacturer's Limited, Non Pro-rated Product Warranty:

      - Plank, Soffit, and Trim: 30 year Warranty.

      - Color-Plus Finish: 15 year Warranty.

      - When installed and maintained in accordance with manufacturer's written requirements, warranty for 15 years from date of purchase covering peeling, cracking, and chipping.

    - Installers Warranty:

      - Provide ____ year installers warranty covering workmanship for work of this Section.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: James Hardie Building Products, Inc., which is located at:
303 E. Wacker Dr.
Chicago, IL 60601 
Toll Free Tel: 877-236-7526
Email: request info (info@jameshardie.com);Web: https://www.jameshardiepros.com | https://www.jameshardie.com

    - Substitutions: In accordance with the requirements of Division 01, General Requirements.

    - Basis of Design: Hardie Artisan Siding, Soffit, and Trim as manufactured by James Hardie Building Products, Inc.

** NOTE TO SPECIFIER ** Delete article if not required.

  - FIBER CEMENT LAP SIDING

    - Fiber Cement Lap Siding:

      - Type A, Grade II, ASTM C1186.

      - Non-combustible Material, ASTM E136.

      - Flame Spread Index: 0, ASTM E84.

      - Smoke Developed Index: Less than 5, ASTM E84.

** NOTE TO SPECIFIER ** Delete type options not required. Then delete product options not required.

    - Type: Lap Siding:

      - Smooth: 5-1/4 inches (133 mm) with 4 inches (102 mm) exposure.

      - Smooth: 7-1/4 inches (184 mm) with 6 inches (152 mm) exposure.

      - Smooth: 8-1/4 inches (210 mm) with 7 inches (178 mm) exposure.

      - Texture: 5-1/4 inches (133 mm) with 4 inches (102 mm) exposure.

      - Texture: 7-1/4 inches (184 mm) with 6 inches (152 mm) exposure.

      - Texture: 8-1/4 inches (210 mm) with 7 inches (178 mm) exposure.

      - Beaded: 8-1/4 inches (210 mm) with 7 inches (178 mm) exposure.

    - Type: Lap Siding with Lock Joint System:

      - V-Groove: 8-1/4 inches (210 mm) with 7 inches (178 mm) exposure.

      - Shiplap: 10-1/4 inches (260 mm) with 9 inches (229 mm) exposure.

      - Square Channel: 10-1/4 inches (260 mm) with 9 inches (229 mm) exposure.

      - Bevel Channel: 10-1/4 inches (260 mm) with 9 inches (229 mm) exposure.

    - Color: __________, selected by Architect from manufacturer's standard range.

    - Basis-of-Design Product: Hardie Artisan Siding; https://www.jameshardie.com.

      - Substitutions: According to requirements of Division 01, General Requirements.

** NOTE TO SPECIFIER ** Delete article if not required.

  - FIBER CEMENT NON-VENTED SOFFIT PANELS

    - Fiber Cement Non-Vented Soffit Panels:

      - Type A, Grade II, ASTM C1186.

      - Non-combustible Material, ASTM E136.

      - Flame Spread Index: 0, ASTM E84.

      - Smoke Developed Index: Less than 5, ASTM E84.

      - Thickness: 1/4 inch (6 mm).

    - Type:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Smooth: 12 inches (305 mm) x 12 feet (3658 mm).

      - Smooth: 16 inches (406 mm) x 12 feet (3658 mm).

      - Smooth: 24 inches (610 mm) x 8 feet (2438 mm).

    - Color: __________, selected by Architect from manufacturer's standard range.

    - Basis-of-Design Products: HardieSoffit; https://www.jameshardie.com.

      - Substitutions: According to requirements of Division 01, General Requirements.

** NOTE TO SPECIFIER ** Delete article if not required.

  - FIBER CEMENT VENTED SOFFIT PANELS

    - Fiber Cement Non-Vented Soffit Panels:

      - Type A, Grade II, ASTM C1186.

      - Non-combustible Material, ASTM E136.

      - Flame Spread Index: 0, ASTM E84.

      - Smoke Developed Index: Less than 5, ASTM E84.

      - Net Free Ventilation: 5 sq inches per linear ft (10583 sq mm per linear m).

      - Thickness: 1/4 inch (6 mm).

    - Type:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Smooth: 12-inches (305 mm) x 12-feet (3658 mm).

      - Smooth: 16-inches (406 mm) x 12-feet (3658 mm),

      - Smooth: 24-inches (610 mm) x 8-feet (2438 mm).

      - Textured Cedarmill: 12-inches (305 mm) x 12-feet (3658 mm).

      - Textured Cedarmill: 16-inches (406 mm) x 12-feet (3658 mm).

      - Textured Cedarmill: 24-inches (610 mm) x 8-feet (2438 mm).

    - Color: __________, selected by Architect from manufacturer's standard range.

    - Basis-of-Design Product: HardieSoffit; https://www.jameshardie.com.

      - Substitutions: According to requirements of Division 01, General Requirements.

** NOTE TO SPECIFIER ** Delete article if not required.

  - FIBER CEMENT VENTED PLUS SOFFIT PANELS

    - Fiber Cement Vented Plus Soffit Panels:

      - Type A, Grade II, ASTM C1186.

      - Non-combustible Material, ASTM E136.

      - Flame Spread Index: 0, ASTM E84.

      - Smoke Developed Index: Less than 5, ASTM E84.

      - Net Free Ventilation: 12.6 sq inches per linear ft (26670 sq mm per linear m).

      - Thickness: 1/4 inch (6 mm).

    - Type:

** NOTE TO SPECIFIER ** Delete product options not required.

      - Smooth, Vented: 12-inches (305 mm) x 12-feet (3658 mm).

      - Smooth, Vented: 16-inches (406 mm) x 12-feet (3658 mm).

      - Smooth, Vented: 24-inches (610 mm) x 8-feet (2438 mm).

      - Textured Cedarmill: 12-inches (305 mm) x 12-feet (3658 mm).

      - Textured Cedarmill: 16-inches (406 mm) x 12-feet (3658 mm).

      - Textured Cedarmill: 24-inches (610 mm) x 8-feet (2438 mm).

    - Color: __________, selected by Architect from manufacturer's standard range.

    - Basis-of-Design Product: HardieSoffit VentedPlus Panels; https://www.jameshardie.com.

      - Substitutions: According to requirements of Division 01, General Requirements.

  - ACCESSORIES

    - HardieTrim Trim Boards; https://www.jameshardie.com.

      - Type A, Grade I, ASTM C1186.

      - Flame Spread Index: 0, ASTM E84.

      - Smoke Developed Index: Less than 5, ASTM E84.

** NOTE TO SPECIFIER ** Delete board, texture, and thickness options not required.

      - Batten Board: 2-1/2 inch (63 mm) width.

      - 4/4 Boards: 3-1/2 inch (89 mm) width.

      - 4/4 Boards: 5-1/2 inch (140 mm) width.

      - 4/4 Boards: 7-1/4 inch (184 mm) width.

      - 4/4 Boards: 9-1/4 inch (235 mm) width.

      - 4/4 Boards: 11-1/4 inch (286 mm) width.

      - 5/4 Boards: 3-1/2 inch (89 mm) width.

      - 5/4 Boards: 5-1/2 inch (140 mm) width.

      - 5/4 Boards: 7-1/4 inch (184 mm) width.

      - 5/4 Boards: 9-1/4 inch (235 mm) width.

      - 5/4 Boards: 11-1/4 inch (286 mm) width.

      - Color: __________, selected by Architect from manufacturer's standard range.

      - Texture: Smooth.

      - Texture: Rustic.

      - Texture: Wood Grained.

      - Length: 12 feet (3658 mm).

      - Thickness: 3/4 inch (19 mm).

      - Thickness: 1 inch (25 mm).

    - Seam Tape: HardieWrap; https://www.jameshardie.com.

    - Flashing:

** NOTE TO SPECIFIER ** Delete flashing option not required.

      - HardieWrap Flashing Tape; https://www.jameshardie.com.

      - HardieWrap Flex Flashing Tape; https://www.jameshardie.com.

    - Fasteners:

** NOTE TO SPECIFIER ** Delete fastener options not required.

      - Wood Framing Fasteners:

        - 4d common corrosion resistant nails.

        - 6d common corrosion resistant nails.

        - 8d box ring common corrosion resistant nails.

        - Corrosion Resistant Siding Nails.

        - 0.089 inch (2.2 mm) shank x 0.221 inch (5.6 mm) head x 2-inches (51 mm) corrosion resistant siding nails.

        - 0.093 inch (2.4 mm) shank x 0.222 inch (5.6 mm) head x 2-inches (51 mm) corrosion resistant siding nails.

        - 0.093 inch (2.4 mm) shank x 0.222 inch (5.6 mm) head x 2-1/2 inches (64 mm) corrosion resistant siding nails.

        - 0.091 inch (2.3 mm) shank x 0.221 inch (5.6 mm) head x 1-1/2 inches (38 mm) corrosion resistant siding nails.

        - 0.091 inch (2.3 mm) shank x 0.225 inch (5.7 mm) head x 1-1/2-inches (38 mm) corrosion resistant siding nails.

        - 0.121 inch (3 mm) shank x 0.371 inch (9.4 mm) head x 1-1/4 inches (32 mm) corrosion resistant roofing nails.

        - No. 11 gauge 1-1/4 inches (32 mm) corrosion resistant roofing nails.

        - No. 11 gauge 1-1/2 inches (38 mm) corrosion resistant roofing nails.

        - No. 11 gauge 1-3/4 inches (44 mm) corrosion resistant roofing nails.

        - 16 gauge 1-1/2 inches (38 mm) stainless finish nails.

      - Metal Framing Fasteners:

        - 1-1/4 inches (32 mm) No. 8-18 x 0.375 inch (9.5 mm) head self-drilling, corrosion resistant S-12 ribbed buglehead screws.

        - 1-5/8 inches (41 mm) No. 8-18 x 0.323 inch (8.2 mm) head self-drilling, corrosion resistant S-12 ribbed buglehead screws.

        - 1 inch (25 mm) No. 8-18 x 0.323 inch (8.2 mm) head self-drilling, corrosion resistant ribbed buglehead screws.

        - 1 inch (25 mm) No. 8-18 x 0.311 inch (7.9 mm) head self-drilling, corrosion resistant S-12 ribbed buglehead screws.

        - 1.5 inch (38 mm) AGS-100 .100 x 25 inches (2540 x 635 mm) ETandF Pin or equivalent pneumatic fastener.

      - Masonry Walls:

        - Aerico Stud Nail, ETandF ASM No.144-125, 0.14 inch (3.6 mm) shank x 0.30 inch (7.6 mm) head by 2 inches (51 mm) long corrosion resistant nails.

  - FINISHES

    - Factory Applied Universal Primer:

      - Primer: Factory primed by Manufacturer.

      - Topcoat: Reference Section 09 90 00, Painting and Coating and project Finish Schedule.

    - Factory Applied Finish: Reference project Finish Schedule.

      - As supplied by Manufacturer.

      - Applied by the same company and facility that manufactures the siding substrate.

        - Applied in controlled environment within manufacturer's facility utilizing multi-coat, heat cured finish within one manufacturing process.

      - Each Finish Color: To have documented color match to Delta E of 0.5 or better between product lines, manufacturing lots, or production runs as measured by photospectrometer and verified by third party.

      - Provide finish protection prior to transit to site. Remove finish protection after installation.

      - Accessories: Complete finishing system including pre-packaged touch-up kit provided by manufacturer.

      - Quantities: As recommended by manufacturer.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - Examine substrate, clean and repair to eliminate conditions detrimental to installation.

    - Do not commence with the Work until unacceptable conditions are remedied.

    - Verify that weather barrier is installed over substrate and ready to receive the Work.

    - If framing preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation prior to commencement of Work.

    - Weather Barrier Installation: See Section 07 25 00, Weather Barriers.

      - Ensure weather barrier is installed with penetration and junction flashing in accordance with manufacturer's written requirements and the Authorities Having Jurisdiction.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using methods recommended by manufacturer's written requirements.

    - Protect surrounding work areas and surface during installation.

    - Repair punctures or tears in weather barrier prior to installation of siding.

** NOTE TO SPECIFIER ** Delete article if not required.

  - INSTALLATION - LAP SIDING

    - Install in strict accordance with manufacturer's written installation instructions at locations shown on Drawings.

    - General:

      - Protect siding from Work of other trades.

      - Surfaces to be dry prior to installation of weather barriers and siding.

      - Reference trim details on Drawings.

      - Touch-up cut edges prior to installation.

    - Flashing:

      - Install above door and window trim and casings.

      - Install above horizontal trim in field of siding.

      - [______________].

      - [______________].

** NOTE TO SPECIFIER ** Delete article if not required.

  - INSTALLATION OF SOFFIT AND VENTED PANELS

    - Install in strict accordance with manufacturer's written installation instructions at locations shown on Drawings.

    - Panels may be installed as soffit or ceiling over wood or steel framing.

    - 20-gauge (33 mil) minimum to 16-gauge (54 mil) in compliance with the Authorities Having Jurisdiction.

    - Additional framing may be necessary to ensure proper fastening of siding.

** NOTE TO SPECIFIER ** Delete article if not required.

  - INSTALLATION OF TRIM BOARDS

    - Install in strict accordance with manufacturer's written installation instructions at locations shown on Drawings.

    - Install flashing around all wall openings.

  - FINISHING

    - Finish: According to paint manufacturer's written application instructions.

** NOTE TO SPECIFIER ** Delete paragraph is not required.

    - Unprimed Siding:

      - Primer: One coat minimum. Manufacturer approved.

** NOTE TO SPECIFIER ** Delete topcoat paint options not required.

      - Exterior Grade Topcoat Paint: One coat minimum. Acrylic: 100 percent.

      - Exterior Grade Topcoat Paint: One coat minimum. Latex: 100 percent.

      - Exterior Grade Topcoat: Two coats within 90-days of installation.

        - Paint: High quality alkali resistant. Acrylic: 100 percent.

        - Paint: High quality alkali resistant. Latex: 100 percent.

** NOTE TO SPECIFIER ** Delete paragraph is not required.

    - Factory Finish Primed Siding: Manufacturer approved.

      - Exterior Grade Topcoat Paint: One coat minimum within 180-days of installation.

** NOTE TO SPECIFIER ** Delete pain option not required.

        - Paint: High quality Acrylic: 100 percent.

        - Paint: High quality Latex: 100 percent.

  - CLEANING

    - See Section 01 70 00, Execution and Closeout Requirements.

    - Clean siding in accordance with manufacturer's written instructions using materials and methods acceptable to manufacturer.

  - PROTECTION

    - Protect installed products until date of Substantial Completion.

    - Touch-up, repair or replace damaged areas prior to date of Substantial Completion.

END OF SECTION